California/Oregon

advanced transportation

Systems

Candidate Early-Winner Projects

Prepared by

Western Transportation Institute

Civil Engineering Department

Montana State University – Bozeman

for the

State of California

Department of Transportation

New Technology and Research Program

and

State of Oregon

Department of Transportation

Traffic Management Section

in cooperation with the

U.S. Department of Transportation

Federal Highway Administration

April 1999

# Table Of Contents

Introduction	1

I.	Bi-State Traveler Safety & Incident Management System	5

II.	Coos Bay Traveler information System	7

III.	Redding Area Traveler Information System	8

IV.	Del Norte County Traveler Safety and Weather Information System	9

V.	Regional Traveler information System	10

VI.	Rural Incident Management Corridor	12

Appendix A	13

Appendix B	14

# Introduction

## Purpose of Document

The purpose of this document is to propose candidate Early-Winner projects to be implemented as part of the California/Oregon Advanced Transportation System (COATS) study area.  This document provides the results the ranking exercise in which various project participants ranked proposed infrastructure locations and the candidate early-winner projects.  These results are detailed in the following sections.  It should be noted that maps detailing legacy systems and proposed infrastructure are NOT included in this document and will be provided at the Spanish Springs Steering Committee meeting.

Previous efforts include the documentation of existing and planned transportation systems and analysis of the potential challenges in the bi-state area.  The documents related to these tasks are included in Technical Memorandum One, Volumes 1 & 2; the “Review of Legacy Systems” report and the “Conditions and Performance” report, respectively.  Using the information from these documents, potential locations for ITS infrastructure was developed and outlined in the “Project Infrastructure” report.

## Project Rankings

General Infrastructure types and specific infrastructure locations were ranked using the Project Infrastructure document and ranking tables.  The level of response to this exercise was that 60% of those queried responded.  A value was assigned to each of the responses (i.e., short-term, medium-term, long-term, delete or existing) in this exercise as shown below.

- Existing	  no value

- Delete		  0

- Long-term	  1

- Medium-term	  2

- Short-term	  3

In essence, locations having a higher score were ranked higher.  The top 10% of the ranked infrastructure locations were considered in determining potential early-winner projects.  Responses were also averaged for each particular infrastructure type.  This allowed both the locations to be ranked AND the type to be ranked.  The average number of project participants that ranked a particular infrastructure was also considered and is shown in Figure 1.  This Figure also shows the average value of the responses made by these participants.  Again, 3 is the highest score that any particular infrastructure type can receive.  From this analysis, the top 12 infrastructure types listed below were determined.

- Regional Incident Management Plan (RIMP)

- Road Weather Information Systems (RWIS)

- Highway Advisory Radio (HAR)

- Closed-Circuit Television Cameras(CCTC)

- Touch Screen Interactive Kiosks (TSIK)

- Automated Flood Warning (AFW)

- Dynamic Warning Variable Message Signs (DWVMS)

- Advisory Television (AT)

- Variable Message Signs (VMS)

- 1-800 Travel Advisory Telephone Hotline (TATH)

- Motorist-Aide Call Boxes (MACB)

- Automated Visibility Warning (AVW)

- Figure 1: Average number of responses and average response of infrastructure ranking exercise.

Using this ranking information, the top 10% of the infrastructure locations and any added infrastructure that was included in one of the top 12 infrastructure types was mapped within the corridor.  General “hot spots” were located by observing where several infrastructure elements were grouped.  These groupings became the geographic areas of focus from which six early-winner project areas were chosen.

## Document Contents

From the analysis explained above, six candidate projects were chosen for consideration in the early-winner selection process.  This document details the candidate early-winner projects within the project limits.  Each section contains information regarding:

- Specific project limits,

- General project description,

- Related legacy systems,

- Ranked infrastructure types and associated locations and

- Estimated costs associated with each infrastructure element.

### Project Limits

A map of the COATS area (Figure 2) is provided within this introductory section to show the project limits of each of the candidate early-winner projects.  Again, there are six candidate projects within the area.  Each project is assigned a roman numeral that will be used throughout the selection process to identify specific candidate projects.  In the section describing the individual candidate projects, a general description of the area as well as a bulleted list of the Route and Milepost information is provided.

### Project Description

This provides a general description of how individual project infrastructure may add value to existing ITS developments or fill needs/gaps in existing infrastructure.  This section also gives a general description of the types of challenges that are faced in this area.

### Infrastructure Location Details and Associated Costs

This section provides a list of the ranked and added infrastructure within the candidate project limits.  Infrastructure that was ranked is listed first and is followed by an asterisk.  Added infrastructure within the project limits is also listed.  When a particular infrastructure type contained both ranked and added locations, the number of ranked locations (tabulated first) follows.  Basis for associated costs can be found in Appendix A.  Appendix A details cost breakdown, assumptions made, and the source for the number for each infrastructure type.

### Related Legacy Systems

A list of legacy systems from the “Review of Legacy Systems” report corresponding to the candidate project area is provided.  This bulleted list is intended to give the reader a general idea of the types of systems that are already in place or planned for deployment within the area.  A more detailed table of legacy systems is given in Appendix B.

Figure 2: Map of study area showing six candidate early-winner focus areas.

I

# I.  Bi-State Traveler Safety & Incident Management System

## Project Limits: Interstate 5 from Yreka, California to Medford, Oregon

- Interstate 5, Postmile Sis-48 to Sis-69, California

- Interstate 5, Milepost 0 to 30, Oregon

## Project Description

Interstate 5 from Yreka, California to Medford, Oregon is a major connecting route between these cities having an average daily truck traffic between 4000 and 6000.  It crossed Siskiyou Pass near the California/Oregon border that frequently experiences inclement weather conditions.  A Variable Message Sign (VMS) North of Junction 96, would help provide traveler information related to weather, road closures or other incidents.  The nearby Medford Traffic Management Center would be able to assist in field data collection and distribution activities in the area.

From project participants’ input it was determined that infrastructure that supports pre-trip traveler information through a cable advisory television channel may be beneficial.  This channel may also display closed circuit television images of road conditions.  These images can be used by the existing and planned Kiosks as well.  This project will also provide a Regional Incident Management Plan for managing road closures and improving emergency notification and response times.  The suggested Dynamic Speed-Warning Variable Message Signs would provide real-time information related to spot hazards.  Suggested Motorist-Aide Call Boxes may be implemented to assist stranded motorists.

## Infrastructure Location Details and Associated Costs:

| Rank | Infrastructure Element | Approx. Cost ($K) | State | Hwy | From MP | To MP | Dir | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Variable Message Sign* | $155 | CA | I-5 | Sis-58 |  | S | North of Junction 96/I-5 |
| 2 | Regional Incident Management Plan | $50 | CA/OR | I-5 |  |  |  | Between Siskiyou Co. - CA and Jackson |
| 3 | Closed-Circuit Television | $43 | OR | I-5 | 3 |  |  | 3 miles North of CA/OR border |
| 4 | Dynamic Warning VMS | $104 | OR | I-5 | 4.6 |  |  | Siskiyou Summit for runaway trucks |
|  |  | $104 | OR | I-5 | 27.9 |  |  | Medford Viaduct curves |
|  |  | $104 | CA | I-5 | Sis-23 | Sis-38 |  | high wind warning |
| 5 | Advisory Television | $0 | CA/OR | I-5 |  |  |  | between Yreka, CA and Ashland, OR |
| 6 | Motorist-Aide Call Box | $5 | OR | I-5 | 4.6 |  |  | Siskiyou Summit (climbing lanes) |
|  |  | $5 | OR | I-5 | 28.7 |  |  | Medford Viaduct (update existing) |
|  | Total Cost: | $570 |  |  |  |  |  |  |

## Related Legacy Systems:

- CCTV – 1 existing, 2 planned

- CMS – 1 existing, 3 planned

- Touch Screen Information Kiosks – 5 existing, 1 planned

- Preclearance – 1 existing

- RWIS – 2 existing, 1 planned

- Traffic Management Center – 1 existing

# II.  Coos Bay Traveler information System

## Project Limits: Junction 101/42 area near Coos Bay, Oregon

- Highway 101, Milepost 235 to 276

- Highway 42, Milepost 0 to 25

- Highway 42S, Milepost 0 to 17

## General Description

The Coos Bay, Oregon area, located on the scenic byway 101, is a major route for tourist traffic with four major recreational destinations having between 200,000 and 400,000 visitors per year and three recreational destinations having between 400,000 and 900,000 visitors per year.  High volumes of recreational traffic make it necessary to provide adequate traveler information.  This project will provide traveler and tourism information through Highway Advisory Radio (HAR), additional Touch Screen Interactive Kiosks and Variable Message Signing (VMS) to benefit both familiar and unfamiliar transportation users.  Travel and tourism service information could be managed locally while safety and traffic management activities would be handled from either the Bend or Medford Traffic Management Center.

From project participants’ input it was determined that infrastructure used for the detection of road and weather conditions and flood conditions through RWIS and automated flood warning systems should be included in this area.  These systems would help with known visibility challenges in the area.  The addition of a Motorist-Aide Call Box was also suggested.

## Infrastructure Location Details and Associated Costs:

| Rank | Infrastructure Element | Approx. Cost ($K) | State | Hwy | From MP | To MP | Dir | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Highway Advisory Radio* | $58 | OR | 42 | 0 |  |  | Visibility problems highway 42 near 101 |
| 2 | Touch Screen Kiosk* | $15 | OR | 101 | 251 |  |  | Sunset Bay State Park |
| 3 | RWIS | $46 | OR | 101 | 258 |  |  | South of Bandon |
| 4 | Automated Flood Warning | $42 | OR | 42S |  |  |  |  |
|  |  | $42 | OR | 35 | 0 | 5 | E,W | Highway 42 approaching 101 |
| 5 | Variable Message Sign | $155 | OR | 101 | 247 |  | N | South of Hwy 35 junction (Ore 42) |
| 6 | Motorist-Aide Call Box | $5 | OR | 101 | 235 | 263 | N,S |  |
|  | Total Cost: | $363 |  |  |  |  |  |  |

## Related Legacy Systems:

- Information Kiosks – 2 existing

- RWIS – 2 existing

# III.  Redding Area Traveler Information System

## Project Limits: Junction 89/ I-5 area North of Redding, California

- Highway 89, Postmile Sis-10 to Sis-35

- Interstate 5, Postmile Sha-50 to Sha-67

- Interstate 5, Postmile Sis-0 to Sis-19

## Project Description

Interstate 5 near this junction is characterized by frequent road closures due to inclement weather in the region.  The average daily truck traffic on Interstate 5 ranges from 4000 to 6000 trucks per day.  Ranked infrastructure included Remote Weather Information Systems and Variable Message Signs.  These proposed systems can be integrated with existing Kiosks, CCTV and RWIS to provide real-time weather and road condition information to travelers.  Project participants indicated that Kiosks might also be used in addition to the HAR and VMS to disseminate pertinent traveler information.  Traveler information and coordination would most likely be provided through the Caltrans District 2 office.

## Infrastructure Location Details and Associated Costs:

| Rank | Infrastructure Element | Approx. Cost ($K) | State | Hwy | From MP | To MP | Dir | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | RWIS* | $46 | CA | Sha-I-5 | 55 |  |  | Road closure due to bad weather |
| 2 | Variable Message Sign* (2) | $155 | CA | Sis-I-5 | 8 |  | N |  |
|  |  | $155 | CA | Sis-89 | 34 |  | W |  |
| 3 | Touch Screen Kiosk | $15 | CA | Sha-I-5 |  |  |  | Churn Creek Bottom truck stop |
|  | Total Cost: | $371 |  |  |  |  |  |  |

## Related Legacy Systems:

- Automated Wind Advisory – 1 planned

- CCTV – 2 planned

- CMS – 2 existing

- EMS Communication – 1 existing

- HAR – 1 existing

- Information Kiosks – 1 planned

- Preclearance – 1 existing

- Dynamic VMS – 2 planned

- RWIS – 2 existing, 3 planned

- WIM – 1 existing

# IV.  Del Norte County Traveler Safety and Weather Information System

## Project Limits: Highway 199, Del Norte County, California

- Highway 199, Postmile DN-0 to DN-36

## Project Description

As a main route from Medford to the Pacific coast, i.e., Crescent City on Highway 101, this highway has frequent tourist traffic.  The Jediah Smith Redwoods State Park has over 200,000 visitors annually.  Data shows that visibility, narrow clear zone and intersection related crashes are significant issues.  Additionally, there are several road closures due to slides.  Notification and response times to crashes are relatively high as compared to other areas in the study area.  As such ranked infrastructure included Touch Screen Interactive Kiosks, Remote Weather Information Systems, Highway Advisory Radio, Variable Message Signs and Automated Visibility Warning Systems.  This project will attempt to coordinate RWIS information including visibility data to provide real-time traveler information.  Related information would be distributed using HAR and Kiosks and existing VMS.  Traveler information and coordination would most likely be provided through the Caltrans District 1 office.

## Infrastructure Location Details and Associated Costs:

| Rank | Infrastructure Element | Approx. Cost ($K) | State | Hwy | From MP | To MP | Dir | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Touch Screen Kiosk* | $15 | CA | DN-199 | 9 |  |  | Jediah Smith Redwoods State Park |
| 2 | RWIS* (2) | $46 | CA | DN-199 | 6 |  |  | visibility challenge |
|  |  | $46 | CA | DN-199 | 34 |  |  | road closures due to bad weather |
| 3 | Highway Advisory Radio* (2) | $58 | CA | DN-101 | 30 |  |  | visibility challenge, road closures, tourism |
|  |  | $58 | CA | DN-199 | 34 |  |  | road closures |
| 4 | Automated Visibility Warning* | $21 | CA | DN-199 | 0.6 | 1.9 |  | visibility challenge |
| 5 | Variable Message Sign | $155 | CA | DN-199 | 0 |  |  | tourist information |
|  | Total Cost: | $399 |  |  |  |  |  |  |

## Related Legacy Systems:

- Motorist-Aide Call Boxes – 8 existing

- CMS – 1 planned

- Traffic Management Center – 1 existing (near)

# V.  Regional Traveler information System

## Project Limits: Loop from Crescent City, CA to Medford, OR to Yreka, CA to Redding, CA to Eureka, CA back to Crescent City, CA

- Highway 199, Postmile DN-0 to DN-36, California

- Highway 199, Milepost 42 to 0, Oregon

- Interstate 5, Milepost 58 to 0, Oregon

- Interstate 5, Postmile Sis-69 to Sis-0, California

- Interstate 5, Postmile Sha-67-Sha-17, California

- Highway 299, Postmile Sha-24 to Sha-0, California

- Highway 299, Postmile Tri-73 to Tri-0, California

- Highway 299, Postmile Hum-41 to Hum-0, California

- Highway 101, Postmile Hum-88 to Hum-137, California

- Highway 101, Postmile DN-0 to DN-30, California

## Project Description

For travelers in Northern California and Southern Oregon, sporadic weather conditions, winding roads and high tourist population associated with recreational destinations warrants the need for an integrated traveler information system in this region.  This project will attempt to provide traveler information to regional travelers using HAR, Touch Screen Interactive Kiosks and advisory television.  This new infrastructure will add value to the existing systems on Interstate and non-Interstate roadways.  Road-weather and traffic operations information would be managed from respective state DOT traffic management centers.  Advisory television information will be managed by a local cable provider.  Given that the area is a common network for those traveling from State to State and for those traveling to the coast, this project focuses on a common intercept point for transportation users in the COATS region.  This project would attempt to initiate infrastructure deployment with the intent of integrating additional infrastructure in the future (i.e. Phase II Showcase).

## Infrastructure Location Details and Associated Costs:

| Rank | Infrastructure Element | Approx. Cost ($K) | ##### State | Hwy | From MP | To MP | Dir | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Highway Advisory Radio* (3) | $58 | CA | DN-101 | 30 |  |  | visibility, road closures, tourist |
|  |  | $58 | CA | DN-199 | 34 |  |  | road closure |
|  |  | $58 | OR | I-5 | 28.33 |  |  | in conjunction with existing VMS |
|  |  | $58 | CA | Tri-299 | 52 |  |  | Weaverville HAR for road conditions |
|  |  | $58 |  |  |  |  |  |  |
| 2 | Touch Screen Kiosk* (3) | $15 | CA | Hum-101 | 108.22 |  |  | Humboldt Lagoons State Park |
|  |  | $15 | CA | Hum-101 | 126.1 |  |  | Prarie Creek Redwoods State Park |
|  |  | $15 | CA | DN-199 | 9 |  |  | Jediah Smith Redwoods State Park |
|  |  | $15 | OR | 199 |  |  |  | Oregon Caves Welcome Center |
|  |  | $15 | CA | Sha-I-5 | 31.1 |  |  | Rest area near O'Brien |
|  |  | $15 | CA | Sha-I-5 | 43.1 |  |  | Rest area near Lakehead |
|  |  | $15 | CA | Sha-I-5 |  |  |  | Churn Creek Bottom truck stop |
|  |  | $15 | CA | Sis-I-5 | 25.6 |  |  | Rest Area near Weed |
|  |  | $15 | CA | Sis-I-5 | 58.6 |  |  | Rest Area near Collier |
| 3 | Advisory Television | $0 | CA/OR | I-5 |  |  |  | between Yreka, CA and Ashland, OR |
|  | Total Cost: | $425 |  |  |  |  |  |  |

## Related Legacy Systems:

- Automated Wind Advisory – 2 planned

- Motorist-Aide Call Boxes – 20 existing

- CCTV – 1 existing, 4 planed

- CMS – 5 existing, 11 planned

- HAR – 4 existing, 1 planned

- Information Kiosks – 9 existing, 5 planned

- Preclearance – 2 existing

- Dynamic VMS – 5 existing

- RWIS – 6 existing, 6 planned

- Satellite Operations Center – 1 existing

- Traffic Management Center – 1 existing

- WIM – 2 existing

# VI.  Rural Incident Management Corridor

## Project Limits: Junction 58/97 South of Bend to Bend and Highway 20 – Bend to Burns

- Highway 97, Milepost 205 to 138

- Highway 20, Milepost 0 to 131

- Highway 58, Milepost 40 to 86

## Project Description

This rural corridor experiences frequent road closures and above average notification and response times to crashes.  Average daily truck traffic on this section of Highway 97 ranges from 500 to 2200 trucks per day and Highway 20 averages below 500 trucks per day.  Implementing a Regional Incident Management Plan would help emergency services and maintenance personnel respond to accidents and road closure situations in this proposed corridor.  Incident management activities would be coordinated in Bend at the Traffic Management Center.  Highway advisory radio could be implemented at the intersection of Highway 97 and Highway 58 to provide pertinent traveler information and assist in distributing existing RWIS information.  Project participants indicated that a VMS would be of assistance for traffic information and would help support HAR system.  Finally, project participants also indicated Dynamic Speed-Warning VMS would provide high-wind speed warnings to high profile vehicles near Hines, Oregon.

## Infrastructure Location Details and Associated Costs:

| ##### Rank | ##### Infrastructure Element | ##### Approx. Cost ($K) | ##### State | ##### Hwy | ##### From MP | ##### To MP | ##### Dir | ##### Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Regional Incident Management Plan* | $50 | OR | 20 | 0 | 65 |  | Road closures |
|  |  | $50 | OR | 97 | 138.1 | 170 |  | Road closures |
| 2 | Highway Advisory Radio* | $58 | OR | 97 | 196 |  |  | Road closures |
| 3 | Dynamic Warning VMS* (1) | $104 | OR | 20 | 128.3 |  | W | High wind warning for mobile homes |
|  |  | $104 | OR | 58 | 56.5 |  | W | Visibility Challenge |
| 4 | Variable Message Sign | $155 | OR | 58 | 69 |  | W | Begin snow zone / mountain corridor |
|  | Total Cost: | $521 |  |  |  |  |  |  |

## Related Legacy Systems:

- RWIS – 7 existing, 2 planned

- CCTV – 1 planned

- Kiosks – 1 existing

- Traffic Management Center – 1 existing

| Infrastructure | Capital Equipment ($K) (1) | Annual Maintenance Cost ($K) (2) | Total Cost | Cost Assumptions | Cost Origin |
| --- | --- | --- | --- | --- | --- |
| Regional Incident Management Plan | $ 50 | $ 0 | $ 50 | -Inventory of resources -Operations Plan -Routing Plan | WTI |
| RWIS | $ 40 | $ 6 | $ 46 | 1. Asst. Chief Maint. Engineer 2. Routine maint. & calibration of detectors | 1.Montana DOT 2. Caltrans |
| HAR | $ 50 per tower | $ 8 | $ 58 | 1. Not including communication cost 2. Routine maintenance | 1. ODOT ITS Strategic Plan 2. TxDOT/ FHWA |
| CCTV | $ 35 | $ 8 | $ 43 | 1. Not including communication cost 2. Routine maintenance for CCTV, camera controls, housing and/or support pole | 1. ODOT ITS Strategic Plan 2. TxDOT |
| Kiosks | $ 10 | $ 5 | $ 15 | 2. Operations: power consumption; Maintenance; routine and repairs to vandalized kiosks | 1. ODOT ITS Strategic Plan 2. TxDOT/ TTI/ FHWA |
| Automated Flood Warning | $ 40 | $ 2 | $ 42 |  | ODOT |
| Dynamic Warning VMS | $ 100 | $ 4 | $ 104 |  | Caltrans, District 2 |
| Advisory TV | $ 0 | $ 0 | $ 0 | Usually cable TV provide municipal access channel | ODOT ITS Strategic Plan |
| VMS | $ 150 | $ 5 | $ 155 |  | ODOT |
| 1-800 Telephone | $ 1,000 |  |  | Hardware/software | ODOT ITS Strategic Plan |
| Call Boxes | $ 4 | $ 0.5 | $ 4.5 | Operations: telephone, incl. Cellular airtime. Maintenance: routine and repairs to vandalism | FHWA/ TTI |
| Automated Visibility Warning | $ 20 | $ 1 | $ 21 |  | Advanced Measurements & Controls |

## References:

- “Caltrans Transportation Management Center Standardization Plan”, prepared for Caltrans, prepared by Odetics ITS, Inc., December 1998.

- “Oregon Department of Transportation ITS Strategic Plan (1997 – 2017)”, prepared for ODOT, prepared by DKS Associates, January 1998.

- “Guidelines for Funding Operations and Maintenance of ITS/ATMS” research Report 1494-1F, prepared for Texas Department of Transportation by Texas Transportation Institute, November 1996.

- “Surveillance, Control, and Driver Information Implementation and Operations Plan”, prepared by Washington Department of Transportation, September 1998.

| ###### I. Bi-State Traveler Safety and Incident Management System |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technology |  | Existing/Planned | State | County | Route | From MP | Dir | Location |
| CCTV |  | P | CA | Sis | 5 | 65.52 | N | Bailey Hill Rd |
| CCTV |  | P | OR |  | 5 | 5 |  |  |
| CCTV |  | E | OR |  | 5 | 28.33 |  | Medford |
| CMS |  | E | OR |  | 5 | 28.33 |  | Medford |
| CMS |  | P | CA | Sis | 5 | 65.52 | N | Bailey Hill Rd |
| CMS |  | P | OR |  | 5 | 16.72 |  |  |
| CMS |  | P | OR |  | 5 | 31.31 |  |  |
| Information Kiosks |  | E | OR |  | 5 | 19.10 |  | Ashland |
| Information Kiosks |  | E | OR |  | 5 | 28.33 |  | Medford |
| Information Kiosks |  | E | OR |  | 5 | 28.33 |  | Medford |
| Information Kiosks |  | E | OR |  | 5 | 28.33 |  | Medford |
| Information Kiosks |  | E | OR |  | 5 | 28.33 |  | Medford |
| Information Kiosks |  | P | CA | Sis | 5 | 58 | S | Collier Rest Area |
| Preclearance |  | E | OR |  | 5 | 19.1 | N/S | Ashland |
| RWIS |  | E | OR |  | 5 | 5.02 |  | Siskiyou Summit |
| RWIS |  | E | OR |  | 5 | 28.33 |  | Medford Viaduct |
| RWIS |  | P | OR |  | 5 | 27 |  | Medford Viaduct |
| Traffic Management Center |  | E | OR |  | 5 | 28.33 |  | Medford |
| II. Coos Bay Traveler Information System |  |  |  |  |  |  |  |  |
| Information Kiosks |  | E | OR |  | 101 | 234.03 |  | North Bend |
| Information Kiosks |  | E | OR |  | 101 | 260.64 |  | Bandon |
| RWIS |  | E | OR |  | 101 | 233.00 |  |  |
| RWIS |  | E | OR |  | 101 | 258.00 |  |  |
| Traffic Management Center |  | E | OR |  | 5 | 28.33 |  | Medford |
| Traffic Management Center |  | E | OR |  | 20 | 18.51 |  | Bend |
| III. Redding Area Traveler Information System |  |  |  |  |  |  |  |  |
| AutomatedWind Advisory |  | P | CA | Sis | 5 | 18.5 | N | Weed Airport |
| CCTV |  | P | CA | Sis | 5 | 14.45 | N | Black Butte Summit |
| CCTV |  | P | CA | Sis | 89 | 29.2 |  | McCloud Summit |
| CMS |  | E | CA | Sis | 5 | 9.68 | N | Ream Rd |
| CMS |  | E | CA | Sis | 5 | 13.19 | S | Abrams Lake Rd |
| EMS Communication |  | E | CA | Sis | 89 | 24.75 |  | McCloud |
| HAR |  | E | CA | Sis | 5 | 13.19 | S | Abrams Lake Rd |
| Information Kiosks |  | P | CA | Sis | 5 | 7.1 | S | Truck Scales |
| Preclearance |  | E | CA | Sis | 5 | 7.1 |  | Dunsmuir Scales |
| RCMS |  | P | CA | Sha | 5 | 49.1 | S | N of Lamoine OC |
| RCMS |  | P | CA | Sha | 5 | 58.1 | N | N of Sima Rd |
| RWIS |  | E | CA | Sis | 5 | 7.1 | S | Truck Scales |
| RWIS |  | E | CA | Sis | 5 | 14.45 | N | Black Butte Summit |
| RWIS |  | P | CA | Sis | 5 | 18.5 | N | Weed Airport |
| RWIS |  | P | CA | Sis | 89 | 3.23 |  | Deadhorse Summit |
| RWIS |  | P | CA | Sis | 89 | 29.2 |  | McCloud Summit |
| WIM |  | E | CA | Sis | 5 | 7.1 |  | Dunsmuir Scales |
| IV. Del Norte County Traveler Safety and Weather Information System |  |  |  |  |  |  |  |  |
| Technology |  | Existing/Planned | State | County | Route | From MP | Dir | Location |
| Call Box |  | E | CA | DN | 199 | 1 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 3 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 8 | Left | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 10.81 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 20.07 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 23.13 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 26.12 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 33.32 | Left | Del Norte County |
| CMS | P |  | CA | DN | 199 | 36.2 | S | S OF STATE LINE |
| Traffic Management Center |  | E | OR |  | 5 | 28.33 |  | Medford |
| V. Regional Traveler Information System |  |  |  |  |  |  |  |  |
| Automated Wind Advisory |  | P | CA | Sis | 5 | 18.5 | N | Weed Airport |
| Automated Wind Advisory |  | P | CA | Sis | 5 | 44.3 | N | Walters Rd |
| Call Box |  | E | CA | DN | 101 | 14.8 | Left | Del Norte County |
| Call Box |  | E | CA | DN | 101 | 18.97 | Right | Del Norte County |
| Call Box |  | E | CA | Hum | 101 | 97.5 |  | Humboldt County |
| Call Box |  | E | CA | Hum | 101 | 97.6 |  | Humboldt County |
| Call Box |  | E | CA | Hum | 101 | 107.2 |  | Humboldt County |
| Call Box |  | E | CA | Hum | 101 | 107.3 |  | Humboldt County |
| Call Box |  | E | CA | Hum | 101 | 131.6 |  | Humboldt County |
| Call Box |  | E | CA | Hum | 101 | 131.7 |  | Humboldt County |
| Call Box |  | E | CA | DN | 199 | 1 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 3 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 8 | Left | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 10.81 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 20.07 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 23.13 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 26.12 | Right | Del Norte County |
| Call Box |  | E | CA | DN | 199 | 33.32 | Left | Del Norte County |
| Call Box |  | E | CA | Hum | 299 | 12.6 |  | Humboldt County |
| Call Box |  | E | CA | Hum | 299 | 21.1 |  | Humboldt County |
| Call Box |  | E | CA | Hum | 299 | 27.8 |  | Humboldt County |
| Call Box |  | E | CA | Hum | 299 | 37.7 |  | Humboldt County |
| CCTV |  | E | OR |  | 5 | 28.33 |  | Medford |
| CCTV |  | P | CA | Sis | 5 | 14.45 | N | Black Butte Summit |
| CCTV |  | P | CA | Sis | 5 | 65.52 | N | Bailey Hill Rd |
| CCTV |  | P | OR |  | 5 | 5 |  |  |
| CCTV |  | P | OR |  | 199 | 16.1 |  |  |
| CMS |  | E | CA | Sha | 5 | 20.98 | N | Pine Grove |
| CMS |  | E | CA | Sis | 5 | 9.68 | N | Ream Rd |
| CMS |  | E | CA | Sis | 5 | 44.3 | N/S | Walters Rd |
| CMS |  | E | CA | Sis | 5 | 13.19 | S | Abrams Lake Rd |
| CMS |  | E | OR |  | 5 | 28.33 |  | Medford |
| CMS |  | P | CA | DN | 101 | 28.5 | N | S OF 101/199 SEP |
| CMS |  | P | CA | DN | 199 | 36.2 | S | S OF STATE LINE |
| CMS |  | P | CA | Hum | 101 | 87.7 | N | S OF 101/299 SEP |
| CMS |  | P | CA | Hum | 101 | 56.5 | N | S OF JCT 36 |
| CMS |  | P | CA | Hum | 101 | 89.4 | S | N OF 101/299 SEP |
| Technology |  | Existing/Planned | State | County | Route | From MP | Dir | Location |
| CMS |  | P | CA | Hum | 101 | 58.7 | S | N OF JCT 36 |
| CMS |  | P | CA | Sha | 5 | 43.5 | S | Lakehead Rest Area |
| CMS |  | P | CA | Sha | 299 | 22.1 | W | Redding |
| CMS |  | P | CA | Sis | 5 | 65.52 | N | Bailey Hill Rd |
| CMS |  | P | OR |  | 5 | 16.72 |  |  |
| CMS |  | P | OR |  | 5 | 31.31 |  |  |
| HAR |  | E | CA | Hum | 101 | 77.3 | N | EUREKA SHOP YARD |
| HAR |  | E | CA | Hum | 101 | 77.3 |  | EUREKA-DIST OFFICE |
| HAR |  | E | CA | Sis | 5 | 13.19 | S | Abrams Lake Rd |
| HAR |  | E | CA | Sis | 5 | 44.31 | S | Walters Rd |
| HAR |  | P | CA | Tri | 299 | 58.2 |  | Douglas City |
| Information Kiosks |  | E | OR |  | 5 | 19.10 |  | Ashland |
| Information Kiosks |  | E | OR |  | 5 | 28.33 |  | Medford |
| Information Kiosks |  | E | OR |  | 5 | 28.33 |  | Medford |
| Information Kiosks |  | E | OR |  | 5 | 28.33 |  | Medford |
| Information Kiosks |  | E | OR |  | 5 | 28.33 |  | Medford |
| Information Kiosks |  | E | OR |  | 5 | 35.44 |  | Central Point |
| Information Kiosks |  | E | OR |  | 5 | 55.38 |  | Grants Pass |
| Information Kiosks |  | E | OR |  | 5 | 58.06 |  | Grants Pass |
| Information Kiosks |  | E | OR |  | 199 | 27.71 |  | Cave Junction |
| Information Kiosks |  | P | CA | Sha | 5 | 31.1 | N | Obrian Rest Area |
| Information Kiosks |  | P | CA | Sha | 5 | 43.5 | S | Lakehead Rest Area |
| Information Kiosks |  | P | CA | Sis | 5 | 25.8 | N/S | Weed Rest Area |
| Information Kiosks |  | P | CA | Sis | 5 | 7.1 | S | Truck Scales |
| Information Kiosks |  | P | CA | Sis | 5 | 58 | S | Collier Rest Area |
| Preclearance |  | E | CA | Sis | 5 | 7.1 |  | Dunsmuir Scales |
| Preclearance |  | E | OR |  | 5 | 19.1 | N/S | Ashland |
| RCMS |  | P | CA | Sha | 5 | 58.1 | N | N of Sima Rd |
| RCMS |  | P | CA | Sha | 5 | 29.9 | S | N of Side Hill Viaduct |
| RCMS |  | P | CA | Sha | 5 | 31.9 | S | N of Obrian UC |
| RCMS |  | P | CA | Sha | 5 | 36.7 | S | N of Upper Salt Creek |
| RCMS |  | P | CA | Sha | 5 | 49.1 | S | N of Lamoine OC |
| RWIS |  | E | CA | Sis | 5 | 14.45 | N | Black Butte Summit |
| RWIS |  | E | CA | Sis | 5 | 23 | N | Edgewood |
| RWIS |  | E | CA | Sis | 5 | 7.1 | S | Truck Scales |
| RWIS |  | E | CA | Hum | 101 | 128.95 |  |  |
| RWIS |  | E | OR |  | 5 | 5.02 |  | Siskiyou Summit |
| RWIS |  | E | OR |  | 5 | 28.33 |  | Medford Viaduct |
| RWIS |  | P | CA | Sis | 5 | 18.5 | N | Weed Airport |
| RWIS |  | P | CA | Sis | 5 | 44.3 | N | Walters Rd |
| RWIS |  | P | CA | Sha | 299 | 68.17 |  | Hatchet Mt. Summit |
| RWIS |  | P | CA | Tri | 299 | 72.2 |  | Buckhorn Summit |
| RWIS |  | P | CA | Tri | 299 | 48.5 |  | Mtn Oregon |
| RWIS |  | P | OR |  | 5 | 27 |  | Medford Viaduct |
| Satellite Operations Center |  | E | CA | Hum | 101 | 75.91 |  | Eureka |
| Traffic Management Center | E |  | OR |  | 5 | 28.33 |  | Medford |
| WIM |  | E | CA | Sha | 5 | 22.14 |  | N of N. Jct 151/I-5 |
| WIM |  | E | CA | Sis | 5 | 7.1 |  | Dunsmuir Scales |
| VI. Rural Incident Management Corridor |  |  |  |  |  |  |  |  |
| Technology |  | Existing/Planned | State | County | Route | From MP | Dir | Location |
| RWIS |  | E | OR |  | 97 | 142.27 |  | Lava Butte |
| RWIS |  | E | OR |  | 20 | 80.77 |  | Santiam Pass |
| RWIS |  | E | OR |  | 20 | 104.62 |  | Riley Junction |
| RWIS* |  | E | OR |  | 20 | 20.00 |  | Horse Ridge |
| RWIS |  | P | OR |  | 97 | 150 |  |  |
| RWIS |  | E | OR |  | 58 | 62.07 |  |  |
| RWIS |  | E | OR |  | 58 | 53.00 |  |  |
| RWIS |  | E | OR |  | 58 | 72.63 |  |  |
| RWIS |  | P | OR |  | 58 | 62.5 |  |  |
| CCTV |  | P | OR |  | 97 | 135.97 |  |  |
| Information Kiosks |  | E | OR |  | 58 | 62.07 |  |  |
| Traffic Management Center |  | E | OR |  | 20 | 18.51 |  | Bend |