California/Oregon Advanced Transportation Systems

Regional Architecture

By

Katharine Hunter-Zaworski	Christopher Strong

Senior Research Engineer	Research Associate

of the

Western Transportation Institute

Civil Engineering Department

Montana State University - Bozeman

and

Ron Ice

R.C. Ice and Associates

Prepared for the

State of California

Department of Transportation

New Technology and Research Program

And

State of Oregon

Department of Transportation

Traffic Management Section

In cooperation with the

U.S. Department of Transportation

Federal Highway Administration

December 2000

Disclaimer

The opinions, findings and conclusions expressed in this publication are those of the authors and not necessarily those of the California Department of Transportation, Oregon Department of Transportation or the U.S. Department of Transportation, Federal Highway Administration.

Alternative accessible formats of this document will be provided upon request. Persons with disabilities who need an alternative accessible format of this information, or who require some other reasonable accommodation to participate, should contact the California Department of Transportation, Coco Briseño, Chief, Rural Branch, New Technology and Research Program, MS #83, P.O. Box 942873, Sacramento, CA 94273-0001; telephone number 916-654-6980, e-mail: Coco_Briseno@dot.ca.gov.

acknowledgements

Grateful appreciation is extended to the following agencies for their support, guidance and advice that facilitated the successful completion of this report. Specifically appreciated are the COATS Steering Committee; the various programs, divisions and districts/regions in the Oregon Department of Transportation and Caltrans; the California Highway Patrol; the Southern Oregon Visitors Association; and the U.S. Department of Transportation, Federal Highway Administration. In addition, thanks to all who took the time from their busy schedules to gather, organize and send the necessary materials. The authors of this report also thank all the regional champions and workshop participants who contributed significantly to the formulation of the COATS Regional Architecture.

This page is intentionally blank

- Glossary of Abbreviations

AASHTO	American Association of State Highway and Transportation Officials

APCD	Air Pollution Control District

AQMD	Air Quality Management District
ARTIC	Advanced Rural Technology Integration Center

ATIS	Advanced Traveler Information Systems

Caltrans	California Department of Transportation

CCTV	Closed-Circuit Television

CHIN	California Highway Information Network

CHP	California Highway Patrol

COATS	California/Oregon Advanced Transportation Systems

COG	Council of Governments

CVAS	Commercial Vehicle Administration Subsystem

CVC	Commercial Vehicle Check

CVO	Commercial Vehicle Operations

DART	Dimensional Associates Resources and Training

DSRC	Dedicated Short Range Communications

EM	Emergency Management

EMS	Emergency Medical Services

HAR	Highway Advisory Radio

HAZMAT	Hazardous Materials

HTCRS	Highway Travel Conditions Reporting System

IEEE	Institute of Electrical and Electronics Engineers

IFTA	International Fuel Tax Agreement

IRP	International Registration Plan

ISP	Information Service Provider

ITE	Institute of Transportation Engineers

ITS	Intelligent Transportation Systems

LTC	Local Transportation Commission

MCMIS	Motor Carrier Management Information System

MPO	Metropolitan Planning Organization

NTCIP	National Transportation Communications for ITS Protocol

ODOT	Oregon Department of Transportation

OSP	Oregon State Police

RTPA	Regional Transportation Planning Agency

RWIS	Road Weather Information System

SAE	Society of Automotive Engineers

SAFER	Safety and Fitness Electronic Records

SOVA	Southern Oregon Visitors Association

TC	Transportation Commission

TCIP	Transit Communications Interface Protocol

TEA-21	Transportation Equity Act for the 21st Century

TM	Traffic Management

TMC	Traffic Management Center

TOC	Transportation Operations Center

TRMS	Transit Management Subsystem

VMS	Variable Message Sign

WTI	Western Transportation Institute at Montana State University – Bozeman

Abstract

This report presents the regional intelligent transportation systems (ITS) architecture developed as a part of the California/Oregon Advanced Transportation Systems (COATS) project. The purpose of the architecture is to define a framework for how various systems and stakeholders within the COATS study region will interact in order to meet regional transportation challenges.

The COATS regional architecture follows the guidelines of the National ITS Architecture; therefore, this report provides a brief conceptual overview of the National ITS Architecture. Following this, the report describes efforts that were used to collect the inputs that were needed to develop the regional architecture. These include the Legacy Systems Report, the Conditions and Performance Report, the Project Infrastructure Report, the Traveler Needs Survey, several outreach and architecture workshops, and a Market Package Exercise.

The regional architecture has a number of components. It includes a high-level architecture for the COATS region. This architecture is further delineated at the subregional level in three subregional architectures. There are subregional architectures for southern Oregon, north central California, and the northern California coast, each of which has architecture flow diagrams included in this report. The regional architecture and related subregional architectures were developed through the U.S. Department of Transportation’s Turbo Architecture software package. The COATS Regional Architecture is designed to reflect the dynamic characteristics of the ITS architecture development process so all the databases are maintained in an electronic format for easy updating and revisions.

A project-level architecture is defined for the COATS early winner project, the Bi-State Traveler Safety and Incident Management System, which is centered around Siskiyou Pass. This is a segment of Interstate 5 that stretches from Yreka, California in the south to Medford, Oregon in the north. The project level architecture provides an example of how the high-level architecture may be used in developing projects and illustrates the opportunities for bi-state partnership and cooperation.

This report documents the first steps in the development of the ITS architecture for the COATS region. Next steps in the architecture development process include a major validation of the data and information flows by regional and subregional stakeholders, as well as updating the infrastructure reports to reflect changes as technology is implemented. This report will serve as background for the COATS ITS Strategic Deployment Plan.

This page is intentionally blank

Table of Contents

List of Tables	ix

List of Figures	x

1	Introduction	1

2	Definition of Architecture	3

2.1	National Architecture	3

2.1.1	Model of ITS Functions (Logical Architecture)	3

2.1.2	Model of ITS Physical Entities (Physical Architecture)	4

2.1.3	Communications	7

2.1.4	Integration of Architecture Layers	8

2.2	Regional Architecture	8

3	Methodology	11

3.1	Identification of Systems and Stakeholders in the COATS Region	11

3.2	Define Regional Needs and Challenges	15

3.2.1	Legacy Systems Report	15

3.2.2	Conditions and Performance Report	16

3.2.3	Traveler Needs Survey	16

3.2.4	Project Infrastructure Report	17

3.2.5	Outreach Workshops	17

3.3	Crafting the Architecture	18

3.3.1	COATS Market Packages	18

3.3.2	Communication & Information Survey	19

3.3.3	Regional Workshops	23

3.3.4	Architecture Review Process	25

3.3.5	Extending the Architecture	26

4	The COATS Regional Architecture	31

4.1	Defining the Regional Architecture for COATS	32

4.2	Common Systems and Interfaces	34

4.3	Subregion Interconnects	39

4.4	Subregional Architectures	41

4.4.1	Southern Oregon	43

4.4.2	North Central California	53

4.4.3	Northern California Coast	61

4.5	Subregional Architecture Flow Definitions	69

5	Early Winner Project	81

5.1	Project Background	81

5.2	Project Architecture	82

5.3	Related ITS Standards	86

6	Next Steps	97

References	99

Appendix A  Communications and Information Survey	101

Appendix B  Architecture Workshop Attendance	108

Appendix C  Inventory of Stakeholders and Subsystems	111

Appendix D  Architecture Flow Diagrams	133

# List of Tables

Table 3-1: Stakeholder Outreach Workshop Locations	12

Table 3-2: A High-Level Stakeholder List by Major ITS Stakeholder Area	14

Table 3-3: Market Package Responses	20

Table 3-4: Priority Listing of COATS Market Packages	21

Table 3-5: Comparison of ITS Technology Rankings	22

Table 3-6: Architecture Workshop Locations	23

Table 3-7: COATS Entity Extensions to National ITS Architecture	26

Table 3-8: COATS Flow Extensions to National ITS Architecture	28

Table 4-1: COATS Regional Architecture Interconnect Definitions	37

Table 4-2: Southern Oregon Inventory	54

Table 4-3: North Central California Inventory	54

Table 4-4: Northern California Coast Inventory	62

Table 4-5: Architecture Flows Included in COATS Regional Architecture	70

Table 5-1: Regional Architecture Flows Related to Early Winner Project	87

Table 5-2: ITS Standards Relevant to Early Winner Project	91

Table C-1: Inventory of Stakeholders and Systems	112

# List of Figures

Figure 1-1: COATS Study Area Boundaries	2

Figure 2-1: Top-Level Data Flow Diagram	5

Figure 2-2: High-Level Diagram of Physical Architecture	6

Figure 2-3: Relationship Between Physical and Logical Architectures	8

Figure 3-1: Methodology for Developing the COATS Regional Architecture	11

Figure 3-2: Identifying Stakeholders and Systems	12

Figure 3-3: Defining Regional Needs and Challenges	16

Figure 3-4: Crafting the COATS Regional Architecture	18

Figure 3-5: Non-Planned Event Scenario	24

Figure 3-6: Planned Event Scenario	24

Figure 4-1: COATS Architecture Subregions	33

Figure 4-2: Subsystems and Interconnects Applicable to the COATS Region	35

Figure 4-3: COATS Regional Architecture Systems and Interconnects	36

Figure 4-4: COATS Regional Interconnects	41

Figure 5-1: Define Early Project Architectures	81

Figure 5-2: Early Winner Project Architecture (Oregon)	83

Figure 5-3: Early Winner Project Architecture (California)	84

Figure 5-4: Potential Coordination Flow Between Medford TOC and Redding ARTIC	86

Figure 6-1: Development and Refinement of a Regional ITS Architecture	97

Figure D-1: Regional Diagram for 1-800-427-ROAD	139

Figure D-2: Regional Diagram for 1-800-977-ODOT	140

Figure D-3: Regional Diagram for Airline Schedule Operations	141

Figure D-4: Regional Diagram for Amtrak Schedule Operations	142

Figure D-5: Regional Diagram for California Highway Information Web Page	143

Figure D-6: Regional Diagram for Caltrans District 1 Public Affairs	144

Figure D-7: Regional Diagram for Caltrans District 2 Public Affairs	145

Figure D-8: Regional Diagram for Caltrans Field Equip District 1 (Eureka)	146

Figure D-9: Regional Diagram for Caltrans Field Equip District 2 (Redding)	147

Figure D-10: Regional Diagram for Caltrans HQ Construction Program	148

Figure D-11: Regional Diagram for Caltrans HQ Permits	149

Figure D-12: Regional Diagram for Caltrans HQ Traffic	150

Figure D-13: Regional Diagram for Caltrans Maintenance District 1 (Eureka)	151

Figure D-14: Regional Diagram for Caltrans Maintenance District 1 (Eureka) Vehicles	152

Figure D-15: Regional Diagram for Caltrans Maintenance District 2 (Redding)	153

Figure D-16: Regional Diagram for Caltrans Maintenance District 2 (Redding) Vehicles	154

Figure D-17: Regional Diagram for Caltrans RWIS	155

Figure D-18: Regional Diagram for Caltrans Weather Warning Systems	156

Figure D-19: Regional Diagram for Caltrans/CHP Eureka	157

Figure D-20: Regional Diagram for Caltrans/CHP Redding	158

Figure D-21: Regional Diagram for CHIN	159

Figure D-22: Regional Diagram for CHP Eureka and Humboldt Division Dispatch and 911 Centers	160

Figure D-23: Regional Diagram for CHP Northern District Dispatch and 911 Centers	161

Figure D-24: Regional Diagram for CHP Vehicles	162

Figure D-25: Regional Diagram for COATS Kiosks	163

Figure D-26: Regional Diagram for Coos Bay-Area City Traffic Control Equip	164

Figure D-27: Regional Diagram for Coos Bay-Area City Traffic Control Systems	165

Figure D-28: Regional Diagram for Coos Bay-Area County Maintenance Systems	166

Figure D-29: Regional Diagram for Coos Bay-Area E-911 Comm Centers	167

Figure D-30: Regional Diagram for Coos Bay-Area EMS Dispatch and Vehicles	168

Figure D-31: Regional Diagram for Coos Bay-Area Fire and Rescue Dispatch and Vehicles	169

Figure D-32: Regional Diagram for Coos Bay-Area Paratransit Dispatch and Vehicles	170

Figure D-33: Regional Diagram for Coos Bay-Area Planning Agencies	171

Figure D-34: Regional Diagram for Coos Bay-Area Police and Sheriff Dispatch and Vehicles	172

Figure D-35: Regional Diagram for Coos Bay-Area Radio Stations and Newspapers	173

Figure D-36: Regional Diagram for Coos Bay-Area Traveler Information Resources	174

Figure D-37: Regional Diagram for Coos Bay-Area Visitor Information Services	175

Figure D-38: Regional Diagram for DEQ Medford Regional Office	176

Figure D-39: Regional Diagram for Downtown Parking District	177

Figure D-40: Regional Diagram for Drivers	178

Figure D-41: Regional Diagram for Eureka-Area Air Districts	179

Figure D-42: Regional Diagram for Eureka-Area City Traffic Control Equip	180

Figure D-43: Regional Diagram for Eureka-Area City Traffic Control Systems	181

Figure D-44: Regional Diagram for Eureka-Area County Maintenance Systems	182

Figure D-45: Regional Diagram for Eureka-Area E-911 Comm Centers	183

Figure D-46: Regional Diagram for Eureka-Area EMS Dispatch and Vehicles	184

Figure D-47: Regional Diagram for Eureka-Area Fire and Rescue Dispatch and Vehicles	185

Figure D-48: Regional Diagram for Eureka-Area Fixed Route Dispatch and Vehicles	186

Figure D-49: Regional Diagram for Eureka-Area Paratransit Dispatch and Vehicles	187

Figure D-50: Regional Diagram for Eureka-Area Planning Agencies	188

Figure D-51: Regional Diagram for Eureka-Area Police and Sheriff Dispatch and Vehicles	189

Figure D-52: Regional Diagram for Eureka-Area Traveler Information Resources	190

Figure D-53: Regional Diagram for Eureka-Area Traveler Information Systems	191

Figure D-54: Regional Diagram for Eureka-Area TV and Radio Stations	192

Figure D-55: Regional Diagram for Eureka-Area Visitor Information Services	193

Figure D-56: Regional Diagram for Highway Travel Conditions Reporting System	194

Figure D-57: Regional Diagram for Inter-City Transit Dispatch and Vehicles	195

Figure D-58: Regional Diagram for Klamath Falls-Area City Traffic Control Equip	196

Figure D-59: Regional Diagram for Klamath Falls-Area City Traffic Control Systems	197

Figure D-60: Regional Diagram for Klamath Falls-Area County Maintenance Systems	198

Figure D-61: Regional Diagram for Klamath Falls-Area E-911 Comm Center	199

Figure D-62: Regional Diagram for Klamath Falls-Area EMS Dispatch and Vehicles	200

Figure D-63: Regional Diagram for Klamath Falls-Area Fire and Rescue Dispatch and Vehicles	201

Figure D-64: Regional Diagram for Klamath Falls-Area Fixed Route Dispatch and Vehicles	202

Figure D-65: Regional Diagram for Klamath Falls-Area Paratransit Dispatch and Vehicles	203

Figure D-66: Regional Diagram for Klamath Falls-Area Planning Agencies	204

Figure D-67: Regional Diagram for Klamath Falls-Area Police and Sheriff Dispatch and Vehicles	205

Figure D-68: Regional Diagram for Klamath Falls-Area Traveler Information Resources	206

Figure D-69: Regional Diagram for Klamath Falls-Area TV, Radio Stations and Newspapers	207

Figure D-70: Regional Diagram for Klamath Falls-Area Visitor Information Services	208

Figure D-71: Regional Diagram for Mayday Service Provider	209

Figure D-72: Regional Diagram for Medford-Area City Traffic Control Equip	210

Figure D-73: Regional Diagram for Medford-Area City Traffic Control Systems	211

Figure D-74: Regional Diagram for Medford-Area County Maintenance Systems	212

Figure D-75: Regional Diagram for Medford-Area E-911 Comm Centers	213

Figure D-76: Regional Diagram for Medford-Area EMS Dispatch and Vehicles	214

Figure D-77: Regional Diagram for Medford-Area Fire and Rescue Dispatch and Vehicles	215

Figure D-78: Regional Diagram for Medford-Area Fixed Route Dispatch and Vehicles	216

Figure D-79: Regional Diagram for Medford-Area Paratransit Dispatch and Vehicles	217

Figure D-80: Regional Diagram for Medford-Area Planning Agencies	218

Figure D-81: Regional Diagram for Medford-Area Police and Sheriff Dispatch and Vehicles	219

Figure D-82: Regional Diagram for Medford-Area Traveler Information Resources	220

Figure D-83: Regional Diagram for Medford-Area TV and Radio Stations	221

Figure D-84: Regional Diagram for Medford-Area Visitor Centers	222

Figure D-85: Regional Diagram for Medford-Area Visitor Information Services	223

Figure D-86: Regional Diagram for ODOT Communications Division	224

Figure D-87: Regional Diagram for ODOT Field Equip (Medford TOC)	225

Figure D-88: Regional Diagram for ODOT Maintenance Districts (Medford TOC)	226

Figure D-89: Regional Diagram for ODOT Maintenance Vehicles (Medford TOC)	227

Figure D-90: Regional Diagram for ODOT Medford TOC	228

Figure D-91: Regional Diagram for ODOT RWIS Server	229

Figure D-92: Regional Diagram for ODOT Statewide Planning	230

Figure D-93: Regional Diagram for ODOT Weather Warning System	231

Figure D-94: Regional Diagram for OSP Dispatch	232

Figure D-95: Regional Diagram for OSP Vehicles	233

Figure D-96: Regional Diagram for Other Weather Data	234

Figure D-97: Regional Diagram for Redding-Area Air Districts	235

Figure D-98: Regional Diagram for Redding-Area City Traffic Control Equip	236

Figure D-99: Regional Diagram for Redding-Area City Traffic Control Systems	237

Figure D-100: Regional Diagram for Redding-Area County Maintenance Systems	238

Figure D-101: Regional Diagram for Redding-Area E-911 Comm Centers	239

Figure D-102: Regional Diagram for Redding-Area EMS Dispatch and Vehicles	240

Figure D-103: Regional Diagram for Redding-Area Fire Dispatch and Vehicles	241

Figure D-104: Regional Diagram for Redding-Area Fixed Route Dispatch and Vehicles	242

Figure D-105: Regional Diagram for Redding-Area Paratransit Dispatch and Vehicles	243

Figure D-106: Regional Diagram for Redding-Area Planning Agencies	244

Figure D-107: Regional Diagram for Redding-Area Police and Sheriff Dispatch and Vehicles	245

Figure D-108: Regional Diagram for Redding-Area Traveler Information Resources	246

Figure D-109: Regional Diagram for Redding-Area Traveler Information Systems	247

Figure D-110: Regional Diagram for Redding-Area TV and Radio Stations	248

Figure D-111: Regional Diagram for Redding-Area Visitor Information Services	249

Figure D-112: Regional Diagram for SOVA Kiosks	250

Figure D-113: Regional Diagram for Transit Users	251

Figure D-114: Regional Diagram for Traveler Information Devices	252

Figure D-115: Regional Diagram for Travelers	253

Figure D-116: Regional Diagram for TripCheck	254

Figure D-117: Regional Diagram for Vehicle Information Systems	255

Figure D-118: Regional Diagram for Vehicle Mayday Systems	256

Figure D-119: Regional Diagram for Vehicles	257

Figure D-120: Regional Diagram for Virtual Transit Mall	258

This page is intentionally blank

# Introduction

Intelligent Transportation Systems (ITS) are developing rapidly, but the implementation of these advanced technologies in rural areas has not advanced as quickly as it has in more urbanized areas. Two reasons for this are a lack of awareness by local officials and stakeholders of the technological opportunities, and the lack of a mechanism for setting local and regional priorities for the implementation of systems that may benefit the traveling public, as well as state and local officials. In order for the benefits of ITS to be realized in rural areas, these barriers need to be overcome at a regional level.

This need led to the creation of an ITS planning, deployment and evaluation initiative called the California/Oregon Advanced Transportation Systems (COATS) project. The purpose of COATS is to encourage regional, public and private sector cooperation between California and Oregon organizations to better facilitate the planning and implementation of advanced technology for the region’s transportation system. The approximate area of focus is between Redding, California and Eugene, Oregon. Figure 1-1 is a map of the COATS study area. This area provides a test bed for research, development, demonstration and deployment of rural ITS technologies and systems to “make rural travel safe, dependable and convenient.” (1)

The purpose of this report is to define an ITS architecture for the COATS region that may serve as a master blueprint for an integrated, multi-modal intelligent transportation system to address transportation challenges in the study area. The architecture clarifies how organizations, both public and private, can work together to deliver a safer, more efficient and convenient transportation system to the rural traveler through deployment of ITS improvements. This report describes the regional architecture in a manner that is consistent with both the National ITS Architecture and the transportation needs of the COATS region.

In addition to the regional architecture, this report documents the process by which the architecture was developed, and suggests rural extensions to the National ITS Architecture that were necessary to support COATS Regional Architecture. Beyond a regional architecture, subregional architectures are presented for three subregions within the study area. In addition, a more-detailed, project-level architecture is presented for the early winner project at Siskiyou Pass, to illustrate how this architecture can highlight the need for improved coordination between organizations across the state border.

This document describes the initial development of the architecture and is the first step in the sustained planning and integration effort that underlies most of the cited benefits of having a regional architecture. The architecture itself, however, will require validation and updating over time to more accurately reflect local needs. To facilitate this future validation effort, an Internet-accessible database was created using the U.S. Department of Transportation’s Turbo Architecture software package. The database allows many different views and focused reports from the regional architecture to be generated, which should facilitate its use and maintenance in the future.

Figure 1-1: COATS Study Area BoundariesFigure 1-1: COATS Study Area Boundaries.

# Definition of Architecture

An ITS architecture provides a conceptual framework for the design and operation of intelligent transportation systems. It is neither a system design nor a design concept; rather, it defines the framework around which multiple design approaches can be developed, each one specifically tailored to meet the needs of individual stakeholders, while maintaining the benefits of a common architecture. The architecture defines:

- the functions, such as gathering traffic information, that must be performed to implement a given user service;

- the physical entities or subsystems where these functions reside, such as the roadside or the vehicle;

- the interfaces/information flows between the physical subsystems; and

- the communication requirements for the information flows (e.g., wireline or wireless).

In addition, the architecture identifies and specifies the requirements for the standards needed to support national and regional interoperability, as well as product standards needed to support economy of scale considerations in deployment.

## National Architecture

The National ITS Architecture, developed under contract with the U.S. Department of Transportation, is the standard for describing the interrelationship of organizations and systems under specific ITS projects. The architecture may be considered from three primary views:

- the logical architecture, which presents a functional view of ITS;

- the physical architecture, which partitions the functions reflected within the logical architecture into systems and subsystems where functions are actually performed; and

- communications, which connect the various systems.

The following sections will describe each of these components in more detail.

### Model of ITS Functions (Logical Architecture)

One critical element in the successful deployment of ITS is to explicitly consider what the ITS technologies are supposed to accomplish. In the architecture, this is primarily reflected in the logical architecture.

The logical architecture serves to present a functional view of the ITS user services, which in turn represent what ITS should accomplish from a user’s perspective. The National ITS Architecture defines the logical architecture as being divorced from likely implementations and physical interface requirements. By doing this, the architecture is open and flexible enough to handle most legacy systems, and does not inordinately constrain the market into certain technologies. At a more detailed level, the logical architecture defines the functions or process specifications that are required to perform ITS user services, and the information or data flows that need to be exchanged between these functions.

The first step in developing a logical architecture is to define a boundary separating those elements that are inside the architecture from those that are not. This is critical because ITS systems typically include many stakeholders who perform many functions not directly related to the transportation system. Therefore, an ITS architecture that does not appropriately define a boundary quickly becomes unmanageable without any accompanying benefit. Those entities that are external to the architecture are called terminators. The following are some examples of how a boundary is defined for the logical architecture.

- Travelers are considered to be external to the architecture, but the equipment that they use to obtain information (such as a variable message sign) or provide inputs to other systems (such as the vehicle’s speed) is inside. In other words, the architecture defines the functions ITS must perform in support of a traveler’s requirements, not the functions of the traveler. The decision processes employed by a traveler in deciding how to undertake a particular trip – such as mode, route, time of departure – are not included within the architecture boundary.

- A financial institution that processes tolls is outside of the architecture, whereas the ITS components that detect vehicles and collect tolls are inside the architecture.

- Existing broadcast media for the transmission of traveler information, such as radio and television, are outside of the architecture, but the elements that provide ITS traveler information to the media are inside.

- Communications within the rail infrastructure – between the cab and the dispatch center, for example – are outside the architecture but the support for the highway-rail interface is inside.

Functions that are to be performed by the intelligent transportation system are selected to represent the requirements of stakeholders and users. These functions are decomposed until they can be mapped to user service requirements that are identified in the National ITS Architecture. These user service requirements, in turn, may be decomposed and tailored in order to develop functional specifications for planned ITS technologies.

Figure 2-1 shows a simplified top-level data flow diagram. The circles represent functions, which may be decomposed into lower levels of detail, and rectangles represent terminators, some of which were described earlier. The lines between functions, and between functions and entities external to the architecture, represent data flows, with arrows indicating the direction of data flow. The data flows may also decomposed. The lowest level of detail of functions and data flows may be used to define detailed functional requirements for new ITS deployments, which is critical to any ITS planning effort.

### Model of ITS Physical Entities (Physical Architecture)

The logical architecture is useful in developing functional specifications, but it does not provide any information about where functions are to be performed. Therefore, the National ITS Architecture defines a physical architecture, which distributes the functions defined by the logical architecture into systems and subsystems, based on the similarity of functions and the location where functions are to be performed.

A high-level diagram of the physical architecture, commonly referred to as a “sausage diagram,” is shown in Figure 2-2. The physical architecture defines four types of systems:

- center subsystems, which deal with those functions normally assigned to public/private administrative, management, or planning agencies;

- roadside subsystems, which involve the deployment of sensors, signals, programmable signs, or other interfaces with travelers and vehicles that must be positioned in a roadside location;

- vehicle subsystems, which are installed in a vehicle; and

- traveler subsystems, which provide interfaces to travelers.

These four types of systems are partitioned into nineteen subsystems. The specific choice of nineteen subsystems represents a lower level of partitioning of functions that is intended to capture all anticipated subsystem boundaries for the present, and twenty years into the future.

Figure 2-1: Top-Level Data Flow DiagramFigure 2-1: Top-Level Data Flow Diagram.

Subsystems are physical only in a relative sense. They can be aggregated, with several subsystems located in the same physical space, such as a traffic management center being collocated with a highway patrol dispatch office. The subsystems may also be replicated, with multiple organizations performing the functions of a given subsystem. This may occur within a small geographic area where, for example, several different organizations may assist in emergency management functions. This can also occur due to the overlapping of geographic jurisdictions as, for example, different parts of the roadside ITS infrastructure in a particular county may be operated and maintained by state, county or city transportation departments.

Figure 2-2: High-Level Diagram of Physical ArchitectureFigure 2-2: High-Level Diagram of Physical Architecture.

Subsystems may be decomposed into equipment packages, which are deployment-oriented pieces within a given subsystem. Equipment packages are defined to support analyses and deployment, and they represent the smallest units within a subsystem that might be purchased. For example, the emergency management subsystem includes five equipment packages: emergency call-taking, emergency data collection, emergency dispatch, emergency response management, and mayday support. There are approximately 140 equipment packages defined in the National ITS Architecture, providing a broad range of options for staged implementation.

### Communications

The National ITS Architecture also provides the framework that ties the transportation and telecommunication worlds together to enable the development and effective implementation of the broad range of ITS user services. There are multiple communications options available to the system designer. The flexibility in choosing between various options allows each implementer the ability to select the specific technology that meets local, regional, or national needs. The architecture identifies and assesses the capabilities of candidate communications technologies, but it does not select or recommend “winning” systems and technologies.

The National ITS Architecture defines four communication media types to support the communications requirements between the nineteen subsystems in the physical architecture. These media types are:

- wireline communications (fixed-point to fixed-point),

- wide-area wireless communications (fixed-point to mobile),

- dedicated short-range communications (fixed-point to mobile), and

- vehicle-to-vehicle (mobile to mobile).

Wireline media, as defined in the architecture, could include a wireless communication technology, such as a microwave link between a roadside sensor and a remote regional server. The important factor in dividing communications technologies is whether the source and/or recipient are fixed or mobile. Three of the four media types are exclusively wireless media; these are distinguished as follows.

- Wide-area wireless communications and dedicated short-range communications are distinguished by range and area of coverage. Wide-area wireless communications are appropriate for instances where information is disseminated to users who are not located near the source of transmission and who require seamless coverage. An example of this type of coverage would be broadcast transmission received over AM or FM radio.

- Dedicated short-range communications is concerned with information transfer at a localized level between a mobile source and a fixed point. Applications using this media could include parking fee collection and commercial vehicle credential checks.

- Vehicle-to-vehicle communications refer to media between two mobile sources. At a very primitive level, this could include a cellular telephone call between two vehicles. The National ITS Architecture envisions this as being required to support automated highway system applications, and more specifically, intersection collision avoidance applications.

One of the fundamental guiding philosophies in the development of the National ITS Architecture has been to build upon existing and emerging transportation and communication infrastructures in its design. This minimizes the risk and cost of deployment. Consequently, the decision on optimal communications media for specific ITS applications may be governed by functionality and cost rather than an arbitrarily selected technology. This is especially critical for the rural COATS region, where many communications media taken for granted in urban environments have spotty areas of coverage.

Integration of Architecture Layers

Figure 2-3: Relationship Between Physical and Logical ArchitecturesFigure 2-3: Relationship Between Physical and Logical Architectures.

The logical, physical and communications aspects of the architecture do not represent separate architectures; rather, they represent different views of the same entity. Figure 2-3 shows how the logical and physical architectures, when decomposed far enough, will reflect the same underlying ITS functionality.

## Regional Architecture

The National ITS Architecture is designed to be implemented at a sub-national level – state, regional or local. The remainder of this report will describe how the National ITS Architecture was used in developing a regional architecture for the COATS study area. The benefits of a regional architecture that demonstrates conformance with the National ITS Architecture include the following.

- Section 5206(e) of the Transportation Equity Act for the 21st Century (TEA-21) requires that ITS projects using funds from the Highway Trust Fund conform to the National ITS Architecture and standards. The COATS Regional Architecture presented in this report meets or exceeds the interim policy guidance for architecture conformity from US DOT, maximizing the opportunity to leverage federal funding.

- A regional architecture facilitates regional integration. It helps agencies and other stakeholders to identify and plan for the many integration and information sharing opportunities which ITS offers.

- A regional architecture that conforms with the National ITS Architecture (including appropriate ITS standards) enables other ITS systems that will be developed for use throughout the U.S. to operate with systems in the COATS region.

- Transportation improvements in the region are typically made incrementally as funding becomes available for various project components. A regional architecture provides guidance for how these projects should fit together, improving interoperability between the projects, making efficient use of scarce resources, and facilitating future ITS expansion in the region.

These benefits will only be realized if the regional architecture is used in planning projects for the region and adapted as requirements change in the future. Therefore, developing a regional architecture for the COATS study area is critical to the project’s long-term success.

This page is intentionally blank

# Methodology

The value of developing a regional ITS architecture having been established in the previous chapter, this chapter describes the methodology that was used for developing the COATS architecture.

Figure 3-1: Methodology for Developing the COATS Regional ArchitectureFigure 3-1: Methodology for Developing the COATS Regional Architecture.The activities that contributed to the development of the COATS Regional Architecture are shown in Figure 3-1. The three main activities have included identification of the stakeholders and systems, definition of regional needs and challenges, and crafting of a regional architecture. Each of these activities will be discussed in greater detail in this chapter.

## Identification of Systems and Stakeholders in the COATS Region

As shown in Figure 3-2, an ongoing activity required in the development of the regional architecture is engaging stakeholders and building consensus. The stakeholder base in the COATS region is much broader than in many other ITS architectures, and this is a reflection of the character of the COATS project and region. Consequently, identification of stakeholders and systems has been an ongoing process throughout the COATS project.

The first significant stakeholder outreach was a series of workshops held at the beginning of the COATS project (2). The purpose of these workshops was to familiarize stakeholders with ITS and the COATS Project, as well as to validate needs and challenges in the COATS region.

Four stakeholder outreach workshops were held throughout the COATS study area; workshop locations and dates are summarized in Table 3-1. These workshops were intended to familiarize stakeholders with potential ITS applications as well as to familiarize the project’s sponsors, the California and Oregon Departments of Transportation (Caltrans and ODOT, respectively) and the Western Transportation Institute (WTI) with the issues that are perceived to be problematic throughout the region. Outreach workshops were conducted in conjunction with regular Steering Committee meetings. Staff worked with state, local, and regional transportation stakeholders to identify appropriate workshop attendees. Invitations to the workshops were issued by letter and followed up with phone calls to key persons to optimize workshop participation. Caltrans and ODOT staff also did outreach presentations prior to the workshops to increase participation.

Table 3-1: Stakeholder Outreach Workshop LocationsTable 3-1: Stakeholder Outreach Workshop Locations.

Figure 3-2: Identifying Stakeholders and SystemsFigure 3-2: Identifying Stakeholders and Systems.Subsequent outreach efforts, such as other Steering Committee meetings and architecture workshops, sought to expand the circle of identified stakeholders and systems in the COATS region.

Through these efforts, existing and future systems in the region were identified along with the public agencies and private interests that are responsible for each system’s operation and maintenance. These identified systems and stakeholders were mapped to the National ITS Architecture. By definition, the COATS Regional Architecture is based on the systems that exist in the region today. Building on this established base, the regional architecture defines how these existing or legacy systems can be integrated with each other and with future systems. Since the regional architecture is so dependent on the inventory of what exists and what is planned for the region, the definition of a regional architecture begins with an inventory of the existing and future systems that will be deployed. These systems together provide the advanced transportation services that will be offered in the COATS region. These systems, and the stakeholders who are responsible for their operation and maintenance, are the heart of the regional architecture.

A high-level stakeholder list and a system inventory by major ITS stakeholder area is presented in the following Table 3-2. Each stakeholder group is mapped to the corresponding National ITS Architecture entity. A list of systems for each stakeholder group is also tabulated. Where possible, the mapping is maintained at a high level, mapping general categories of agencies (e.g., State Departments of Transportation) to corresponding elements in the National ITS Architecture. This level of abstraction allows the regional architecture to be more easily maintained and understood. It also highlights the common interfaces and requirements that are shared by stakeholders in the region.

In general, it is better to define a few general interfaces that accommodate the range of regional implementations rather than specifying a unique architecture connection for each physical interface that may be implemented. The objective is to identify the major classes of systems and system interfaces in the region.

For purposes of discussion, the COATS architecture is organized at three levels:

- the COATS region as a whole;

- three subregional architectures – southern Oregon, north central California, and the northern California coast; and

- the early winner project architecture, focused on Siskiyou Pass.

The subregional systems include city, county and state dispatch centers for departments of transportation or public works, operations and maintenance, regional traffic control centers, law enforcement, fire and rescue, emergency management, 911 emergency communication centers, hazardous materials (HAZMAT), and emergency medical services. In the COATS region there are also specific local stakeholders in the subregions. For example, in the coastal areas, the U.S. Coast Guard may be the lead agency in emergency management, and in northern California, the California Department of Forestry is a lead agency in fire emergency management. Other regional systems include ITS infrastructure for Commercial Vehicle Operations (CVO) such as weigh-in-motion, Oregon’s Green Light weight station preclearance system, and other

Table 3-2: A High-Level Stakeholder List by Major ITS Stakeholder AreaTable 3-2: A High-Level Stakeholder List by Major ITS Stakeholder Area.

| Stakeholder Groups | Architecture Groups | Systems |
| --- | --- | --- |
| Law Enforcement - Highway Patrol - State Police - County Sheriffs - City Police | Emergency Management | - Law Enforcement Dispatch Systems - 911 Communication Centers - Emergency Coordinators |
| Emergency Services - Fire/Rescue - Ambulance - Emergency Medical Services - Hospitals/Clinics - HAZMAT - Towing | Emergency Management | - Emergency Operations - Fire District Dispatch - Emergency Medical Dispatch |
|  | Emergency Vehicle |  |
| Depts of Transportation - State: Operations and Maintenance - City: Operations and Maintenance - Traffic/Transportation Management Centers - Transportation Operation Centers | Traffic Management | - Traffic Management Centers - Advanced Rural Technology Integration Centers - Transportation Operations Centers |
|  | Roadway: Construction and Maintenance | - Dispatch Centers |
| Planning Agencies - Regional Transp. Planning Agencies - Metropolitan Planning Organizations - Bureau of Land Management - U.S. Forest Service - Bureau of Indian Affairs | Information Service Users | - Visitor Information Services |
| Parks and Recreation - National Parks - State Parks - City and County - Recreation Areas (ski resorts, summer resorts) | Information Service Providers and Users | - Visitor Information Services - Traveler Information Resources |
| Tourism - State and Local Visitor Centers | Information Service Providers and Users | - Visitor Information Services - Traveler Information Resources |
| Trucking - California Trucking Association - Green Light - HELP | Commercial Vehicle Operations | - Carrier Dispatch - Green Light - IRP Clearinghouse - IFTA Clearinghouse |
| Motorists - AAA - Insurance Agencies | Information Service Providers and Users | - Mayday Systems - Vehicle Information Systems - Vehicle Safety Systems |
| Public Transportation - Local bus: Fixed route, school bus - Demand Responsive Service (HHS, DoED seniors) - TDM: Rideshare, Shuttles - Inter-City (over-the-road coaches) - AMTRAK - Regional Air Carriers - Parking Management | Transit Management | - Transit Dispatch |
|  | Transit Vehicle | - Vehicle Information Systems - Vehicle Safety Systems |
|  | Transit Operator | - Transit Dispatch |
|  | Multimodal Service Provider | - Traveler Information Resources |
| Media - Radio, Television - CCTV, Internet, Newspaper | Media | - TV and Radio Stations |
|  | Information Provider | - Traveler Information Resources |

credentialing systems. The Commercial Vehicle Operations along the Interstate 5 corridor are part of another major ITS initiative covering several western states and therefore are not covered in this project. There are a number of major tourist destinations in the COATS region, including State and National Parks and Monuments as well as several scenic byways. The Visitor Information Services and Traveler Information Resources systems meet the needs of the stakeholders associated with planning agencies, parks and recreation organizations, tourism, public transportation, and the media. A number of systems are associated with public transportation services including fixed-route, demand-responsive, inter-city and intermodal transportation services. These systems include: transit dispatch, vehicle information systems, vehicle safety systems, as well as traveler information resources. In the future, the motorists in the COATS region will also have several systems available, including Mayday systems, vehicle information systems and vehicle safety systems.

## Define Regional Needs and Challenges

There were five work products and associated activities that were used as a basis for defining the regional needs and challenges. These include the Legacy Systems Report, Conditions and Performance Report, the Project Infrastructure Report, Traveler Needs Survey as well as the outreach workshops (Figure 3-3). A short summary of each of these activities is provided below.

### Legacy Systems Report

The Legacy Systems Report (3) summarizes existing transportation systems and planned transportation improvements within the COATS region and includes them in GIS maps. The term “legacy” includes not only existing transportation systems, projects and programs, but also those systems which, at the time of publication, were not yet built but were planned and had committed funding.

The three main steps in developing the Legacy Systems Report were to: 1) make contacts and gather information regarding legacy systems, 2) review and organize information and 3) tabulate and geographically map important transportation systems/solutions. Information collected from project participants and stakeholders was organized into two main categories: advanced technology-related projects, programs and improvements, and traditional projects, programs and improvements. The traditional projects, programs, and improvements were divided into three categories: highway, intermodal, and commercial vehicle operations. The appendices in the Technical Memorandum contain the organized data from the surveys and personal interviews. Maps were formed using this information to show the geographic locations of these technologies.

Information contained in this report cannot, by itself, be used to make recommendations regarding the implementation of ITS-related projects and programs. Rather, this information was gathered to document previous and future efforts to avoid duplication of programs or projects.

Conditions and Performance Report

Figure 3-3: Defining Regional Needs and ChallengesFigure 3-3: Defining Regional Needs and Challenges.

The Conditions and Performance Report, which in combination with the Legacy Systems Report comprises Technical Memorandum One, identifies and defines potential transportation challenges within the COATS study area (4). The report provides an overview of the major transportation challenges in the region, and identifies a geographic area of focus for each of the challenges. These challenges include safety problems, congestion, freight movement, incident response, mobility, tourism, environmental impacts and deteriorating infrastructure.

### Traveler Needs Survey

A traveler needs survey was conducted as an early task in the COATS project (5, 6). The specific objectives of the survey were to determine:

- what information the rural traveler needs and wants,

- the medium through which information would be presented to the traveler, and

- where the traveler would want this information presented.

The survey’s purpose was to identify and investigate the transportation and information needs of motorists in northern California and southern Oregon. In addition, the survey investigated traveler acceptance of ITS applications and new user services throughout the study area. The results, along with other information, allow decisions to be made as to what technologies could be applied to the roadway or in surrounding areas to help disseminate information to the traveler. Understanding the information needs of travelers, as well as their willingness to use new ITS-related systems, was one of the objectives of the COATS project.

### Project Infrastructure Report

The purpose of the Project Infrastructure Report was to propose a list of potential advanced technology infrastructure enhancements that can be implemented in the COATS study area (7). The report considered input from organizations within the project limits responsible for transportation, enforcement, tourism, commercial vehicle operations, transit and fleet management, and incorporated their needs in order to prioritize infrastructure, early winner projects and ultimately strategic deployment. The report details the potential ITS infrastructure elements within the project limits. For each infrastructure element, this report provides:

- a description of the proposed ITS technology,

- a list of objectives that the technology attempts to address,

- location selection criteria,

- maps of potential locations, and

- an appendix detailing locations of recommended deployment shown on the maps.

Potential infrastructure elements were identified based on the qualitative (e.g. stakeholder input) and quantitative (3, 4) challenges identified in other project tasks.

To ensure that locations for infrastructure have the greatest potential benefit, many of the criteria used to find locations are based on previously identified challenges. The locations were occasionally modified to reflect stakeholder and steering committee input on problem areas.

### Outreach Workshops

Through the outreach workshop sessions, stakeholders provided information for Caltrans, ODOT and WTI that was helpful in validating regional transportation needs. WTI analyzed this information in order to develop rural ITS early winner project candidates (8). ITS applications selected for demonstration are those identified in the COATS effort as the most viable ITS candidates for early deployment.

The stakeholders were divided into groups based on their area of interest. The groups that were formed focused on Safety and Emergency Services, Travel and Tourism, Operations and Maintenance, or Commercial Vehicle Operations. The groups were asked to list the challenges that they perceived existed in their area. Based upon these challenges, the stakeholders were asked to list the opportunities that they felt existed to remedy the various problems.

## Crafting the Architecture

The crafting of the regional architecture involved three major activities, as shown in Figure 3-4: the Market Package Exercise, the Communication and Information Survey, and the six regional architecture workshops. These activities were conducted after the identification of stakeholders and systems and the definition of regional needs and challenges.

COATS Market Packages

Figure 3-4: Crafting the COATS Regional ArchitectureFigure 3-4: Crafting the COATS Regional Architecture.

The National ITS Architecture defines 65 market packages that represent a range of potential ITS implementations that address specific transportation challenges. “Broadcast Traveler Information,” “Road Weather Information Systems,” and “Transit Vehicle Tracking” are examples of the market packages that have been defined. The market packages combine this service-oriented view of ITS with a detailed identification of the pieces of the National ITS Architecture that are required to implement these services. For instance, the “Broadcast Traveler Information” market package identifies the pieces of the architecture that collect traveler information and broadcast this information, and the devices used by the travelers to receive these broadcasts. This combination of service-oriented and architecture-oriented views makes the market packages a useful entry point to the National ITS Architecture definition. They provide an accessible, service-oriented view of the potential for ITS that can be used to build consensus on what needs to be done in a region. This service-oriented view is easily converted into a more technical architecture view because of the connectivity between market packages and architecture definition. The market packages are used in this chapter as a convenient way to define the transportation services that the COATS Regional Architecture will provide.

Members of the Steering Committee reviewed the market packages identified in the National ITS Architecture and selected a set of priority market packages that best address the COATS regional challenges. Table 3-3 lists all of the market packages, indicating which packages were prioritized by Steering Committee members. The data compiled in the table are from COATS Steering Committee members, where “O” refers to Oregon respondents, “C” refers to California respondents, and “O/C” refers to respondents from both Oregon and California. As can be seen, there was substantial agreement between the two states’ respondents.

Responses to this Market Package Exercise were tabulated in order to develop a priority listing of the market packages, as shown in Table 3-4. Higher rankings indicate that a market package has greater applicability for the COATS region than other market packages. Steering Committee members from both Oregon and California were in agreement on the priority ranking of the market packages.

From the information provided by the stakeholders, a comparison was made between the feedback given from the various methods of data collection. The methods included the Traveler Needs Survey, the Market Package Exercise, the Project Infrastructure Report, as well as the stakeholder workshops. The result of this comparison is summarized in Table 3-5. The table consists of the ITS opportunities that surfaced from the various exercises conducted.

The opportunities were assigned a score of importance with respect to the other opportunities that were considered, with lower numbers indicating a higher priority. The rankings varied according to the methods of the exercise. The Traveler Needs Survey results were easily quantified based on the averages of the survey responses. The Market Package Exercise also allowed for an absolute ranking based on the score that each market package received. The stakeholder outreach, however, was not scored so the ranking of the opportunities was not easily achievable. The items were assigned either a ranking of 1 or 2 depending on whether a particular opportunity was recognized by at least three of the four workshops.

### Communication & Information Survey

In October 1999, a survey was distributed to key stakeholders in the COATS region to identify their existing and desired information exchange activities with other organizations. (The survey form is included as Appendix A.) The survey was intended to have responding organizations identify:

- how they currently receive and provide various types of transportation information, and

- how they would like to receive and provide these types of information in the future.

Respondents were asked to list organizations and/or technologies along with their associated communication methods for each of several types of transportation-related

Table 3-3: Market Package ResponsesTable 3-3: Market Package Responses.

Table 3-3: Market Package Responses (cont.).

Table 3-4: Priority Listing of COATS Market PackagesTable 3-4: Priority Listing of COATS Market Packages.

information. The survey was distributed to dozens of organizations in both California and Oregon, representing a broad mix of stakeholders.

Table 3-5: Comparison of ITS Technology RankingsTable 3-5: Comparison of ITS Technology Rankings.

In reviewing survey responses, it was evident that these would not be adequate by themselves to develop a regional architecture. One problem was that the response rate was lower than what would be appropriate for developing a comprehensive regional architecture: of 216 surveys mailed out, there were 72 responses, representing a 33 percent response rate. Moreover, among the surveys that were received, some were incomplete, and others were vague in defining technologies and/or organizations. Follow-up telephone calls to selected stakeholders identified by the Steering Committee and Regional Teams improved the quality of the data received, such that the responses yielded a helpful starting point for understanding the communications and information needs of stakeholders in the COATS region.

### Regional Workshops

A series of six architecture workshops was held in order to build upon the results gathered in the Communication and Information Survey. Workshop sites and dates are shown in Table 3-6. The purpose of these workshops was twofold:

Table 3-6: Architecture Workshop LocationsTable 3-6: Architecture Workshop Locations.

- to confirm information flows and stakeholders identified in the survey, and

- to identify missing information flows and stakeholders.

This is largely parallel to the information that was requested in the survey. It was felt that the workshops would improve upon the results of the survey in two primary ways. First, the workshops would attract stakeholders who did not receive or respond to the survey. Second, by assembling participants from a broad cross-section of transportation, law enforcement, transit, emergency management and response, tourism and other organizations, the workshops could spawn discussion that would help to identify information needs not originally considered. Invitations for each workshop were extended by standard mail to stakeholders identified by Caltrans and ODOT staff. A list of stakeholders in attendance at each workshop is provided as Appendix B.

The workshop format consisted of an educational component as well as a discussion component. The educational and outreach segment was necessary in part because these workshops represented the first COATS meetings in two of the cities, Klamath Falls and Ukiah. It was also necessary because many workshop participants had little or no familiarity with the COATS project or ITS before the workshop. This segment of the workshop consisted of a series of presentations discussing needs and challenges identified in the COATS study area, as well as how ITS may be used to address those challenges.

Figure 3-5: Non-Planned Event ScenarioFigure 3-5: Non-Planned Event Scenario.

Figure 3-6: Planned Event ScenarioFigure 3-6: Planned Event Scenario.

To guide the discussion, two scenarios – a non-planned event (such as a weather-related traffic incident) and a planned event (such as a summer festival) – were presented (Figures 3-5 and 3-6). The purpose of the scenarios was to provide a real-world, tangible setting to assist stakeholders in assessing their communication and information needs. For each scenario, a vision was presented based on subsystems from the National ITS Architecture, presenting how organizations could work together to keep travelers informed and to collect operational data. After presenting the vision, a moderated discussion sought to identify each organization’s informational needs and their organization’s informational availability. In addition to generating information useful in compiling a regional architecture, these discussions served to open or improve communications channels between organizations that often do not sit in the same room.

Subsequent to this, a list of stakeholders identified through previous outreach and research efforts was presented to workshop attendees for their review. Stakeholders were organized according to the National ITS Architecture’s nineteen subsystems. This review helped to ensure that all pertinent parties were accounted for in the development of the regional architecture.

The educational and outreach benefit of these six workshops is difficult to assess, although it was clear that both the understanding and interest level of stakeholders in the COATS project increased as a direct result of the workshops. Some of the primary benefits of these workshops for completing the regional architecture include the following.

- It brought a sharper focus on the local impacts of regional needs and challenges that had been identified through earlier COATS work tasks.

- It confirmed that data obtained through the communication and information survey was reliable and essentially complete.

- It demonstrated that a highly detailed regional architecture would be difficult to obtain without significant additional time and resources, due to the multiplicity of local organizations, especially for law enforcement, emergency response and planning.

### Architecture Review Process

The initial draft architecture was based strictly on the National ITS Architecture and did not provide any customization based on regional needs. A draft architecture was complete by June 2000 and presented to the Steering Committee for review and comment. It was hoped that the review process would help to identify missing stakeholders and subsystems, and to add or delete information data flows as necessary.

ODOT conducted internal review of many subsystems with which it had some operational familiarity. WTI conducted two outreach workshops for Caltrans staff – one in Redding and one in Eureka – to assist in a similar review process. Additional feedback was received from one Steering Committee member.

### Extending the Architecture

As a result of the review process, it was determined that significant extensions to the National ITS Architecture would be required in order for it to most completely reflect actual information exchange relationships between various subsystems. In mapping existing and future systems to the National ITS Architecture, it was found that there were several systems for which no adequate architecture counterpart has yet been defined at a national level. This has occurred because the National ITS Architecture has been designed to function around traditionally urban ITS deployments. The National ITS Architecture therefore places less emphasis on roadway maintenance and weather, which are among the primary concerns of COATS stakeholders. Table 3-7 lists the extensions to the National ITS Architecture that were deemed necessary to appropriately define the COATS Regional Architecture.

In addition to these additional subsystems, several extensions to the National ITS Architecture were required in order to define all required information data flows. These are indicated in Table 3-8. In some cases, these information data flows are already included in the National ITS Architecture, but have different subsystems identified for the source and destination of the information flow. In other cases, completely new information data flows were identified, often with some of the subsystems with which the COATS Regional Architecture was extended.

Table 3-7: COATS Entity Extensions to National ITS ArchitectureTable 3-7: COATS Entity Extensions to National ITS Architecture.

28Western Transportation Institute

Table 3-8: COATS Flow Extensions to National ITS Architecture.

| Flow Name | Description | From | To |
| --- | --- | --- | --- |
| construction activity status | Information about impending construction projects coordinated at a statewide level and distributed to districts. | Construction and Maintenance | Maintenance Dispatch Office** |
| incident data* | Data and imagery from the roadside supporting incident detection and verification. | Roadway Subsystem | Emergency Management |
|  |  | Roadway Subsystem | Maintenance Dispatch Office** |
| incident information* | Notification of existence of incident and expected severity, location, time and nature of incident. | Maintenance Dispatch Office** | Information Service Provider |
|  |  | Traffic Management | Information Service Provider |
| incident response vehicle coordination | Communication between vehicles involved in emergency or incident response activities. This communication is essential to reducing the impact of incidents, especially road closures, on travelers. It may require several different communication media, as this will typically span over many jurisdictional lines. | Emergency Vehicle Subsystem | Emergency Vehicle Subsystem |
|  |  | Emergency Vehicle Subsystem | Maintenance Vehicles** |
|  |  | Maintenance Vehicles** | Emergency Vehicle Subsystem |
| localized weather information | Select weather information (e.g. wind, precipitation, humidity, temperature, etc.) that may be used to active a local weather warning system. The information is transmitted between a field-based weather data station and a collocated traveler advisory system. | Weather Service | Weather Warning Systems** |
| maintenance activity coordination | Information communicated from the maintenance dispatch center to maintenance vehicles to coordinate maintenance activities. This could include information about locations of activities, resource deployment status, and any actions which should be taken. | Maintenance Dispatch Office** | Maintenance Vehicles** |
| maintenance activity information | Information about non-planned and planned road conditions that may affect travel conditions, including construction-related lane closures and detours, and weather-related events. | Maintenance Dispatch Office** | Information Service Provider |

* - Existing flow in National ITS Architecture with different source and/or destination

** - Architecture extension

Table 3-8: COATS Flow Extensions to National ITS Architecture (cont.).

Western Transportation Institute29

| Flow Name | Description | From | To |
| --- | --- | --- | --- |
| maintenance activity status | Any communication sent from maintenance vehicles back to the maintenance dispatch office. This would include information about the current status of the action, expected duration of an action, and the need for additional resources. | Maintenance Vehicles** | Maintenance Dispatch Office** |
| maintenance dispatch office coordination | Information communicated between different maintenance dispatch centers regarding locations of critical maintenance actions, like road closures, detours, winter road maintenance, etc. | Maintenance Dispatch Office** | Other Maintenance Dispatch Office** |
|  |  | Other Maintenance Dispatch Office** | Maintenance Dispatch Office** |
| maintenance vehicle coordination | Any type of vehicle to vehicle communication between maintenance vehicles. | Maintenance Vehicles** | Other Maintenance Vehicles** |
|  |  | Other Maintenance Vehicles** | Maintenance Vehicles** |
| maintenance vehicle location | Information transmitted from maintenance vehicles to the maintenance dispatch office, characterizing the vehicle’s location. | Maintenance Vehicles** | Maintenance Dispatch Office** |
| media information request* | Request from the media for current transportation information. | Media | Public Information Office** |
| permit request | Information about the date, time and nature of activities permitted on the right-of-way. | Permitting Office** | Traffic Management |
| permit support request | Information provided to maintenance staff to assist in permits for right-of-way access granted by the permitting agency. This would include date, time and location of requests for infringement of highway right-of-way, along with details about any precautions that maintenance staff should undertake to protect the traveling public, maintenance staff, and those authorized onto the right-of-way, including traffic cones, VMS messages, flaggers, etc. | Permitting Office** | Maintenance Dispatch Office** |
| permit support request confirmation | Maintenance dispatch offices confirm that information regarding a specific permit request has been understood, and that any maintenance tasks required to protect the traveling public and permittees will be undertaken. | Maintenance Dispatch Office** | Permitting Office** |

* - Existing flow in National ITS Architecture with different source and/or destination

** - Architecture extension

Table 3-8: COATS Flow Extensions to National ITS Architecture (cont.).

30Western Transportation Institute

| Flow Name | Description | From | To |
| --- | --- | --- | --- |
| road construction information for transit | Information regarding construction activities which may affect transit service, requiring long-term realignment of routes and/or delay to transit users. | Construction and Maintenance | Transit Management |
|  |  | Traffic Management | Transit Management |
| seismic conditions | Current seismic conditions as measured by remote sensors and communicated by supporting field equipment. | Roadway Subsystem | Traffic Management |
| traffic images* | High fidelity, real-time traffic images suitable for surveillance monitoring by the operator or for use in machine vision applications. | Roadway Subsystem | Emergency Management |
|  |  | Roadway Subsystem | Maintenance Dispatch Office** |
| traffic information for media* | Report of current traffic conditions, incidents, maintenance activities and other traffic-related information prepared for public dissemination through the media. | Public Information Office** | Media |
| weather information coordination | Information exchange between agencies who are collecting weather-related information, including generalized atmospheric and precipitation conditions, as well as local stream gauge readings. | Traffic Management | Construction and Maintenance |
|  |  | Traffic Management | Other TM |
|  |  | Traffic Management | Weather Service |
|  |  | Weather Service | Traffic Management |

* - Existing flow in National ITS Architecture with different source and/or destination

** - Architecture extension

# The COATS Regional Architecture

It was decided that the recently released Turbo Architecture 1.0 software package, prepared under contract for the U.S. Department of Transportation would be used to facilitate the development of a regional architecture. This software reflects version 3.0 of the National ITS Architecture. It allows the user to develop an inventory of systems and stakeholders, identify applicable market packages, and customize data flows. Of particular value to the COATS project, given the architecture extensions identified earlier, is the program’s capacity to extend the national architecture, through adding subsystems and/or information data flows.

The Turbo Architecture database developed for the COATS project has the following characteristics:

- 169 subsystems;

- 17 market packages;

- over 700 architecture interconnects; and

- over 2,100 information data flows.

The COATS Regional Architecture has the following characteristics.

- It does focus on the major system and institutional boundaries in the region. This emphasis is because optimal system performance can only be achieved when the overall system works as well at the jurisdictional boundaries as it does everywhere else.

- It does address conformance with the National ITS Architecture. The regional architecture is defined to the architecture flow level, which should be adequate to demonstrate conformance with the National ITS Architecture. Note that this regional architecture was developed as part of an overall strategic planning effort. As such, the architecture was not developed in detail comparable to that which might result from a dedicated architecture development effort. The core architecture that was developed can be extended and elaborated by on-going larger-scale statewide architecture efforts.

- It does provide a long-term view that can guide ITS planning and project development in the region. Many systems and interfaces that are identified in the regional architecture have not yet been implemented.

- It does encourage better cooperation between agencies, by highlighting the ways that they can more effectively use and share data to meet the needs of travelers in the region.

- It does include subsystems that may not be used by any of the market packages specified for the COATS study area, such as various CVO-related applications. This recognizes that this architecture be eventually merged with other architectures where these market packages are considered to be more critical.

- It does not define the internal design for individual systems in the region, since internal design decisions are better left to the implementing agencies and their contractors.

- It does not alter the existing institutional arrangements or the authority of participating agencies. A fundamental requirement of the regional architecture is to operate within the existing institutional framework in the region.

- It does not mandate specific technology choices in the region. The regional stakeholders may consider technology agreements, but these extend beyond the fundamentally technology-neutral framework defined by the regional architecture.

Several different depictions of the COATS architecture were developed through Turbo Architecture in the course of this project. The architecture includes high-level “sausage diagram” renderings of the architecture to the architecture flow level presentation that defines hundreds of information flows. The different levels of presentation are each suited to different audiences and applications.

The purpose of this chapter is to present the COATS Regional Architecture in a top-down manner, beginning with a high-level “sausage diagram” that is approachable and familiar, particularly for those with previous exposure to the National ITS Architecture. As the chapter progresses, additional detail is added to this general framework to provide a more detailed description of the systems in the region and a framework for integrating these systems together into a cohesive, scalable transportation network that will cost-effectively meet the needs of the region.

## Defining the Regional Architecture for COATS

The COATS Regional Architecture is a master blueprint for an integrated, multimodal intelligent transportation system. The architecture identifies the major systems in the region and defines the interfaces between these systems. It clarifies how organizations can work together to deliver better safety, efficiency, and convenience to the rural traveler through deployment of ITS improvements.

The regional architecture was developed with participation from many sectors, including transportation, tourism, law enforcement, emergency management, transit and the private sector, that operate and use the transportation systems in the region. These regional partners, through the COATS Steering Committee and Regional Teams, have identified the central requirements for the regional architecture. This collaborative effort will continue as the regional architecture is reviewed, refined and adapted to meet changing needs and evolving preferences in the future.

One of the most striking challenges about the COATS region is that, while it is geographically contiguous, it is intersected by dozens of jurisdictional boundaries. These boundaries reflect the geographic areas of responsibility for many stakeholders in the region. Accordingly, the study area was subdivided into three subregions, as shown in Figure 4-1, to try to reflect these areas of responsibility. These subregions include:

- Figure 4-1: COATS Architecture SubregionsFigure 4-1: COATS Architecture Subregions.southern Oregon,

- north central California, and

- northern California coast.

The COATS Regional Architecture spans and integrates these three subregional architectures. More than a simple aggregation, the COATS Regional Architecture also provides an overarching framework for the subregions. This unifying structure works on two levels. First, it identifies common systems and interfaces that are implemented in all subregions (see section 4.2), and second, it provides a plan for interconnecting the subregions where this makes sense (see section 4.3).

## Common Systems and Interfaces

As an initial introduction to the COATS Regional Architecture, the systems in the three subregions can be recast into a “sausage diagram” based on the mapping between regional systems and the National ITS Architecture. The sausage diagram is a mainstay of National ITS Architecture presentations and has been widely adopted as a useful way to present major systems 
and communications interconnects at the highest level.

Figure 4-2 highlights the elements of the National ITS Architecture sausage diagram that are applicable to the COATS region. The figure shows that most of the subsystems and the majority of the communications interconnects defined in the National ITS Architecture have application in the COATS region. All of these subsystems are included in the current detailed regional architecture definition for the COATS region except for the Toll Administration and Toll Collection subsystems. By correlating the systems in all the COATS subregions with the National ITS Architecture sausage diagram, like systems are grouped together.

The standard National ITS Architecture sausage diagram has a couple of key limitations.

- The standard sausage diagram only shows the 19 subsystems. It does not include, by definition, any extensions to the architecture that are appropriate for a given region. For this reason, Figure 4-2 was amended to include additional subsystems identified through extending the architecture.

- The standard sausage diagram omits the National ITS Architecture terminators, some of which are very important to the COATS Regional Architecture. This limitation was preserved in order to preserve the simplicity of the sausage diagram.

- The sausage diagram also shows a deceptively simple, fully networked communications system that connects every subsystem with every other on a single region-wide backbone. The COATS Regional Architecture does not require such complete connectivity between systems; in fact, a partitioned network that limits access to various systems and assets is important from a practical standpoint.

To address these issues, the high-level regional interconnect diagram in Figure 4-3 can provide a more detailed view of how the systems in each region can be integrated. It shows how the general classes of systems that are implemented in every subregion within COATS are interconnected. It offers a more complete view than the sausage diagram of the types of systems in the region and the potential connectivity between these systems. The general connection strategy is a structure of several sub-networks that are interconnected through defined gateways to improve information sharing across the region without sacrificing the performance, reliability, and security of the most safety-critical portions of the network.

Western Transportation Institute35

Figure 4-2: Subsystems and Interconnects Applicable to the COATS RegionFigure 4-2: Subsystems and Interconnects Applicable to the COATS Region.

36Western Transportation Institute

Figure 4-3: COATS Regional Architecture Systems and InterconnectsFigure 4-3: COATS Regional Architecture Systems and Interconnects.

State

Table 4-1: COATS Regional Architecture Interconnect DefinitionsTable 4-1: COATS Regional Architecture Interconnect Definitions.

| Interconnect | Definition |
| --- | --- |
| Commercial Wireline/Wireless Networks | This interconnect represents the communications channels that connect information providers in the COATS region with their customers. A variety of commercial wireline and wireless communications systems can provide these connections today; on-going technical innovation in this dynamic, market-driven area suggest that new communications options will be offered in the future. Beyond accommodating the information services and delivery mechanisms that develop as marketable products for the region and a general endorsement of those services that are supported by open standards, the regional architecture is not prescriptive for this interconnect since it is strongly influenced by larger market forces. The objective is not to constrain the number of systems that are deployed in the region, but to encourage private sector participation by making public domain information available to information service providers and developing clear policies that encourage broad participation in the region. Wireline communications systems that are available in the COATS region include the traditional circuit-switched telephone system (analog today with increasing numbers of higher-speed digital options for the consumer in the future) and cable systems in the larger cities. Perhaps the most relevant wireless media for the region are FM subcarrier and cellular communications in the near term and satellite communications when increasing competition makes this option more affordable. |
| DSRC | Dedicated Short Range Communications (DSRC) is a short-range airlink used for close-proximity (less than 50-100 feet) transmissions between a mobile user and a base station. This type of interconnect has many applications in ITS wherever location-specific communication with a vehicle is required. The applications for DSRC that are envisioned in the COATS region are initially limited to mainline screening of commercial vehicles at weigh stations. As depicted, the same communications technologies could also be applied to make parking and other traveler/user fee collections more convenient and efficient in the future. The regional architecture suggests that the emerging ITS standards for these interconnects be adopted and applied in the region. |
| Wide-Area Wireless –Trunked/Dedicated Radio Systems | This wireless infrastructure includes the 800 MHz radio systems, specialized mobile radio systems, and other wide-area wireless infrastructure that enables vehicle fleet communications for public safety, maintenance, commercial vehicle, and transit fleets in the region. While primarily viewed as voice communications systems today, data applications will increasingly be supported by these systems in the future. The regional architecture suggests use of industry standards for these communications where they are available. Emerging Transit Communications Interface Protocol (TCIP) standards should be reviewed for use in the transit fleet communications deployments. Although ITS standards work has not yet been initiated for emergency vehicle fleets, interagency agreements and implementations may be considered to facilitate incident command systems and resource sharing for large incidents which require the involvement of many agencies in the COATS region. |

Table 4-1: COATS Regional Architecture Interconnect Definitions (cont.).

| Interconnect | Definition |
| --- | --- |
| Wireline/Wide-Area Wireless | Often, wireline communications infrastructure is simply not available at likely deployment locations for roadside ITS infrastructure like variable message signs, highway advisory radio, and environmental sensor stations. The cost-effective alternative for relatively low-bandwidth applications is to use wireless communications to support this field equipment. This interconnect represents the radio systems and cellular infrastructure that will cost-effectively support limited bandwidth communications with remote field equipment. |
| Wireline Communications (A) | Safety-critical incident management communications between public safety, transportation, and other allied agencies is supported by this access-restricted network. Since regional agencies rely on this interconnect to coordinate incident management, this communications system should be reliable, secure, and offer deterministic performance. The regional architecture uses bridge systems like the Highway Travel Conditions Reporting System (HTCRS), the California Highway Information Network (CHIN), and successor highway conditions reporting systems to shield this network from direct traveler information requests and serve as a filter that preserves privacy and security of the communications on the network. Wireless as well as traditional wireline communications links will be used for this interconnect, in order to support communications with command posts, staging areas, and other remote command centers that may be established for larger incidents under unified command systems. The Institute of Electrical and Electronics Engineers (IEEE) Incident Management Message Sets, Institute of Transportation Engineers (ITE) Traffic Management Data Dictionary and External TMC Communications standards are among the key standards that apply to this interconnect that are recommended by the regional architecture. |
| Wireline Communications (B) | This wireline infrastructure makes current transportation information available to information service providers and other stakeholders who desire real-time transportation information, but do not have the direct operations role that would give them access to Wireline Communications (A). While adequate performance and reliability that is scalable to meet future needs is important here, it is less imperative than with Wireline Communications (A). Only data that has been cleansed to support data privacy principles is made available on this network. In the regional architecture, a service like ODOT’s TripCheck is a key source for this traveler information that has been sanitized from the operations version of the same information where necessary. In addition to incident data and other real-time information, this interconnect also makes available more static visitor and traveler information as a resource to information service providers in the region. This interface can be implemented using one or more of the many alternative existing public or private networks that may physically include wireless (e.g. microwave) as well as wireline infrastructure. This interconnect is supported primarily by the Society of Automotive Engineers (SAE) Advanced Traveler Information Systems (ATIS) Data Dictionary and Message Set standards. |

Table 4-1: COATS Regional Architecture Interconnect Definitions (cont.).

| Interconnect | Definition |
| --- | --- |
| Wireline Communications (C) | This wireline infrastructure connects centers with distributed field equipment that is monitored and controlled by the center. Since this interface enables the monitoring and control of field equipment, access to this interface must be restricted to those authorized to control the field equipment. In the COATS region, this interconnect allows the remote monitoring and control of roadside devices including variable message signs, highway advisory radio, closed-circuit television (CCTV) cameras, etc. The same class of communications interconnect integrates commercial vehicle administration centers with weigh stations. This interface can be implemented using any of the alternative existing public or private networks that may physically include wireless (e.g. microwave) as well as wireline infrastructure. Where relatively inexpensive low-bandwidth wireless communications alternatives are an option, the regional architecture explicitly shows a connection from “Wireline Communications (C)” to “Wireline/Wide Area Wireless.” This reflects the likely scenario where wireline communications is used to carry message traffic between the center and the transceiver that often is remotely located to achieve the best line of sight and coverage. The National Transportation Communications for ITS Protocol (NTCIP) standards activities have already published a number of standards that address the interface to traffic management field equipment. The regional architecture suggests that these standards be used for future implementations in the region to avoid proprietary alternatives and ultimately achieve economies of scale by using standards in the region that will be broadly adopted across the United States. |
| Wireline Communications (D) | This wireline infrastructure connects the fleet dispatcher operating at a fixed point with the trunked and/or dedicated radio systems that provide the wireless connection to the vehicle fleet. This may represent a fairly short run to a building-top antenna or more extensive use of public or private wireline networks for access to remotely located transceivers or trunked radio services. The regional architecture levies no specific requirements for these “dispatcher to antenna” links. |

Figure 4-3 includes a number of interconnects that are more specific than the four basic interconnects (Wireline, Wide Area Wireless, Dedicated Short Range Communications [DSRC], and Vehicle to Vehicle) defined by the National ITS Architecture. For example, several different “wireline” interconnects are defined to segregate the different applications for fixed point to fixed point communications in the region. This separation is important because different wireline communications applications have different reliability, security, and performance constraints. Each of the interconnects in the graphic are described in more detail in Table 4-1.

## Subregion Interconnects

In the preceding section, a generalized view of the way the systems connect to one another in a COATS subregion was defined. For typical, day-to-day operations, the scope of concern and the breadth of communications connectivity need not expand beyond this generalized subregional view. In order to support broader regional transportation services and to support localized transportation services at subregional boundaries, this section establishes a general plan for connecting the COATS subregions.

There are two major factors that require an overarching layer of communications connectivity between the COATS subregions.

- There are heavily traveled routes that connect the COATS subregions, such as Interstate 5, US Routes 97, 101, and 199, California Routes 20 and 299. If the COATS architecture is to work as well at these subregional boundaries as it does elsewhere, then the COATS architecture must allow for communications between agencies that manage the transportation system on each side of these subregional boundaries. Perhaps the best of many examples is the Siskiyou Pass section of Interstate 5 that connects the southern Oregon and north central California subregions, the location of the COATS early winner project. To best manage projects that improve transportation efficiency and safety along these connecting routes, the managing agencies in adjacent subregions must be interconnected.

- There are important services – most notably incident management and traveler information – that require a broader, region-wide view of the transportation system. For incident management, for example, specific region-wide scenarios (as shown in Figures 3-5 and 3-6) were used to derive requirements during the architecture workshops. In order to respond to incidents that have impacts encompassing more than one COATS subregion, information sharing between subregions is important. Today, voice communications are used to support inter-regional coordination of agencies during major incidents. While these voice channels will always exist, they will increasingly be supplemented by data communications in the future. For traveler information, travelers desire to receive information seamlessly without respect to state or county boundaries. This is an area that will require significant data communications to be truly effective.

Figure 4-4 shows the general connections between subregions that have been identified through the COATS project. As shown, the Medford Transportation Operations Center (TOC) and the Eureka and Redding Advanced Rural Technology Integration Centers (ARTIC) have been identified as regional hubs for southern Oregon and northern California, respectively. These operations centers connect to satellite systems in their respective states. This state-level integration allows coordination across subregions within each state. The regional hubs also connect to each other to support communications between transportation agencies in southern Oregon and northern California for coordinating traveler information, large-scale incident management and CVO operations coordination. The specific centers that communicate are ODOT’s Medford TOC and Caltrans’ Eureka and Redding ARTICs and the California Highway Patrol (CHP) and Oregon State Police (OSP) dispatch  centers. These subregional interconnects support safety-critical, operational applications and, as such, should be afforded the same levels of service and reliability as specified for Wireline Communications (A) in the previous section. It should be noted that ODOT and OSP are co-located at the Medford TOC; however, in Redding and Eureka, the ARTICs and the CHP dispatch centers are not co-located.

In addition to these connections between operational centers, the media will also broadcast travel advisories throughout the region. For example, if there is an incident on the Oregon side of the Siskiyou Pass, the incident information is entered and disseminated through ODOT’s Web-based TripCheck traveler information system. This information can be picked up by the media, which will broadcast advisories with coverage extending to the southern approaches to the Siskiyou Pass in northern California.

## Subregional Architectures

Figure 4-4: COATS Regional InterconnectsFigure 4-4: COATS Regional Interconnects.As was mentioned in the beginning of this chapter, the COATS Regional Architecture was divided into three subregions to better map the unique relationships between stakeholders in each part of the COATS region.

To accommodate this subregional division, the Turbo Architecture database includes one regional architecture (encompassing all of the COATS regional inventory and interface definitions) and three subregional architectures, one for each identified COATS subregion. This allows the COATS architecture to be viewed from an overall regional perspective as well as a subregional perspective. This section provides a detailed definition of each subregional architecture based on products derived from the Turbo Architecture database.

Before moving on to the detailed subregional architecture definitions, a few clarifications are in order.

- Where it makes sense, the COATS ITS inventory was defined as a relatively short list of general classes of systems, rather than as a detailed enumeration of every individual system in this expansive region. For example, instead of enumerating every law enforcement dispatch system in the COATS inventory, more general “Area Police Dispatch” elements were defined, each of which represents more than one specific dispatch system.

Defining a more general ITS inventory has two benefits. It substantially simplifies the architecture model since detailed interfaces for every specific system (e.g., every municipal police dispatch system) are not required. This facilitates architecture development when detailed input from every system owner in the region is not available, and it increases the likelihood that the model will be maintained in the future since less effort will be required.

Second, it underscores the fact that all specific systems represented by the more general inventory elements should integrate into the overall COATS architecture in a consistent fashion, using common standards where possible.

The drawback to generalization is that the specific dispatch system for a particular municipal police department is not individually identified in the inventory. This does not affect the validity or usefulness of the architecture in any substantial way. Furthermore, in further architecture development, the generalization may be relaxed if an individual system has distinctively different interface requirements.

Even with this generalization, each element definition includes a comprehensive list of the specific systems that are represented by it. This means that every COATS architecture stakeholder should find a general element that represents their system and also find a specific reference to their system in the definition for that element.

- The detailed architecture that is defined in this section should be viewed as “draft” since it has not been subjected to detailed review with the affected stakeholders at this time. During development of this draft architecture, all likely interfaces and architecture flows were retained so that they can be offered as options to stakeholders in future architecture reviews. This approach ensures that the widest array of connectivity options is included in the draft architecture for consideration by stakeholders. Not every option will be retained; many will be deleted during a detailed stakeholder review. This means that the architecture currently includes a superset of the ITS integration opportunities that will ultimately be selected and implemented in the COATS region.

- The COATS architecture inventory includes many elements that are relevant to more than one subregion. In these cases, the inventory element is included in each relevant subregional architecture. This creates some redundancy between the subregional architectures, but this redundancy is necessary to show a complete architecture for each subregion.

- Although they are included as placeholders in the Turbo Architecture database, this report does not include the commercial vehicle-related portions of the COATS architecture. This omission recognizes that the CVO architecture is largely being developed through larger state, multi-state, and national CVO ITS initiatives. The CVO portions of the COATS architecture may be directly accessed using Turbo Architecture software.

- To further focus the architecture presentation, the subregional architectures that are presented isolate those potential systems and interfaces that are relevant to the high priority market packages identified in Chapter 3. This means that the details associated with many ancillary systems (e.g., emissions monitoring in northern California) are not included. These other systems may be added in the future.

An inventory and a detailed set of architecture flow “context diagrams” have been developed for each inventory element. The inventory is included as Appendix C; the context flow diagrams for the region are included as Appendix D.

### Southern Oregon

The southern Oregon regional architecture covers the counties of Coos, Curry, Douglas, Jackson, Josephine, Klamath and Lake. Although the following counties are in the COATS region they were not included in the southern Oregon architecture: Crook, Deschutes, Harney, Jefferson, Lane, Linn and Malheur. The major cities included the subregional architecture include Medford, Grants Pass, Roseburg, Klamath Falls, Ashland, Coos Bay, Central Point, North Bend, Florence, Brookings, Reedsport, Coquille, Winston and Lakeview.

The southern Oregon subregion combines the findings of the three architecture workshops in Oregon. This aggregation was determined to be beneficial to concurrent efforts by ODOT to develop a statewide ITS architecture. It was anticipated that the boundary of the COATS architecture on the Oregon side would extend to the boundary of the rest of ODOT’s statewide architecture. This results in the following consequences.

- ODOT’s Bend and Salem TOCs are treated as terminators in the COATS Regional Architecture – i.e. systems that are outside of the boundary of this architecture. Within the COATS region, the Bend TOC is currently responsible for the southeastern portion of Oregon. It is anticipated that when a TOC is built in eastern Oregon that the Bend TOC’s area of responsibility will shift so that it will be responsible for Klamath Falls, which is currently under the jurisdiction of the Medford TOC. The Salem TOC is responsible for only Lane County within the COATS region. It is anticipated that, in both cases, statewide ITS architecture efforts will be able to provide more detailed information about the relationship of these TOCs to other local systems and stakeholders.

- To preserve the local detail gained during the architecture workshops, systems were made less generalized than they were in the California subregions. For example, instead of having a “Southern Oregon City Police Dispatch System,” separate police dispatch systems were designated for Coos Bay-area cities, Klamath Falls-area cities, and Medford-area cities.

Table 4-2 lists all identified inventory elements for this subregion as defined in the Turbo Architecture database for COATS. This inventory was reviewed and derived from input received during the COATS architecture workshops in Coos Bay, Medford, and Klamath Falls.

Table 4-2: Southern Oregon Inventory.

| Element Name | Status | Description |
| --- | --- | --- |
| 1-800-977-ODOT | Existing | This toll-free number provides current road conditions from all areas within Oregon by touchtone phone (outside of Oregon, this is available at 503-588-2941). The service is provided by ODOT. This may be superceded by 511 at a later date. |
| Airline Schedule Operations | Existing | Includes Ascot, Horizon, Skywest, United and other airlines offering regular scheduled passenger service. |
| Amtrak Schedule Operations | Existing | This is the operational entity that provides static and real-time Amtrak schedule information for the COATS architecture. |
| COATS Kiosks | Planned | This is a planned system which would provide real-time traveler and tourism information to travelers throughout the COATS region. |
| Commercial Vehicle (Onboard) Systems | Existing | This subsystem resides in a commercial vehicle and provides the sensory, processing, storage, and communications functions necessary to support safe and efficient commercial vehicle operations. The Commercial Vehicle Subsystem provides two-way communications between the commercial vehicle drivers, their fleet managers, and roadside officials, and provides HAZMAT response teams with timely and accurate cargo contents information after a vehicle incident. |
| Commercial Vehicle Driver | Existing | This terminator represents the human entity that operates vehicles transporting goods including both long haul trucks and local pick up and delivery vans. |
| Commercial Vehicles | Existing | The actual commercial vehicle along with the special aspects of large commercial vehicles and vehicles designed to carry cargo that extend beyond the characteristics defined for the Basic Vehicle. This terminator thus represents a special type of Basic Vehicle that is used to transport goods or services which are operated by professional drivers, typically administered as part of a larger fleet, and regulated by a Commercial Vehicle Manager. |
| Coos Bay-Area City Traffic Control Equip | Existing | This includes traffic equipment maintained by the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, and Roseburg. |
| Coos Bay-Area City Traffic Control Systems | Existing | This includes traffic management activities for the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, Brookings, Port Orford, Gold Beach, Bandon, Myrtle Point, Elkton, Sutherland, Myrtle Creek, and Wells Creek. |
| Coos Bay-Area County Maintenance Systems | Existing | This includes Coos County, Curry County, Douglas County and Lane County Maintenance Department systems. |
| Coos Bay-Area E-911 Comm Centers | Existing | This includes the two communications centers each in Coos and Curry Counties. |
| Coos Bay-Area EMS Provider Dispatch | Existing | This includes the broad range of private EMS, ambulance and air evacuation providers that serve the Coos Bay area, including Bay Cities Ambulance, Coquille Valley Ambulance, Lower Umpqua EMS, Medic 4 Ambulance, Myrtle Point, Curry County EMS, and Del Norte. |
| Coos Bay-Area EMS Vehicles | Existing | This includes the broad range of private EMS, ambulance and air evacuation vehicles that serve the Coos Bay area, including Bay Cities Ambulance, Coquille Valley Ambulance, Lower Umpqua EMS, Medic 4 Ambulance, Myrtle Point, Curry County EMS, and Del Norte. |

Table 4-2: Southern Oregon Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Coos Bay-Area Fire and Rescue Dispatch | Existing | This includes fire and rescue dispatch centers for Coos Bay, North Bend, Florence, Reedsport, Coquille, Hauser, East Bay, Millington, and Charleston. (The last four cities are volunteer.) |
| Coos Bay-Area Fire and Rescue Vehicles | Existing | This includes fire and rescue vehicles serving Coos Bay, North Bend, Florence, Reedsport, Coquille, Hauser, East Bay, Millington, and Charleston. (The last four cities are volunteer.) |
| Coos Bay-Area Paratransit Dispatch | Existing | Includes Curry-Coos County Transit System, Senior Transportation. |
| Coos Bay-Area Paratransit Vehicles | Existing | Includes Curry-Coos County Transit System, Senior Transportation. |
| Coos Bay-Area Planning Agencies | Existing | This includes various city and county planning agencies that may have use for archived data, including but not limited to the Umpqua Regional Council of Governments. |
| Coos Bay-Area Police and Sheriff Dispatch | Existing | This includes the police departments for the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, Brookings, Port Orford, Gold Beach, Bandon, Myrtle Point, Elkton, Sutherland, Myrtle Creek, and Wells Creek. This also includes county sheriffs from Coos, Curry and Douglas Counties, as well as tribal police. |
| Coos Bay-Area Police and Sheriff Vehicles | Existing | This includes law enforcement vehicles for the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, Brookings, Port Orford, Gold Beach, Bandon, Myrtle Point, Elkton, Sutherland, Myrtle Creek, and Wells Creek. This also includes county sheriffs from Coos, Curry and Douglas Counties, as well as tribal police. |
| Coos Bay-Area Radio Stations and Newspapers | Existing | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the radio and print media intended for human, rather than system, processing and interpretation. |
| Coos Bay-Area Towing/Recovery | Existing | This includes the private tow dispatch systems that clear incidents in the Coos Bay area. This includes numerous individual operators that serve in the region. |
| Coos Bay-Area Traveler Information Resources | Existing | This includes Parks and Recreation Departments for the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, and Roseburg; the Bureau of Land Management; U.S. Forest Service; Oregon Parks & Recreation; Coos County Bikeway Committee; Coos County and Curry County Parks; Bandon Dunes Recreational Area; as well as area hotels, restaurants, gas stations, shopping, etc. |
| Coos Bay-Area Visitor Information Services | Existing | This includes City Chambers of Commerce, Central Oregon Coast Association, Southern Oregon Visitors Association, Oregon Tourism Commission, Oregon Lodging Association, and Coos Bay-North Bend Tourism Promotion Bureau. |
| CVO Inspector | Existing | This terminator represents the human entities who perform regulatory inspection of Commercial Vehicles in the field. CVO Inspectors support the roadside inspection, weighing, and checking of credentials either through automated preclearance or manual methods. |

Table 4-2: Southern Oregon Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| DEQ Medford Regional Office | Existing | DEQ is responsible for protecting and enhancing Oregon’s water and air quality, for cleaning up spills and releases of hazardous materials, and for managing the proper disposal of solid and hazardous wastes. DEQ operates a vehicle inspection program in the Rogue Valley to make sure cars are maintained properly to reduce emissions. |
| Downtown Parking District | Existing | This district covers the downtown Klamath Falls area. |
| Drivers | Existing | This terminator represents the human entity that operates a licensed vehicle on the roadway. Included are operators of private, Transit, Commercial, and Emergency vehicles where the data being sent or received is not particular to the type of vehicle. Thus this external originates driver requests and receives driver information that reflects the interactions which might be useful to all drivers, regardless of vehicle classification. |
| Green Light Admin System | Existing | The Green Light program equips trucks with small pager-like devices called transponders. Transponders allow trucks to identify themselves while they’re weighed at high speed by sensors and scales in the highway. A truck equipped with a Green Light transponder can go past an Oregon weigh station at normal cruising speed, instead of pulling off the highway and lining up to weigh at static scales. |
| Highway Travel Conditions Reporting System | Existing | This is an ODOT-run interface in which information about road closures, construction activities and traffic incidents can be entered. The database is accessible in a variety of formats, including a Web-based format (TripCheck) and an 800-number. |
| IFTA Clearinghouse | Existing | This is a national clearinghouse designed to allocate fuel taxes between multiple states for motor carrier activities across jurisdictional lines, in accordance with the International Fuel Tax Agreement. |
| Inter-City Transit | Existing | This includes dispatch systems for Amtrak ThruWay Bus, Coastline Enterprises, Greyhound, North Coast Railroad and Porter Stage Lines. |
| Inter-City Transit Vehicles | Existing | This includes transit vehicles for Amtrak ThruWay Bus, Coastline Enterprises, Greyhound, and Porter Stage Lines. |
| Klamath Falls-Area City Traffic Control Equip | Existing | This includes traffic control equipment owned and operated by the cities of Klamath Falls and Lakeview. |
| Klamath Falls-Area City Traffic Control Systems | Existing | This includes the traffic control systems operated by the cities of Klamath Falls and Lakeview. |
| Klamath Falls-Area County Maintenance | Existing | This includes the maintenance systems (road maintenance, snow removal, etc.) operated in Klamath and Lake Counties. |
| Klamath Falls-Area E-911 Comm Centers | Existing | This represents the public safety answering point for Klamath County that receives 911 calls. |
| Klamath Falls-Area EMS Provider Dispatch | Existing | This includes the broad range of private EMS, ambulance and air evacuation providers that serve the Klamath Falls area, including Klamath County Emergency Services. |

Table 4-2: Southern Oregon Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Klamath Falls-Area EMS Vehicles | Existing | This includes the broad range of private EMS, ambulance and air evacuation vehicles that serve the Klamath Falls area, including Klamath County Emergency Services. |
| Klamath Falls-Area Fire Dispatch | Existing | This includes Klamath County, Lake County and Keno Rural Fire Districts. |
| Klamath Falls-Area Fire Vehicles | Existing | This includes Klamath County, Lake County and Keno Rural Fire Districts. |
| Klamath Falls-Area Fixed Route Dispatch | Existing | This includes all fixed-route transit systems serving the greater Klamath Falls area, including Basin Transit Service Transportation District. |
| Klamath Falls-Area Fixed Route Vehicles | Existing | This includes transit vehicles operating on fixed-route transit systems in the greater Klamath Falls area, including Basin Transit Service Transportation District. |
| Klamath Falls-Area Paratransit Dispatch | Existing | This includes Red Ball Stage Lines Transit Dispatch Operations. |
| Klamath Falls-Area Paratransit Vehicles | Existing | This includes Red Ball Stage Lines Transit. |
| Klamath Falls-Area Planning Agencies | Existing | This includes city and county planning agencies that may use data archived from ITS. |
| Klamath Falls-Area Police and Sheriff Dispatch | Existing | This includes the cities of Klamath Falls and Lakeview. This also includes county sheriffs’ dispatch operations in Klamath County and Lake County. |
| Klamath Falls-Area Police and Sheriff Vehicles | Existing | This includes law enforcement vehicles for the cities of Klamath Falls and Lakeview, and for county sheriffs in Klamath and Lake Counties. |
| Klamath Falls-Area Towing/Recovery | Existing | This includes the private tow dispatch systems that clear incidents in the Klamath Falls area. This includes numerous individual operators that serve in the region. |
| Klamath Falls-Area Traveler Information Resources | Existing | This includes Collier State Park, Crater Lake National Park, Klamath Falls Parks & Recreation, Kla-Mo-Ya Casino, Lava Beds National Monument, Running Y Ranch, and Winema National Forest. It also includes area hotels, restaurants, gas stations, shopping, etc. |
| Klamath Falls-Area TV, Radio Stations and Newspapers | Existing | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the analog TV, radio and print media intended for human, rather than system, processing and interpretation. |
| Klamath Falls-Area Visitor Information Services | Existing | This includes Klamath County Department of Tourism, Klamath Falls Chamber of Commerce, and Lakeview Chamber of Commerce. |
| Mayday Service Provider | Existing | Service centers that provide Mayday and other special concierge services for their clients. These centers will typically be outside the COATS region, but will interact with local agencies when their client’s have an emergency in the region. Current examples of this system include the service centers operated by GM (OnStar), ATX Technologies (Ford and others), and AAA. |

Table 4-2: Southern Oregon Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| MCMIS | Existing | MCMIS contains information on the safety fitness of commercial motor carriers and hazardous material shippers subject to the Federal Motor Carrier Safety Regulations and the Hazardous Materials Regulations. This information is available to the general public through the MCMIS Data Dissemination Program. |
| Medford-Area City Traffic Control Equip | Existing | This includes field equipment for the cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston. |
| Medford-Area City Traffic Control Systems | Existing | This includes traffic control systems for the cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston that are owned and operated by the respective Public Works Departments for each city. |
| Medford-Area County Maintenance Systems | Existing | This includes the highway maintenance systems for Douglas County, Jackson County, and Josephine County. |
| Medford-Area E-911 Comm Centers | Existing | This includes public safety answering points for the greater Medford area, including Douglas Co. 911 Comm Center, Jackson Co. 911 Comm Center, Josephine Co. 911 Comm Center, Myrtle Creek Comm Center, Medford Comm Center, Ashland Comm Center, and Southern Oregon Regional Communications Center. |
| Medford-Area EMS Provider Dispatch | Existing | This includes Douglas County Emergency Services, Jackson County Emergency Management, Josephine County Emergency Services, Mercy Flights and Pacific Flights. |
| Medford-Area EMS Vehicles | Existing | This includes vehicles operated by Douglas County Emergency Services, Jackson County Emergency Management, Josephine County Emergency Services, Mercy Flights and Pacific Flights. |
| Medford-Area Fire/Rescue Dispatch | Existing | This includes the Fire/Rescue Dispatch Operations in the Medford region including county fire districts (Douglas, Jackson, and Josephine County) and municipal fire departments (Cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, Winston, and Wolf Creek). |
| Medford-Area Fire/Rescue Vehicles | Existing | This includes the Fire/Rescue vehicles in the Medford region including those owned by county fire districts (Douglas, Jackson, and Josephine County) and municipal fire departments (Cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, Winston, and Wolf Creek). |
| Medford-Area Fixed Route Dispatch | Existing | This includes all fixed-route transit systems serving the greater Medford area, including Rogue Valley Transportation District and Roseburg Transit. |
| Medford-Area Fixed Route Vehicles | Existing | This includes transit vehicles operating on fixed-route transit systems in the greater Medford area, including Rogue Valley Transportation District and Roseburg Transit. |
| Medford-Area Paratransit Dispatch | Existing | This includes private shuttle services (e.g., Airport Transit) and public services (Southern Oregon Transportation) that provide door-to-door transit services in the Medford area. |
| Medford-Area Paratransit Vehicles | Existing | This includes private shuttle services (e.g., Airport Transit) and public services (Southern Oregon Transportation) that provide door-to-door transit services in the Medford area. |

Table 4-2: Southern Oregon Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Medford-Area Planning Agencies | Existing | This includes various city and county planning agencies that may have use for archived data, including but not limited to the Rogue Valley Council of Governments. |
| Medford-Area Police and Sheriff Dispatch | Existing | This includes the local police department dispatch systems operating in the cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston. This also includes county sheriffs for Jackson and Josephine Counties. |
| Medford-Area Police and Sheriff Vehicles | Existing | This includes law enforcement vehicles serving the cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston. This also includes county sheriff vehicles for Jackson and Josephine Counties. |
| Medford-Area Towing/Recovery | Existing | This includes the private tow dispatch systems that clear incidents in the Medford area. This includes numerous individual operators that serve in the region. |
| Medford-Area Traveler Information Resources | Existing | This includes the State Parks and Recreation Department, parks and recreation departments for Douglas County, Jackson County, and Josephine County, the City Parks and Recreation areas for the cities in the region (Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston), Oregon Caves National Monument, Shakespeare Festival, Britt Festival, and Seven Feathers Casino. It also includes area hotels, restaurants, gas stations, shopping, etc. |
| Medford-Area TV and Radio Stations | Existing | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the analog TV, radio and print media intended for human, rather than system, processing and interpretation. |
| Medford-Area Visitor Centers | Existing | This includes visitor centers in Medford and Ashland. |
| Medford-Area Visitor Information Services | Existing | This includes the systems operated by Oregon Tourism Commission, Chambers of Commerce and other organizations that provide tourist information for the local region. |
| Motor Carrier Dispatch and Back Office Systems | Existing | These provide the capability for commercial drivers and dispatchers to receive real-time routing information and access databases containing vehicle and cargo locations as well as carrier, vehicle, cargo, and driver information. |
| ODOT Bend TOC | Existing | This center is at the periphery of the COATS region. It provides traffic control capabilities for the ODOT-operated facilities in eastern Oregon. |
| ODOT Communications Division | Existing | The Communications Division produces and/or oversees ODOT’s internal and external communications, including the agency’s Internet site. It has offices located in each of ODOT’s five regions to provide information to local news media outlets. |
| ODOT Field Equip (Medford TOC) | Existing | This includes the environmental sensors, variable message signs, HAR, signals, traffic detectors, and any other field equipment and instrumentation operated by ODOT Region 3. |
| ODOT Maintenance Districts (Medford TOC) | Existing | Covers the resource management and dispatch systems operated by ODOT maintenance districts in Coos Bay, Roseburg, Medford and Klamath Falls. |

Table 4-2: Southern Oregon Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| ODOT Maintenance Vehicles (Medford TOC) | Existing | These ODOT vehicles are responsible for maintenance activities, including setting up road closures, assisting in incident removal, and performing winter maintenance. It is assumed that there is seamless communication between the vehicle and its driver, who will actually be directing and performing the maintenance activity. |
| ODOT Medford TOC | Existing | The ODOT Transportation Operations Center in Medford plays a central role in the COATS architecture. It connects to centers in Bend and Salem and Caltrans centers in Eureka and Redding. |
| ODOT RWIS Server | Existing | This is a central, statewide server which collects data from each of the state’s regional RWIS servers. |
| ODOT Salem TOC | Existing | This center is at the periphery of the COATS region. It provides traffic control capabilities for the ODOT-operated facilities in the Lane, Linn and Marion Counties portion of the COATS region. |
| ODOT Statewide Planning | Existing | This includes all divisions within ODOT that will use archived data. |
| ODOT Weather Warning System | Existing | These are driver advisory systems which are activated based on certain meteorological conditions or thresholds (e.g. high wind, icy bridge, etc.). They process information provided by a localized weather station and provide a warning to motorists when specified conditions arise. |
| Oregon Weigh Stations | Existing | These are locations where commercial vehicles may be weighed and inspected. |
| OSP Dispatch | Existing | The Oregon State Police operates several dispatch/communications centers in southern Oregon, all of which operate on a common computer-aided dispatch system for tracking call and response activities. These are often collocated with ODOT facilities for improved incident management, although ODOT and OSP staff will have differing roles in incident management. |
| OSP Vehicles | Existing | Oregon State Police vehicles. |
| Other Weather Data | Existing | This terminator refers to weather information received from the National Weather Service or state agencies which also have weather-related information. This could include fully equipped weather stations, isolated stream gauges, or other similar technologies. |
| SAFER | Existing | SAFER provides carrier, vehicle, and driver safety and credential information to fixed and mobile roadside inspection stations. This information will allow the roadside inspector to select vehicles and/or drivers for inspection based on the number of prior carrier inspections, as well as carrier, vehicle, and driver safety and credential historical information. |
| SOVA Kiosks | Existing | These kiosks will be upgraded to be Web-based with potential access to TripCheck and other traveler information services in the Southern Oregon region. |
| Transit Users | Existing | This terminator represents the human entities using Public Transit vehicles. They may be in the act of embarking or debarking the vehicles and are thus sensed for the purpose of determining passenger loading and fares, or on the vehicles and able to request and receive information. |

Table 4-2: Southern Oregon Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Traveler Information Devices | Planned | These are personal devices which may be used to access real-time transportation information. |
| Travelers | Existing | This terminator represents any individual (human) who uses transportation services. At the time that data is passed to or from the terminator the individual is neither a driver, pedestrian, or transit user. This means that the data provided is that for pre-trip planning or multi-modal personal guidance and includes their requests for assistance in an emergency. Subsequent to receipt of pre-trip information, a Traveler may become a vehicle driver, passenger, transit user, or pedestrian. |
| TripCheck | Existing | TripCheck is the latest version of ODOT’s travel information Internet Web site. Its goal is to provide the best travel information possible in a way that is easy to access. TripCheck uses maps extensively, providing users with a convenient interface and a fast way to navigate the site. It includes roadway incident maps, camera images, RWIS information, and other features. |
| Vehicle Information Systems | Existing | These are devices installed in passenger vehicles that support access to digital traveler information. A wide range of evolving technologies are available. |
| Vehicle Mayday Systems | Existing | These are devices installed in passenger vehicles that provide Mayday services to the vehicle occupants. In many cases, these systems are integrated with Vehicle Information Systems (see related Inventory entry). |
| Vehicles | Existing | This terminator represents the basic vehicle platform that interfaces with and hosts ITS electronics. The Basic Vehicle terminator provides an interface to drive train, driver convenience and entertainment systems, and other non-ITS electronics on-board the vehicle. This interface allows general vehicle systems (e.g., the stereo speaker system) to be shared by ITS and non-ITS systems. It also allows monitoring and control of the vehicle platform for advanced vehicle control system applications. |
| Virtual Transit Mall | Planned | This is a Web-based service that will coordinate Oregon’s 200-plus transit providers in order to offer: greater access to transit information, statewide trip planning, improved connections between providers, and enhanced transit planning data. |

### North Central California

The north central California regional architecture covers Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama, and Trinity Counties. This includes the cities of Redding, Susanville, Red Bluff, Anderson, Yreka, Willows, Corning, Orland, Mount Shasta, Alturas and Weed. Table 4-3 lists all identified inventory elements for this subregion as defined in the Turbo Architecture database. This inventory was reviewed and derived from input received during the COATS architecture workshop in Redding.

Table 4-2: Southern Oregon InventoryTable 4-3: North Central California InventoryTable 4-3: North Central California Inventory.

| Element Name | Status | Description |
| --- | --- | --- |
| 1-800-427-ROAD | Existing | This is a toll-free number run by Caltrans which provides current road conditions information. This may be superceded by 511 at a later date. |
| Airline Schedule Operations | Existing | Includes Ascot, Horizon, Skywest, United and other airlines offering regular scheduled passenger service. |
| Amtrak Schedule Operations | Existing | This is the operational entity that provides static and real-time Amtrak schedule information for the COATS architecture. |
| California Highway Information Web Page | Existing | This interactive Web-based service (http://www.dot.ca.gov/hq/roadinfo) provides key road construction, detour and closure information based on user-specified route numbers. |
| California Weigh Stations | Existing | These are locations where commercial vehicles may be weighed and inspected. |
| Caltrans District 2 Public Affairs | Existing | This office is responsible for communicating with local media all news regarding key Caltrans activities in District 2. It may be used to disseminate information about road closures, ongoing construction projects, and other activities that may affect highway travel. |
| Caltrans Field Equip District 2 (Redding) | Existing | This includes the environmental sensors, variable message signs, HAR, signals, traffic detectors, and any other field equipment and instrumentation operated by Caltrans District 2. |
| Caltrans HQ Construction Program | Existing | This program is responsible for implementing the state’s highway construction program. It will provide information to local districts about future construction projects as they are introduced. |
| Caltrans HQ Permits | Existing | The Encroachment Permits department is a part of the office of development services within Caltrans’ Traffic Operations program. As the responsible department for protecting the public’s investment in the State highway system, the permits office reviews all requests from utility companies, developers, volunteers, nonprofit organizations, etc., desiring to conduct various activities within the right of way. Such activities could include for example: construction of highway improvements, driveway installation and maintenance, highway landscaping and graffiti removal, commercial filming, special events such as parades commemorating an event, e.g. Independence Day. |
| Caltrans HQ Traffic | Existing | This office is responsible for coordinating the state’s transportation management centers, facilitating data exchange and interoperability. It will also serve as a repository for field data collected at districts across the state. |
| Caltrans Maintenance District 2 (Redding) | Existing | Covers the resource management and dispatch systems operated by Caltrans District 2. |
| Caltrans Maintenance District 2 (Redding) Vehicles | Existing | These Caltrans vehicles are responsible for maintenance activities, including setting up road closures, assisting in incident removal, and performing winter maintenance. It is assumed that there is seamless communication between the vehicle and its driver, who will actually be directing and performing the maintenance activity. |

Table 4-3: North Central California Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Caltrans RWIS | Existing | These are RWIS stations maintained by Caltrans for providing information on atmospheric and pavement temperature conditions at remote sites. Information from these sites is helpful for maintenance officials in making maintenance decisions. |
| Caltrans Weather Warning Systems | Existing | These are driver advisory systems which are activated based on certain meteorological conditions or thresholds (e.g. high wind, icy bridge, etc.). They process information provided by a localized weather station and provide a warning to motorists when specified conditions arise. |
| Caltrans/CHP Redding ARTIC | Existing | This ARTIC (Advanced Rural Technology Integration Center) coordinates transportation operations for north central California, serving Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama, and Trinity Counties. It is a Tier 3 Satellite Operation Center which is connected to the Sacramento TMC. |
| Caltrans/CHP Sacramento Regional TMC | Existing | This Tier 1 TMC is tied to Tier 2 (Urban) TMCs in Stockton and Fresno and Tier 3 Satellite Operation Centers in Redding and Kingvale |
| CHIN | Existing | The California Highway Information Network provides daily adverse travel conditions information. Information is made available to telephone and Internet. |
| CHP Northern District Dispatch and 911 Centers | Existing | This placeholder includes the Redding, Susanville and Yreka dispatch centers. These centers are responsible for CHP offices in Alturas, Cottonwood, Mt. Shasta, Quincy, Redding, Red Bluff, Susanville, Trinity River, and Yreka, as well as Dunsmuir Grade Inspection Facility. |
| CHP Vehicles | Existing | California Highway Patrol vehicles. |
| COATS Kiosks | Planned | This is a planned system which would provide real-time traveler and tourism information to travelers throughout the COATS region. |
| Commercial Vehicle (Onboard) Systems | Existing | This subsystem resides in a commercial vehicle and provides the sensory, processing, storage, and communications functions necessary to support safe and efficient commercial vehicle operations. The Commercial Vehicle Subsystem provides two-way communications between the commercial vehicle drivers, their fleet managers, and roadside officials, and provides HAZMAT response teams with timely and accurate cargo contents information after a vehicle incident. |
| Commercial Vehicle Driver | Existing | This terminator represents the human entity that operates vehicles transporting goods including both long haul trucks and local pick up and delivery vans. |
| Commercial Vehicles | Existing | The actual commercial vehicle along with the special aspects of large commercial vehicles and vehicles designed to carry cargo that extend beyond the characteristics defined for the Basic Vehicle. This terminator thus represents a special type of Basic Vehicle that is used to transport goods or services which are operated by professional drivers, typically administered as part of a larger fleet, and regulated by a Commercial Vehicle Manager. |

Table 4-3: North Central California Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| CVO Inspector | Existing | This terminator represents the human entities who perform regulatory inspection of Commercial Vehicles in the field. CVO Inspectors support the roadside inspection, weighing, and checking of credentials either through automated preclearance or manual methods. |
| Drivers | Existing | This terminator represents the human entity that operates a licensed vehicle on the roadway. Included are operators of private, Transit, Commercial, and Emergency vehicles where the data being sent or received is not particular to the type of vehicle. Thus this external originates driver requests and receives driver information that reflects the interactions which might be useful to all drivers, regardless of vehicle classification. |
| HELP | Existing | HELP (Heavy Vehicle Electronic License Plate, Inc.), is a non-profit partnership between motor carriers and government agencies (including Caltrans) whose mission is to develop and deploy advanced technology systems to create a cooperative operating and regulatory environment which improves the efficient and safe movement of commercial vehicles and the performance of highway systems. |
| Inter-City Transit | Existing | This includes dispatch systems for Amtrak ThruWay Bus, Coastline Enterprises, Greyhound, North Coast Railroad and Porter Stage Lines. |
| Inter-City Transit Vehicles | Existing | This includes transit vehicles for Amtrak ThruWay Bus, Coastline Enterprises, Greyhound, and Porter Stage Lines. |
| IRP Clearinghouse | Existing | This is a registration reciprocity agreement among jurisdictions in the United States and Canada which provides for payment of license fees on the basis of fleet miles operated in various jurisdictions |
| Mayday Service Provider | Existing | Service centers that provide Mayday and other special concierge services for their clients. These centers will typically be outside the COATS region, but will interact with local agencies when their client’s have an emergency in the region. Current examples of this system include the service centers operated by GM (OnStar), ATX Technologies (Ford and others), and AAA. |
| MCMIS | Existing | MCMIS contains information on the safety fitness of commercial motor carriers and hazardous material shippers subject to the Federal Motor Carrier Safety Regulations and the Hazardous Materials Regulations. This information is available to the general public through the MCMIS Data Dissemination Program. |
| Motor Carrier Dispatch and Back Office Systems | Existing | These provide the capability for commercial drivers and dispatchers to receive real-time routing information and access databases containing vehicle and cargo locations as well as carrier, vehicle, cargo, and driver information. |
| Other Weather Data | Existing | This terminator refers to weather information received from the National Weather Service or state agencies which also have weather-related information. This could include fully equipped weather stations, isolated stream gauges, or other similar technologies. |

Table 4-3: North Central California Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Redding-Area Air Districts | Existing | This placeholder includes Colusa County APCD, Glenn County APCD, Lassen County APCD, Modoc County APCD, Northern Sierra APCD, Shasta County AQMD, Siskiyou County APCD, and Tehama County APCD |
| Redding-Area City Traffic Control Equip | Existing | This includes traffic equipment for the cities of Alturas, Anderson , Corning, Mount Shasta, Orland, Red Bluff, Redding, Shasta Lake, Susanville, Weed, Willows, and Yreka. |
| Redding-Area City Traffic Control Systems | Existing | This includes traffic management functions for the cities of Alturas, Anderson, Corning, Mount Shasta, Orland, Red Bluff, Redding, Shasta Lake, Susanville, Weed, Willows, and Yreka. |
| Redding-Area County Maintenance Systems | Existing | This includes public works departments for the following counties: Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama, and Trinity. |
| Redding-Area E-911 Comm Centers | Existing | This includes public safety answering points receiving wireline 911 calls in the Redding area. |
| Redding-Area EMS Provider Dispatch | Existing | This includes the broad range of private EMS, ambulance, and air evacuation providers that serve the Redding area, including American Med. Response (includes North Valley Ambulance), City of Dorris, Emergency Medical Center, Mount Shasta Ambulance Service, NorCal EMS, Northern Siskiyou Ambulance Service, Shascom, and Trinity County Life Support. |
| Redding-Area EMS Vehicles | Existing | This includes the broad range of private EMS, ambulance, and air evacuation vehicles that serve the Redding area, including American Med. Response (includes North Valley Ambulance), City of Dorris, Emergency Medical Center, Mount Shasta Ambulance Service, NorCal EMS, Northern Siskiyou Ambulance Service, Shascom and Trinity County Life Support. |
| Redding-Area Fire Dispatch | Existing | This includes fire departments for the communities of Alturas, Anderson, Corning, Cottonwood, Happy Valley, Jones Valley, Mountain Gate, Mount Shasta, Orland, Red Bluff, Redding, Shasta County, Shasta, Shasta Lake City, Susanville, Weed, Willows, and Yreka. This also includes other agencies that have fire/rescue jurisdiction in certain areas, including California Department of Forestry; County Health Departments (Hazmat), National Park Service, Office of Emergency Services (Hazmat), and US Forest Service. |
| Redding-Area Fire Vehicles | Existing | This includes vehicles for community fire departments in Alturas, Anderson, Corning, Cottonwood, Happy Valley, Jones Valley, Mountain Gate, Mount Shasta, Orland, Red Bluff, Redding, Shasta County, Shasta, Shasta Lake City, Susanville, Weed, Willows, and Yreka. This also includes vehicles from other agencies that have fire/rescue jurisdiction in certain areas, including California Department of Forestry; County Health Departments (Hazmat), National Park Service, Office of Emergency Services (Hazmat), and US Forest Service. |

Table 4-3: North Central California Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Redding-Area Fixed Route Dispatch | Existing | This includes the following transit systems: Lassen Rural Bus, serving Lassen County; Modoc Sage Stage; Redding Area Bus Authority (RABA) serving Redding and the surrounding communities; Siskiyou County Transit (the Stage) serving the Yreka area; Tehama Area Rural Express (TRAX) serving the Red Bluff area; and Trinity County Transit. |
| Redding-Area Fixed Route Vehicles | Existing | This includes vehicles operating on the following transit systems: Lassen Rural Bus, serving Lassen County; Modoc Sage Stage; Redding Area Bus Authority (RABA) serving Redding and the surrounding communities; Siskiyou County Transit (the Stage) serving the Yreka area; Tehama Area Rural Express (TRAX) serving the Red Bluff area; and Trinity County Transit. |
| Redding-Area Municipal Planning Agencies | Existing | These data collection and analysis system(s) provide planning data for cities and towns in Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama and Trinity Counties. |
| Redding-Area Paratransit Dispatch | Existing | This includes California Association for Coordinated Transit, DART, Lassen Rural Bus, Modoc Senior Citizens/Meals on Wheels |
| Redding-Area Paratransit Vehicles | Existing | This includes California Association for Coordinated Transit, DART, Lassen Rural Bus, Modoc Senior Citizens/Meals on Wheels |
| Redding-Area Police and Sheriff Dispatch | Existing | This includes the cities of Alturas, Anderson, Corning, Lake Shastina, Mount Shasta, Orland, Red Bluff, Redding, Susanville, Weed, Willows, and Yreka. This also includes county sheriffs for Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama, and Trinity Counties. |
| Redding-Area Police and Sheriff Vehicles | Existing | This includes the vehicles for the cities of Alturas, Anderson, Corning, Lake Shastina, Mount Shasta, Orland, Red Bluff, Redding, Susanville, Weed, Willows, and Yreka; and for the county sheriffs for Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama, and Trinity Counties. |
| Redding-Area Regional Transportation Planning Agencies | Existing | This includes county agencies responsible for transportation planning in the Redding vicinity. This includes the Lassen County Transportation Commission (TC), Modoc County Local Transportation Commission (LTC), Plumas County TC, Shasta County RTPA (also a MPO), Siskiyou County TC, Tehama County TC, and Trinity County TC. |
| Redding-Area Towing/Recovery | Existing | This includes the private tow dispatch systems that clear incidents in the Redding area. This includes numerous individual operators that serve in the region. |
| Redding-Area Traveler Information Resources | Existing | This includes Ashland (OR) Shakespeare Festival, BLM-Alturas Resource Area, Castle Crags State Park, Klamath National Forest, Lassen National Forest, Lassen Volcanic National Park, Lavabeds National Monument, Modoc National Forest, Mt. Shasta Ranger Station, Mt. Shasta Ski Park, Redding Recreation Department, Shasta-Trinity National Forest, Turtle Bay Park / Museum, U.S. Forest Service, Whiskeytown National Recreational Area. It also includes area hotels, restaurants, gas stations, shopping, etc. |

Table 4-3: North Central California Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Redding-Area Traveler Information Systems | Planned | These systems collect traveler information from operational sources (e.g., local traffic and public safety agencies/systems), visitor information systems (e.g. chambers of commerce), and local travel destinations (e.g., Parks). This information is then made available to travelers through both broadcast and interactive digital information services through the Internet and other media. |
| Redding-Area TV and Radio Stations | Existing | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the analog TV, radio and print media intended for human, rather than system, processing and interpretation. |
| Redding-Area Visitor Information Services | Existing | This includes chambers of commerce in Alturas, Dunsmuir, Redding, Trinity County, Weaverville and Yreka. It also includes Northern California Tourism , Redding Convention and Visitors Bureau, Shasta Cascade Wonderland Association, and Siskiyou County Tourism. |
| SAFER | Existing | SAFER provides carrier, vehicle, and driver safety and credential information to fixed and mobile roadside inspection stations. This information will allow the roadside inspector to select vehicles and/or drivers for inspection based on the number of prior carrier inspections, as well as carrier, vehicle, and driver safety and credential historical information. |
| Transit Users | Existing | This terminator represents the human entities using Public Transit vehicles. They may be in the act of embarking or debarking the vehicles and are thus sensed for the purpose of determining passenger loading and fares, or on the vehicles and able to request and receive information. |
| Traveler Information Devices | Planned | These are personal devices which may be used to access real-time transportation information. |
| Travelers | Existing | This terminator represents any individual (human) who uses transportation services. At the time that data is passed to or from the terminator the individual is neither a driver, pedestrian, or transit user. This means that the data provided is that for pre-trip planning or multi-modal personal guidance and includes their requests for assistance in an emergency. Subsequent to receipt of pre-trip information, a Traveler may become a vehicle driver, passenger, transit user, or pedestrian. |
| Vehicle Information Systems | Existing | These are devices installed in passenger vehicles that support access to digital traveler information. A wide range of evolving technologies are available. |
| Vehicle Mayday Systems | Existing | These are devices installed in passenger vehicles that provide Mayday services to the vehicle occupants. In many cases, these systems are integrated with Vehicle Information Systems (see related Inventory entry). |

Table 4-3: North Central California Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Vehicles | Existing | This terminator represents the basic vehicle platform that interfaces with and hosts ITS electronics. The Basic Vehicle terminator provides an interface to drive train, driver convenience and entertainment systems, and other non-ITS electronics on-board the vehicle. This interface allows general vehicle systems (e.g., the stereo speaker system) to be shared by ITS and non-ITS systems. It also allows monitoring and control of the vehicle platform for advanced vehicle control system applications. |

### Northern California Coast

The northern California coast subregional architecture covers Humboldt and Del Norte Counties and includes the cities of Eureka, Arcata, Fortuna, Crescent City, Rio Dell, Ferndale, Blue Lake and Trinidad. Table 4-4 lists all identified inventory elements for this subregion as defined in the Turbo Architecture database. This inventory was reviewed and derived from input received during the Eureka architecture workshop.

Table 4-4: Northern California Coast InventoryTable 4-4: Northern California Coast Inventory.

| Element Name | Status | Description |
| --- | --- | --- |
| 1-800-427-ROAD | Existing | This is a toll-free number run by Caltrans which provides current road conditions information. This may be superceded by 511 at a later date. |
| Airline Schedule Operations | Existing | Includes Ascot, Horizon, Skywest, United and other airlines offering regular scheduled passenger service. |
| Amtrak Schedule Operations | Existing | This is the operational entity that provides static and real-time Amtrak schedule information for the COATS architecture. |
| California Highway Information Web Page | Existing | This interactive Web-based service (http://www.dot.ca.gov/hq/roadinfo) provides key road construction, detour and closure information based on user-specified route numbers. |
| California Weigh Stations | Existing | These are locations where commercial vehicles may be weighed and inspected. |
| Caltrans District 1 Public Affairs | Existing | This office is responsible for communicating with local media all news regarding key Caltrans activities in District 1. It may be used to disseminate information about road closures, ongoing construction projects, and other activities that may affect highway travel. |
| Caltrans Field Equip District 1 (Eureka) | Existing | This includes the environmental sensors, variable message signs, HAR, signals, traffic detectors, and any other field equipment and instrumentation operated by Caltrans District 1. |
| Caltrans HQ Construction Program | Existing | This program is responsible for implementing the state’s highway construction program. It will provide information to local districts about future construction projects as they are introduced. |
| Caltrans HQ Permits | Existing | The Encroachment Permits department is a part of the office of development services within Caltrans’ Traffic Operations program. As the responsible department for protecting the public’s investment in the State highway system, the permits office reviews all requests from utility companies, developers, volunteers, nonprofit organizations, etc., desiring to conduct various activities within the right of way. Such activities could include for example: construction of highway improvements, driveway installation and maintenance, highway landscaping and graffiti removal, commercial filming, special events such as parades commemorating an event, e.g. Independence Day. |
| Caltrans HQ Traffic | Existing | This office is responsible for coordinating the state’s transportation management centers, facilitating data exchange and interoperability. It will also serve as a repository for field data collected at districts across the state. |
| Caltrans Maintenance District 1 (Eureka) | Existing | Covers the resource management and dispatch systems operated by Caltrans District 1. |
| Caltrans Maintenance District 1 (Eureka) Vehicles | Existing | These Caltrans vehicles are responsible for maintenance activities, including setting up road closures, assisting in incident removal, and performing winter maintenance. It is assumed that there is seamless communication between the vehicle and its driver, who will actually be directing and performing the maintenance activity. |

Table 4-4: Northern California Coast Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Caltrans RWIS | Existing | These are RWIS stations maintained by Caltrans for providing information on atmospheric and pavement temperature conditions at remote sites. Information from these sites is helpful for maintenance officials in making maintenance decisions. |
| Caltrans Weather Warning Systems | Existing | These are driver advisory systems which are activated based on certain meteorological conditions or thresholds (e.g. high wind, icy bridge, etc.). They process information provided by a localized weather station and provide a warning to motorists when specified conditions arise. |
| Caltrans/CHP Eureka ARTIC | Existing | This ARTIC (Advanced Rural Technology Integration Center) plays a central role in the COATS architecture for managing and monitoring the transportation system in Del Norte, Humboldt, Mendocino and Lake Counties. It is a Tier 3 Satellite Operation Center connected to the Oakland Regional TMC. |
| Caltrans/CHP Oakland Regional TMC | Existing | This Tier 1 TMC is tied to Tier 3 Satellite Operation Centers in Eureka and San Luis Obispo. |
| CHIN | Existing | The California Highway Information Network provides daily adverse travel conditions information. Information is made available to telephone and Internet. |
| CHP Eureka and Humboldt Division Dispatch and 911 Centers | Existing | This includes offices in Crescent City, Humboldt, Garberville, Ukiah and Clearlake. |
| CHP Vehicles | Existing | California Highway Patrol vehicles. |
| COATS Kiosks | Planned | This is a planned system which would provide real-time traveler and tourism information to travelers throughout the COATS region. |
| Commercial Vehicle (Onboard) Systems | Existing | This subsystem resides in a commercial vehicle and provides the sensory, processing, storage, and communications functions necessary to support safe and efficient commercial vehicle operations. The Commercial Vehicle Subsystem provides two-way communications between the commercial vehicle drivers, their fleet managers, and roadside officials, and provides HAZMAT response teams with timely and accurate cargo contents information after a vehicle incident. |
| Commercial Vehicle Driver | Existing | This terminator represents the human entity that operates vehicles transporting goods including both long haul trucks and local pick up and delivery vans. |
| Commercial Vehicles | Existing | The actual commercial vehicle along with the special aspects of large commercial vehicles and vehicles designed to carry cargo that extend beyond the characteristics defined for the Basic Vehicle. This terminator thus represents a special type of Basic Vehicle that is used to transport goods or services which are operated by professional drivers, typically administered as part of a larger fleet, and regulated by a Commercial Vehicle Manager. |
| Coos Bay-Area EMS Provider Dispatch | Existing | This includes the broad range of private EMS, ambulance and air evacuation providers that serve the Coos Bay area, including Bay Cities Ambulance, Coquille Valley Ambulance, Lower Umpqua EMS, Medic 4 Ambulance, Myrtle Point, Curry County EMS, and Del Norte. |

Table 4-4: Northern California Coast Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Coos Bay-Area EMS Vehicles | Existing | This includes the broad range of private EMS, ambulance and air evacuation vehicles that serve the Coos Bay area, including Bay Cities Ambulance, Coquille Valley Ambulance, Lower Umpqua EMS, Medic 4 Ambulance, Myrtle Point, Curry County EMS, and Del Norte. |
| CVO Inspector | Existing | This terminator represents the human entities who perform regulatory inspection of Commercial Vehicles in the field. CVO Inspectors support the roadside inspection, weighing, and checking of credentials either through automated preclearance or manual methods. |
| Drivers | Existing | This terminator represents the human entity that operates a licensed vehicle on the roadway. Included are operators of private, Transit, Commercial, and Emergency vehicles where the data being sent or received is not particular to the type of vehicle. Thus this external originates driver requests and receives driver information that reflects the interactions which might be useful to all drivers, regardless of vehicle classification. |
| Eureka-Area Air Districts | Existing | This includes various districts responsible for air quality management, including the North Coast Unified AQMD (responsible for Del Norte, Humboldt and Trinity Counties), the Lake County AQMD and the Mendocino County AQMD. |
| Eureka-Area City Traffic Control Equip | Existing | This includes the field equipment owned and operated by the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Lakeport, Point Arena, Rio Dell, Trinidad. Ukiah and Willits. |
| Eureka-Area City Traffic Control Systems | Existing | This includes the traffic management functions performed by the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Lakeport, Point Arena, Rio Dell, Trinidad. Ukiah and Willits. |
| Eureka-Area County Maintenance Systems | Existing | This includes public works or transportation departments for Del Norte, Humboldt, Lake and Mendocino Counties. |
| Eureka-Area E-911 Comm Centers | Existing | This includes the wireline public safety answering points on the northern California coast, including but not limited to a center in Curry County, Oregon which provides service to northern Del Norte County. |
| Eureka-Area EMS Provider Dispatch | Existing | This includes the broad range of private EMS, ambulance and air evacuation providers, including Arcata Mad River Ambulance Inc, City Ambulance of Eureka, City of Lakeport, Coast Guard, Del Norte Air Ambulance, Hoopa Ambulance, Medivac (CalSTAR), Mendocino Coast Hospital Ambulance, North Coast EMS, North Pacific Emergency Services, Redwood Empire Life Support, and Ukiah Ambulance. |
| Eureka-Area EMS Vehicles | Existing | This includes the broad range of private EMS, ambulance and air evacuation vehicles, including those operated by Arcata Mad River Ambulance Inc, City Ambulance of Eureka, City of Lakeport, Coast Guard, Del Norte Air Ambulance, Hoopa Ambulance, Medivac (CalSTAR), Mendocino Coast Hospital Ambulance, North Coast EMS, North Pacific Emergency Services, Redwood Empire Life Support, and Ukiah Ambulance. |

Table 4-4: Northern California Coast Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Eureka-Area Fire and Rescue Dispatch | Existing | This includes dispatch functions provided for Arcata Fire District, Blue Lake Fire District, Brooktrails Fire Dept , California Department of Forestry, Clearlake Fire District, Coast Guard (Clear Lake), Coast Guard (Fort Bragg), Crescent City Volunteer Fire Department, Eureka Fire District, Ferndale Fire District, Fort Bragg Fire Department, Fortuna Fire District, Garberville Rescue, Lakeport Fire Dept, Redwood Valley Fire District, Samoa Peninsula Fire District, South Trinity Area Rescue, Ukiah Fire Dept, Ukiah Valley Fire District, Upper Lake Fire District, and Willits Fire Department. This also includes California Fish & Game, National Park Service, U.S. Fish & Wildlife and U.S. Forest Service. |
| Eureka-Area Fire and Rescue Vehicles | Existing | This includes vehicles owned by Arcata Fire District, Blue Lake Fire District, Brooktrails Fire Dept , California Department of Forestry, Clearlake Fire District, Coast Guard (Clear Lake), Coast Guard (Fort Bragg), Crescent City Volunteer Fire Department, Eureka Fire District, Ferndale Fire District, Fort Bragg Fire Department, Fortuna Fire District, Garberville Rescue, Lakeport Fire Dept, Redwood Valley Fire District, Samoa Peninsula Fire District, South Trinity Area Rescue, Ukiah Fire Dept, Ukiah Valley Fire District, Upper Lake Fire District, and Willits Fire Department. This also includes California Fish & Game, National Park Service, U.S. Fish & Wildlife and U.S. Forest Service. |
| Eureka-Area Fixed Route Dispatch | Existing | This includes dispatch and management operations for the following transit systems: Arcata-Mad River Transit System, which serves Arcata and Eureka; Eureka Transit Service (run by Humboldt Transit Authority), which provides service to Eureka; Mendocino Transit Authority, which connects cities in Mendocino County; Redwood Coast Transit, which serves Crescent City; and Redwood Transit System (Humboldt Transit Authority), which goes throughout Humboldt County. |
| Eureka-Area Fixed Route Vehicles | Existing | This includes transit vehicles for the following transit systems: Arcata-Mad River Transit System, which serves Arcata and Eureka; Eureka Transit Service (run by Humboldt Transit Authority), which provides service to Eureka; Mendocino Transit Authority, which connects cities in Mendocino County; Redwood Coast Transit, which serves Crescent City; and Redwood Transit System (Humboldt Transit Authority), which goes throughout Humboldt County. |
| Eureka-Area Municipal Planning Agencies | Existing | This includes planning divisions for the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Hoopa, Lakeport, Point Arena, Rio Dell, Trinidad, Ukiah, and Willits. |
| Eureka-Area Paratransit Dispatch | Existing | This includes various paratransit dispatch operations, including Coastal Cab, Crescent City Paratransit Services, Door to Door Airporter, Fortuna Senior Bus, Lake Transit Authority, Mendocino Limousine, Mendocino Transit Authority, Redwood Dial A Ride, Yellow Cab. This also includes various senior centers (such as in Mendocino County) that each have paratransit services. |

Table 4-4: Northern California Coast Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Eureka-Area Paratransit Vehicles | Existing | This includes transit vehicles to support paratransit operations, including Coastal Cab, Crescent City Paratransit Services, Door to Door Airporter, Fortuna Senior Bus, Lake Transit Authority, Mendocino Limousine, Mendocino Transit Authority, Redwood Dial A Ride, Yellow Cab. This also includes various senior centers (such as in Mendocino County) that each have paratransit services. |
| Eureka-Area Police and Sheriff Dispatch | Existing | This includes law enforcement agencies for the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Lakeport, Point Arena, Rio Dell, Trinidad, Ukiah and Willits, and for the Hoopa Tribe. It also includes county sheriff’s offices for Del Norte, Humboldt, Lake and Mendocino Counties. This also includes various other law enforcement agencies with specialized jurisdictions, such as California Department of Forestry, California Fish & Game, National Park Service, and the U.S. Forest Service. |
| Eureka-Area Police and Sheriff Vehicles | Existing | This includes vehicles used for law enforcement by the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Lakeport, Point Arena, Rio Dell, Trinidad, Ukiah and Willits, and for the Hoopa Tribe. It also includes county sheriff’s vehicles for Del Norte, Humboldt, Lake and Mendocino Counties. This also includes vehicles associated with various other law enforcement agencies with specialized jurisdictions, such as California Department of Forestry, California Fish & Game, National Park Service, and the U.S. Forest Service. |
| Eureka-Area Regional Transportation Planning Agencies | Existing | This includes organizations responsible for transportation planning along the northern California coast, including the Del Norte Local Transportation Commission (LTC); the Humboldt County Association of Governments; the Lake Council of Governments; and the Mendocino Council of Governments. |
| Eureka-Area Towing/Recovery | Existing | This includes the private tow dispatch systems that clear incidents in the Eureka area. This includes numerous individual operators that serve in the region. |
| Eureka-Area Traveler Information Resources | Existing | This includes Bureau of Land Management, California Parks & Recreation, Department of Fish and Game, Harbor Recreation and Conservation, National Forest Service, National Park Service, North Coast Redwoods, Redwood National and State Parks, Six Rivers (includes Smith River), various State Parks (including Anderson Marsh, Clear Lake, Jackson, Jughandle, Richardson Grove, Russian Gulch, Standish-Hickey). This also includes City (Clearlake, Ukiah) and County (Humboldt, Lake and Mendocino) Recreation Departments. This also includes area hotels, restaurants, gas stations, shopping, etc. |
| Eureka-Area Traveler Information Systems | Planned | These systems collect traveler information from operational sources (e.g., local traffic and public safety agencies/systems), visitor information systems (e.g. chambers of commerce), and local travel destinations (e.g., Parks). This information is then made available to travelers through both broadcast and interactive digital information services through the Internet and other media. |

Table 4-4: Northern California Coast Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| Eureka-Area TV and Radio Stations | Existing | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the analog TV, radio and print media intended for human, rather than system, processing and interpretation. |
| Eureka-Area Visitor Information Services | Existing | This includes chambers of commerce for Clearlake, Eureka, Hopland, Lakeport, Mendocino Coast/Fort Bragg, Ukiah, and Willits. Eureka Convention and Visitors Bureau, Mendocino County Convention and Visitors Bureau, Northern California Tourism, and Redwood Empire Association. |
| HELP | Existing | HELP (Heavy Vehicle Electronic License Plate, Inc.), is a non-profit partnership between motor carriers and government agencies (including Caltrans) whose mission is to develop and deploy advanced technology systems to create a cooperative operating and regulatory environment which improves the efficient and safe movement of commercial vehicles and the performance of highway systems. |
| Inter-City Transit | Existing | This includes dispatch systems for Amtrak ThruWay Bus, Coastline Enterprises, Greyhound, North Coast Railroad and Porter Stage Lines. |
| Inter-City Transit Vehicles | Existing | This includes transit vehicles for Amtrak ThruWay Bus, Coastline Enterprises, Greyhound, and Porter Stage Lines. |
| IRP Clearinghouse | Existing | This is a registration reciprocity agreement among jurisdictions in the United States and Canada which provides for payment of license fees on the basis of fleet miles operated in various jurisdictions |
| Mayday Service Provider | Existing | Service centers that provide Mayday and other special concierge services for their clients. These centers will typically be outside the COATS region, but will interact with local agencies when their client’s have an emergency in the region. Current examples of this system include the service centers operated by GM (OnStar), ATX Technologies (Ford and others), and AAA. |
| MCMIS | Existing | MCMIS contains information on the safety fitness of commercial motor carriers and hazardous material shippers subject to the Federal Motor Carrier Safety Regulations and the Hazardous Materials Regulations. This information is available to the general public through the MCMIS Data Dissemination Program. |
| Motor Carrier Dispatch and Back Office Systems | Existing | These provide the capability for commercial drivers and dispatchers to receive real-time routing information and access databases containing vehicle and cargo locations as well as carrier, vehicle, cargo, and driver information. |
| Other Weather Data | Existing | This terminator refers to weather information received from the National Weather Service or state agencies which also have weather-related information. This could include fully equipped weather stations, isolated stream gauges, or other similar technologies. |

Table 4-4: Northern California Coast Inventory (cont.).

| Element Name | Status | Description |
| --- | --- | --- |
| SAFER | Existing | SAFER provides carrier, vehicle, and driver safety and credential information to fixed and mobile roadside inspection stations. This information will allow the roadside inspector to select vehicles and/or drivers for inspection based on the number of prior carrier inspections, as well as carrier, vehicle, and driver safety and credential historical information. |
| Transit Users | Existing | This terminator represents the human entities using Public Transit vehicles. They may be in the act of embarking or debarking the vehicles and are thus sensed for the purpose of determining passenger loading and fares, or on the vehicles and able to request and receive information. |
| Traveler Information Devices | Planned | These are personal devices which may be used to access real-time transportation information. |
| Travelers | Existing | This terminator represents any individual (human) who uses transportation services. At the time that data is passed to or from the terminator the individual is neither a driver, pedestrian, or transit user. This means that the data provided is that for pre-trip planning or multi-modal personal guidance and includes their requests for assistance in an emergency. Subsequent to receipt of pre-trip information, a Traveler may become a vehicle driver, passenger, transit user, or pedestrian. |
| Vehicle Information Systems | Existing | These are devices installed in passenger vehicles that support access to digital traveler information. A wide range of evolving technologies are available. |

## Subregional Architecture Flow Definitions

Table 4-5 lists every architecture flow that is included in the subregional architectures presented in the previous sections. Some flows were customized for the COATS study area as described in Chapter 3 (as indicated by asterisks), but all of the remaining architecture flow definitions were extracted from the definitions used in version 3.0 of the National ITS Architecture. The National ITS Architecture also identifies relevant standards information for each of these architecture flows. Consult the National ITS Architecture web site at http://www.iteris.com/itsarch for the latest standards information that is relevant to the COATS architecture.

Table 4-5: Architecture Flows Included in COATS Regional ArchitectureTable 4-5: Architecture Flows Included in COATS Regional Architecture.

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| activity reports |  | Activity reports containing records of citations, accidents, inspections, etc. |
| archive coordination |  | Catalog data, meta data, published data, and other information exchanged between archives to support data synchronization and satisfy user data requests. |
| archive requests |  | A request to a data source for information on available data (i.e. “catalog”) or a request that defines the data to be archived. The request can be a general subscription intended to initiate a continuous or regular data stream or a specific request intended to initiate a one-time response from the recipient. |
| archive status |  | Notification that data provided to an archive contains erroneous, missing, or suspicious data or verification that the data provided appears valid. If an error has been detected, the offending data and the nature of the potential problem are identified. |
| basic vehicle measures |  | Information provided to on-board ITS equipment from the vehicle platform indicating current vehicle status. |
| border clearance event record |  | Results of border clearance check. |
| broadcast advisories |  | General broadcast advisories that are provided over wide-area wireless communications direct to the vehicle radio. These analog advisory messages may provide similar content to ITS broadcast information flows, but include no digital data component. Existing Highway-Advisory Radio (HAR) advisory messages are a prime example of this flow. |
| broadcast information |  | General broadcast information that contains link travel times, incidents, advisories, transit services and a myriad of other traveler information. |
| citation data |  | Safety problems related to the carrier, driver and vehicle that may lead to a citation. |
| clearance event record |  | Results of vehicle clearance activity. |
| closure coordination |  | Coordination between subsystems regarding construction and maintenance closure times and durations. |
| compliance review report |  | Report containing data from facility activity logs from various roadside facilities. |
| construction activity status | Yes | Information about impending construction projects coordinated at a statewide level and distributed to districts. |
| construction and maintenance archive data |  | Information describing road construction and maintenance activities identifying the type of activity, the location of the activity, and the activity status. Content may include a catalog of available information, the actual information to be archived, and associated meta data that describes the archived information. |
| credential application |  | Application for commercial vehicle credentials for a particular route/trip. |
| credentials and safety information request |  | Request for additional credentials and safety information. |

Table 4-5: Architecture Flows Included in COATS Regional Architecture (cont.).

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| credentials and safety information response |  | Instructions to commercial vehicle managing and/or information systems indicating which vehicles are to be allowed to pass and which are out of service or have not been credentialed. |
| credentials information |  | Response containing credentials information. |
| credentials information request |  | Request for credential information. |
| current network conditions |  | Current traffic information, road conditions, and camera images that can be used to locate and verify reported incidents, and plan and implement an appropriate response. |
| CVAS information exchange |  | Tax and credential fee information exchanged between cooperating commercial vehicle administration offices (e.g. regional or inter-state preclearance data). |
| CVC override mode |  | Manual override by the commercial vehicle roadside facility inspector of automated pass/pull-in signage information. |
| CVO database update |  | Credential information and safety problem list updates. |
| CVO driver initialization |  | Commercial vehicle driver and vehicle information and requests to the commercial vehicle managing system. |
| CVO Pull in Message |  | Message sent to commercial vehicle driver requesting pull in to inspection/verification stop along with inspection results. |
| CVO weight and presence |  | Weigh-In-Motion message to indicate presence of commercial vehicle and its weight. |
| demand responsive transit plan |  | Plan regarding overall demand responsive transit schedules and deployment. |
| demand responsive transit request |  | Request for paratransit support. |
| driver and vehicle information |  | Requests from the driver and vehicle for routing, payment, and enrollment information. |
| driver information |  | General advisory and traffic control information provided to the driver while en-route. |
| driver inputs |  | Driver commands to the vehicle. |
| electronic clearance data |  | Information required for electronic clearance (toll, safety, customs, etc.). |
| electronic clearance request |  | Request for electronic clearance data (Toll, safety, customs, etc.). |
| electronic credentials |  | Authenticated credentials including route enrollment and payment confirmation. |
| emergency acknowledge |  | Acknowledge request for emergency assistance and provide additional details regarding actions and verification requirements. |
| emergency archive data |  | Logged incident information that characterizes the identified incidents and provides a record of the corresponding incident response. Content may include a catalog of available information, the actual information to be archived, and associated meta data that describes the archived information. |

Table 4-5: Architecture Flows Included in COATS Regional Architecture (cont.).

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| emergency data request |  | A request for additional information or a control command issued by the emergency response agency in response to an emergency request for assistance from a traveler. |
| emergency dispatch requests |  | Emergency vehicle dispatch instructions including incident location and available information concerning the incident. |
| emergency notification |  | An emergency request for assistance originated by a traveler using an in-vehicle, public access, or personal device. Sufficient information is provided so that the recipient can determine the location of the emergency as a minimum. Additional information identifying the requestor and requesting device and the nature and severity of the emergency may also be provided (and required) by some systems. |
| emergency traffic control request |  | Special request to preempt the current traffic control strategy in effect at one or more signalized intersections or highway segments. For example, this flow can request all signals to red-flash, request a progression of traffic control preemptions along an emergency vehicle route, or request another special traffic control plan. |
| emergency vehicle tracking data |  | The current location and operating status of the emergency vehicle. |
| emissions archive data |  | Air quality and vehicle emissions information that is collected by sensors or derived from models. Content may include a catalog of available information, the actual information to be archived, and associated meta data that describes the archived information. |
| emissions data |  | Emissions data and associated imagery collected by roadside equipment. |
| environmental conditions |  | Current environment conditions (e.g., air temperature, wind speed, surface temperature) as measured by environmental sensors and communicated by supporting field equipment. |
| equipment maintenance status |  | Current status of field equipment maintenance actions. |
| external reports |  | Traffic and incident information that is collected by the media through a variety of mechanisms (e.g., radio station call-in programs, air surveillance). |
| fault reports |  | Reports from field equipment (sensors, signals, signs, controllers, etc.) which indicate current operational status. |
| fleet to driver update |  | Updated instructions to the driver including dispatch, routing, and special instructions |
| incident command information |  | Information that supports local management of an incident. It includes resource deployment status, hazardous material information, traffic, road, and weather conditions, evacuation advice, and other information that enables emergency personnel in the field to implement an effective, safe incident response. |
| incident command request |  | Request for resources, commands for relay to other allied response agencies, and other requests that reflect local command of an evolving incident response. |

Table 4-5: Architecture Flows Included in COATS Regional Architecture (cont.).

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| incident data |  | Data and imagery from the roadside supporting incident detection and verification. |
| incident data* | Yes | Data and imagery from the roadside supporting incident detection and verification. |
| incident information |  | Notification of existence of incident and expected severity, location, time and nature of incident. |
| incident information for media |  | Report of current desensitized incident information prepared for public dissemination through the media. |
| incident information request |  | Request for incident information, clearing time, severity. The request can be a subscription that initiates as-needed information updates as well as a one-time request for information. |
| incident information* | Yes | Notification of existence of incident and expected severity, location, time and nature of incident. |
| incident report |  | Report of an identified incident including incident location, type, severity and other information necessary to initiate an appropriate incident response. |
| incident response coordination |  | Incident response procedures, resource coordination, and current incident response status that are shared between allied response agencies to support a coordinated response to incidents. This flow also coordinates a positive hand off of responsibility for all or part of an incident response between agencies. |
| incident response status |  | Status of the current incident response including traffic management strategies implemented at the site (e.g., closures, diversions, traffic signal control overrides). |
| incident response vehicle coordination | Yes | Communication between vehicles involved in emergency or incident response activities. This communication is essential to reducing the impact of incidents, especially road closures, on travelers. It may require several different communication media, as this will typically span over many jurisdictional lines. |
| incident status |  | Information gathered at the incident site that more completely characterizes the incident and provides current incident response status. |
| information request |  | General purpose information request for data stored within the commercial vehicle operations information exchange network. |
| international border crossing data |  | Cleared commercial vehicle data to allow pass-thru international border crossings. |
| international border crossing data update |  | Update from commercial vehicle check stations of international border crossing events. |
| ISP coordination |  | Coordination and exchange of transportation information between centers. This flow allows a broad range of transportation information collected by one ISP to be redistributed to many other ISPs and their clients. |
| local signal preemption request |  | Direct control signal or message to a signalized intersection that results in preemption of the current control plan and grants right-of-way to the requesting vehicle. |

Table 4-5: Architecture Flows Included in COATS Regional Architecture (cont.).

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| local signal priority request |  | Request from a vehicle to a signalized intersection for priority at that intersection. |
| localized weather information | Yes | Select weather information (e.g. wind, precipitation, humidity, temperature, etc.) that may be used to active a local weather warning system. The information is transmitted between a field-based weather data station and a collocated traveler advisory system. |
| lock tag data |  | Tag information on cargo lock. |
| lock tag data request |  | Request to supply lock information on cargo lock for retransmission to international border crossing station. |
| maintenance activity coordination | Yes | Information communicated from the maintenance dispatch center to maintenance vehicles to coordinate maintenance activities. This could include information about locations of activities, resource deployment status, and any actions which should be taken. |
| maintenance activity information | Yes | Information about non-planned and planned road conditions that may affect travel conditions, including construction-related lane closures and detours, and weather-related events. |
| maintenance activity status | Yes | Any communication sent from maintenance vehicles back to the maintenance dispatch office. This would include information about the current status of the action, expected duration of an action, and the need for additional resources. |
| maintenance dispatch office coordination | Yes | Information communicated between different maintenance dispatch centers regarding locations of critical maintenance actions, like road closures, detours, winter road maintenance, etc. |
| maintenance resource request |  | Request for road maintenance resources that can be used in the diversion of traffic (cones, portable signs), clearance of an incident, and repair of ancillary damage. |
| maintenance resource response |  | Current status of maintenance resources included availability and deployment status. |
| maintenance vehicle coordination | Yes | Any type of vehicle to vehicle communication between maintenance vehicles. |
| maintenance vehicle location | Yes | Information transmitted from maintenance vehicles to the maintenance dispatch office, characterizing the vehicle’s location. |
| media information request |  | Request from the media for current transportation information. |
| media information request* | Yes | Request from the media for current transportation information. |
| multimodal information |  | Schedule information for alternate mode transportation providers such as train, ferry, air and bus. |
| multimodal information request |  | Information request for alternate mode transportation providers such as train, ferry, air and bus. |
| on board safety data |  | Vehicle safety data measured by vehicle sensors and sent to inspection stations |
| on board vehicle data |  | Condition of the commercial vehicle sent to commercial vehicle manager primarily for maintenance purposes. |

Table 4-5: Architecture Flows Included in COATS Regional Architecture (cont.).

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| on-board safety request |  | Request for onboard vehicle safety data. |
| parking archive data |  | Data used to analyze and monitor trends in parking demand, pricing, and operational actions. Content may include a catalog of available information, the actual information to be archived, and associated meta data that describes the archived information. |
| parking availability |  | Current parking lot occupancy, parking availability, and cost information. |
| parking demand management request |  | Request to change the demand for parking facility use through pricing or other mechanisms. |
| parking demand management response |  | Response to parking demand management change requests indicating level of compliance with request. |
| parking instructions |  | Information that allows local parking facilities to be managed to support regional traffic management objectives. |
| pass/pull-in |  | Command to commercial vehicle to pull into inspection station. |
| permit request | Yes | Information about the date, time and nature of activities permitted on the right-of-way. |
| permit support request | Yes | Information provided to maintenance staff to assist in permits for right-of-way access granted by the permitting agency. This would include date, time and location of requests for infringement of highway right-of-way, along with details about any precautions that maintenance staff should undertake to protect the traveling public, maintenance staff, and those authorized onto the right-of-way, including traffic cones, VMS messages, flaggers, etc. |
| permit support request confirmation | Yes | Maintenance dispatch offices confirm that information regarding a specific permit request has been understood, and that any maintenance tasks required to protect the traveling public and permittees will be undertaken. |
| pollution data |  | Measured emissions data comprised of various atmospheric pollutants. |
| provider profile confirm |  | Confirmation of profile information received by a service provider (e.g. for a hotel or restaurant). |
| provider profile data |  | Information supplied by a service provider (e.g., a hotel or restaurant) that identifies the service provider and provides details of the service offering. This flow covers initial registration of a service provider and subsequent submittal of new information and status updates so that data currency is maintained. |
| request for right-of-way |  | Forwarded request from signal prioritization, signal preemption, pedestrian call, multi-modal crossing activation, or other source for right-of-way. |
| request for service |  | A traveler service request initiated by a driver or traveler. The request may result in a financial transaction, summon an emergency response, or initiate another service at the behest of the driver. |

Table 4-5: Architecture Flows Included in COATS Regional Architecture (cont.).

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| request for traffic information |  | Request for traffic information that specifies the region/route of interest, the desired effective time period, and other parameters that allow preparation of a tailored response. The request can be a subscription that initiates as-needed information updates as well as a one-time request for information. |
| resource deployment status |  | Status of traffic management center resource deployment identifying the resources available and their current deployment status. |
| resource request |  | A request for traffic management resources to implement special traffic control measures, assist in clean up, verify an incident, etc. |
| road construction information for transit | Yes | Information regarding construction activities which may affect transit service, requiring long-term realignment of routes and/or delay to transit users. |
| road network use |  | Aggregated route usage and associated travel data from clients for planning and analysis. |
| roadside archive data |  | A broad set of data derived from roadside sensors that includes current traffic conditions, environmental conditions, and any other data that can be directly collected by roadside sensors. This data also indicates the status of the sensors and reports of any identified sensor faults. |
| roadside log update |  | Update of activities at commercial vehicle check stations including clearance events and inspection reports. |
| roadway information system data |  | Information used to initialize, configure, and control roadside systems that provide driver information (e.g., dynamic message signs, highway advisory radio, beacon systems). This flow can provide message content and delivery attributes, local message store maintenance requests, control mode commands, status queries, and all other commands and associated parameters that support remote management of these systems. |
| roadway information system status |  | Current operating status of dynamic message signs, highway advisory radios, beacon systems, or other configurable field equipment that provides dynamic information to the driver. |
| safety information |  | Response containing commercial vehicle safety information. |
| safety information request |  | Request for commercial vehicle safety information. |
| safety inspection record |  | Record containing results of commercial vehicle safety inspection. |
| screening data |  | Data stored in vehicle’s tag allowing electronic clearance at border crossings, debits at toll plazas, and clearance at safety inspections. |
| screening request |  | Request for screening data based on vehicle and possibly cargo’s tags. |
| seismic conditions | Yes | Current seismic conditions as measured by remote sensors and communicated by supporting field equipment. |
| selected routes |  | Routes selected based on route request criteria. |
| sensor and surveillance control |  | Information used to configure and control sensor and surveillance systems at the roadside. |

Table 4-5: Architecture Flows Included in COATS Regional Architecture (cont.).

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| signal control data |  | Information used to configure and control traffic signal systems. |
| suggested route |  | Suggested route for a dispatched emergency vehicle that may reflect current network conditions and the additional routing options available to en-route emergency vehicles that are not available to the general public. |
| tax filing, audit data |  | Commercial vehicle tax filing and audit data. |
| traffic archive data |  | Information describing the use and vehicle composition on transportation facilities and the traffic control strategies employed. Content may include a catalog of available information, the actual information to be archived, and associated meta data that describes the archived information. |
| traffic control coordination |  | Information transfers that enable remote monitoring and control of traffic management devices. This flow is intended to allow cooperative access to, and control of, field equipment during incidents and special events and during day-to-day operations. This flow also allows 24-hour centers to monitor and control assets of other centers during off-hours, allows system redundancies and fail-over capabilities to be established, and otherwise enables integrated traffic control strategies in a region. |
| traffic equipment status |  | Identification of field equipment requiring repair and known information about the associated faults. |
| traffic flow |  | Raw and/or processed traffic detector information which allows derivation of traffic flow variables (e.g., speed, volume and density measures). |
| traffic images |  | High fidelity, real-time traffic images suitable for surveillance monitoring by the operator or for use in machine vision applications. |
| traffic images* | Yes | High fidelity, real-time traffic images suitable for surveillance monitoring by the operator or for use in machine vision applications. |
| traffic information |  | Current and forecasted traffic information, road and weather conditions, incident information, and pricing data. Either raw data, processed data, or some combination of both may be provided by this architecture flow. |
| traffic information coordination |  | Traffic information exchanged between TMC’s. Normally would include incidents, congestion data, traffic data, signal timing plans, and real-time signal control information. |
| traffic information for media |  | Report of current traffic conditions, incidents, maintenance activities and other traffic-related information prepared for public dissemination through the media. |
| traffic information for media* | Yes | Report of current traffic conditions, incidents, maintenance activities and other traffic-related information prepared for public dissemination through the media. |
| traffic information for transit |  | Current and forecasted traffic information and incident information. |

Table 4-5: Architecture Flows Included in COATS Regional Architecture (cont.).

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| transit and fare schedules |  | Specific transit and fare schedule information including schedule adherence. |
| transit archive data |  | Data used to describe and monitor transit demand, fares, operations, and system performance. Content may include a catalog of available information, the actual information to be archived, and associated meta data that describes the archived information. |
| transit emergency coordination data |  | Data exchanged between centers dealing with a transit-related incident. |
| transit emergency data |  | Initial notification of transit emergency at a transit stop or on transit vehicles and further coordination as additional details become available and the response is coordinated. |
| transit information for media |  | Report of transit schedule deviations for public dissemination through the media. |
| transit information request |  | Request for transit operations information including schedule and fare information. The request can be a subscription that initiates as-needed information updates as well as a one-time request for information. |
| transit request confirmation |  | Confirmation of a request for transit information or service. |
| transit schedule information |  | Current and projected transit schedule adherence. |
| transit system data |  | Current transit system operations information indicating current transit routes, the level of service on each route, and the progress of individual vehicles along their routes for use in forecasting demand and estimating current transportation network performance. |
| transit traveler information |  | Transit information prepared to support transit users and other travelers. It contains transit schedules, real-time arrival information, fare schedules, and general transit service information. |
| transit traveler request |  | Request by a Transit traveler to summon assistance, request transit information, or request any other transit services. |
| transit vehicle location data |  | Current transit vehicle location and related operational conditions data provided by a transit vehicle. |
| transit vehicle schedule performance |  | Estimated times of arrival and anticipated schedule deviations reported by a transit vehicle. |
| travel service info |  | Reservation information or yellow pages data. |
| travel service request |  | Request for reservation or other service (e.g., yellow pages). |
| traveler information |  | Traveler information comprised of traffic status, advisories, incidents, payment information and many other travel-related data updates and confirmations. |
| traveler information for media |  | General traveler information regarding incidents, unusual traffic conditions, transit issues, or other advisory information that has been desensitized and provided to the media. |

Table 4-5: Architecture Flows Included in COATS Regional Architecture (cont.).

| Flow Name | Customized? | Flow Description |
| --- | --- | --- |
| traveler inputs |  | Request by a traveler to summon assistance, request travel information, make a reservation, or request any other traveler service. |
| traveler interface updates |  | Visual or audio information (e.g., routes, messages, guidance) to the traveler. |
| traveler profile |  | Information about a traveler including equipment capabilities, personal preferences and recurring trip characteristics. |
| traveler request |  | Request by a traveler to summon assistance, request information, make a reservation, or initiate any other traveler service. |
| TRMS coord |  | Coordination information between local/regional transit organizations including schedule, on-time information and ridership. |
| vehicle control |  | Vehicular control commands |
| vehicle pollution criteria |  | Vehicular pollution acceptance criteria. |
| weather information |  | Accumulated forecasted and current weather data (e.g., temperature, pressure, wind speed, wind direction, humidity, precipitation, visibility, light conditions, etc.). |
| weather information coordination | Yes | Information exchange between agencies who are collecting weather-related information, including generalized atmospheric and precipitation conditions, as well as local stream gauge readings. |
| web traveler information data | Yes | A variety of information that may be provided to a Web-browser traveler information interface, including congestion maps, camera images, construction and road closure reports, weather-related traveler advisories, transit access, etc. |
| widearea statistical pollution information |  | Aggregated region-wide measured emissions data and possible pollution incident information. |
| work zone status |  | Status of maintenance work zone. |
| yellow pages information |  | Travel service information covering tourist attractions, lodging, restaurants, service stations, emergency services, and other services and businesses of interest to the traveler. |
| yellow pages request |  | Request for information through a yellow pages type service. |

This page is intentionally blank

# Early Winner Project

One important benefit of developing a regional architecture is that it can assist in the project development process. Therefore, as shown in Figure 5-1, the next step in developing the COATS Regional Architecture was to define early project architectures. Of the six candidate projects identified in the COATS early winner report (8), the Bi-State Traveler Safety and Incident Management System centered around Siskiyou Pass was selected for early demonstration and evaluation. Consequently, this project was selected for further architecture development.

This chapter identifies the portions of the COATS Regional Architecture that will be implemented by the Siskiyou Pass early winner project. The basic architecture definition for this project is extended to identify specific architecture flows that will be implemented and ITS standards that may be relevant to the project.

## Figure 5-1: Define Early Project ArchitecturesFigure 5-1: Define Early Project Architectures.Project Background

Interstate 5 from Yreka, California to Medford, Oregon is a major connecting route between California and Oregon, and is part of a significant north-south corridor between Canada and Mexico. Siskiyou Pass, located approximately two miles north of the state border on Interstate 5, is at an elevation of 1,314 meters (4,310 feet), and consequently is subject to significant weather-related transportation challenges, including rapidly changing atmospheric conditions, reduced visibility due to fog, rain and/or snow, and slippery pavement. Daily traffic volumes at Siskiyou Pass are approximately 12,000 vehicles per day (9), representing the highest volume traffic crossing between the two states (10). Of that traffic, approximately 36 percent is commercial vehicle traffic (10).

## Project Architecture

The early winner project includes a variety of technologies to facilitate traveler information, roadway maintenance and incident management activities in the Siskiyou Pass vicinity. Figures 5-2 and 5-3 show simplified architecture flow diagrams representing existing and short-term deployment in Oregon and California, respectively. These figures were developed based off of the Turbo Architecture database, with customized flows and subsystems created as necessary to reflect deployed technologies. These diagrams are simplified in that they represent the subset of equipment deployed on either side of the border without showing how the two states will relate to each other. The diagrams also do not include the current and future coordination of ODOT and Caltrans with law enforcement agencies on both sides of the border to manage incidents on the pass.

In Figure 5-2, it is clear that the Medford TOC is pivotal in the management of ITS infrastructure on the pass. Projects included with the early winner include closed-circuit television (CCTV) cameras, road weather information systems (RWIS), and variable message signs. These elements will allow dispatchers to more readily determine conditions on Siskiyou Pass, and communicate that information to motorists. Several other elements on the Oregon side of the border are being considered as well.

- Cable Television Station. From project participants and stakeholder input it was determined that infrastructure that supports pre-trip traveler information through a cable advisory television channel would be beneficial. This channel would also display closed circuit television (CCTV) images of road conditions.

- Advanced Vehicle Detection. These systems could provide additional information about traffic volumes and speeds throughout the pass area.

- Automatic Vehicle Location. Installed on maintenance fleets, this technology could help to optimize the efficiency of vehicles in clearing the roadway and responding to incidents.

- Motorist-Aide Call Boxes. When conditions at the pass break down, installation of these call boxes can help motorists to summon help, such as a tow truck or emergency services.

- Information Kiosks. Kiosks currently installed in the Ashland and Medford areas, which are maintained by the Southern Oregon Visitors Association (SOVA), are being upgraded to take advantage of the real-time information available through the

Figure 5-2: Early Winner Project Architecture (Oregon)Figure 5-2: Early Winner Project Architecture (Oregon).

Western Transportation Institute83

84Western Transportation Institute

Figure 5-3: Early Winner Project Architecture (California)Figure 5-3: Early Winner Project Architecture (California).Internet. Once this has been accomplished, these kiosks may provide information similar to the cable television station, a link to ODOT’s TripCheck interface, or other helpful pre-trip information.

- Automated Anti-Icing. This technology involves spot application of anti-icing chemicals at areas which tend to freeze earlier and more frequently, such as bridges. The Medford viaduct was identified as a short-term location for this technology. This can reduce the number of trips maintenance workers need to devote to the viaduct and potentially direct more of their efforts to keeping the pass open.

- Dynamic Warning Variable Message Signs. This technology combines dynamic messaging with vehicle detection technology to provide customized messages to vehicles regarding appropriate driving speeds. Short-term deployment is recommended for the immediate vicinity around the Pass, with one sign in each travel direction.

- Automated Visibility Warning. To deal with sudden poor visibility in the pass area, automated visibility warning systems may provide advanced notification to drivers in order to reduce rear-end collisions. This system may notify the TOC when it is activated.

- Highway Advisory Radio. Located in Medford, this proposed deployment would help to provide en-route traveler information in addition to variable message signs. This technology permits more detailed information about expected delays, detours or chain requirements to be provided.

The architecture flow diagram shows each of these technologies having real-time communications with the TOC. The specific media employed will depend upon the availability of nearby landline coverage and/or the coverage area for wireless communications. The amount of data required to be transmitted will vary considerably between RWIS, which send a short stream of textual data on an intermittent basis, and CCTV, which are capable of transmitting real-time video. In addition to the data exchange indicated on the flow diagram, technologies are shown sending “fault reports” to the TOC, which may help to provide early notification of malfunctioning equipment.

Figure 5-2 presents a simplified picture in that it does not include the communications involved with the Siskiyou Pass Incident Management Plan, which coordinates emergency response and dispatch activities among several agencies on both sides of the state border, and SOVA, which would continue to be responsible for kiosks.

Figure 5-3 depicts data flows on the California side of the border. The Redding ARTIC has responsibility over this section of Interstate 5, and will have oversight over Caltrans’ ITS devices. Caltrans will be employing similar technologies as ODOT. However, as different vendors may be used to supply technologies, it is critical that a conscious effort be made to coordinate information flow between the Medford TOC and Redding ARTIC, as shown in Figure 5-4. As ITS infrastructure in the vicinity of the border increases, it is critical that information exchange across the state boundary be as seamless as possible.

Table 5-1 provides a brief definition of each architecture flow in the figure and an indication of how the flow will be used in the COATS region.

Figure 5-4: Potential Coordination Flow Between Medford TOC and Redding ARTICFigure 5-4: Potential Coordination Flow Between Medford TOC and Redding ARTIC.

The core project includes controllers, a variable message sign, a kiosk, RWIS sensor stations, CCTV cameras and the communications system that connects these field elements with the Medford TOC or the Redding ARTIC. Several major avenues for project enhancement are available. These include: 1) the integration of deployed RWIS, traffic monitoring, and variable message sign technologies to implement dynamic warning systems that warn drivers that are driving too fast for current conditions; and 2) broader distribution of current pass conditions, including CCTV camera images, via a local cable channel or the internet. These more advanced features are taken into consideration in the project architecture so that these expansion options can be addressed in future upgrades.

## Related ITS Standards

The standards that are related to the early winner project are identified in Table 5-2. Standards are documented agreements that contain technical specifications or other precise criteria to be used consistently as rules, guidelines, or definitions of characteristics for the interchange of data.  Each row in the table identifies a standard and the architecture flow that it supports. Standards that are applicable to multiple flows are repeated more than once in the table. The last three columns in the table indicate whether the standard establishes communications protocol, data dictionary, or message set standards for ITS. Communications protocol standards define sets of rules for moving data and associated messages. A data dictionary standard defines the organization of the database so that the data is given meaning. The message set standards identify associated data elements for typical applications. All three types of standards may be required to fully define an interoperable interface. This comprehensive standards list was generated based on the relationships between the project architecture, the National ITS Architecture, and the standards activities. These lists should be reviewed and refined to identify the subset of these standards that should actually be considered for use in project implementation.

Table 5-1: Regional Architecture Flows Related to Early Winner ProjectTable 5-1: Regional Architecture Flows Related to Early Winner Project.

| Source | Destination | Flow | Definition | COATS Application |
| --- | --- | --- | --- | --- |
| Advanced Vehicle Detection | Medford TOC and Redding ARTIC | traffic flow | Raw and/or processed traffic detector information which allows derivation of traffic flow variables (e.g., speed, volume and density measures). | “Advanced” refers to non-intrusive vehicle detection technologies that could perform acceptably under the varied climatic conditions at Siskiyou Pass. Traffic data provided by these detection stations could supplement information provided by CCTV cameras about current traffic conditions. |
| Automated Anti-Icing, Automated Visibility Warning, Dynamic Warning VMS, Highway Advisory Radio, Variable Message Sign | Medford TOC and Redding ARTIC | roadway information system status | Current operating status of dynamic message signs, highway advisory radios, beacon systems, or other configurable field equipment that provides dynamic information to the driver. | Each of these technologies can return their current operational status. In addition to this, the Dynamic Warning VMS can return current traffic measurements that can be used to determine the affect the sign is having on average vehicle speeds. |
| Automated Anti-Icing, Dynamic Warning VMS | Medford TOC | environmental conditions | Current environment conditions (e.g., air temperature, wind speed, surface temperature) as measured by environmental sensors and communicated by supporting field equipment. | Sensors that measure current conditions in order to provide more accurate dynamic warnings to drivers can also be used for remote sensing and data collection at the TOC/ARTIC. |
| Automated Visibility Warning, Dynamic Warning VMS, and Variable Message Sign | Drivers & Vehicles | driver information | General advisory and traffic control information provided to the driver while en-route. | The Variable Message Sign will receive messages directly from the TOC, while the other two technologies are designed to provide information to motorists independently of human input. Due to space limitations, communications using these technologies are required to be brief but clear. |
| Automatic Vehicle Location | Medford TOC | fleet vehicle status* | For a maintenance vehicle, this could include not only the vehicle’s location, but also its current maintenance activity (i.e. plow raised, door open, etc.). | Maintenance vehicles equipped with AVL could be dispatched more effectively in order to expedite response to incidents and adverse road surface conditions. |
| CCTV Cameras | Medford TOC and Redding ARTIC | incident data | Data and imagery from the roadside supporting incident detection and verification. | Included in the initial project architecture. CCTV may be used to verify the location and severity of incidents in the Siskiyou Pass area. Their area of coverage will be limited by surrounding topography and visibility conditions, and consequently will not be able to verify all incidents occurring in the pass area. |
| CCTV Cameras | Medford TOC and Redding ARTIC | traffic images | High fidelity, real-time traffic images suitable for surveillance monitoring by the operator or for use in machine vision applications. | Included in the initial project architecture. Although the National ITS Architecture definition specifies “real-time” and “high fidelity,” this is not a true requirement for many applications, including this COATS application. |

Western Transportation Institute87

* - Customized Architecture Flow

Table 5-1: Regional Architecture Flows Related to Early Winner Project (cont.).

| Source | Destination | Flow | Definition | COATS Application |
| --- | --- | --- | --- | --- |
| Highway Advisory Radio | Drivers & Vehicles | broadcast advisories | General broadcast advisories that are provided over wide-area wireless communications direct to the vehicle radio. These analog advisory messages may provide similar content to ITS broadcast information flows, but include no digital data component. | These systems could provide more detailed en-route traveler information based on current conditions, such as descriptions of detours or road closures, information about chains requirements, and estimates of expected travel delay. |
| Information Kiosk | Medford TOC | traveler request | Request by a traveler to summon assistance, request information, make a reservation, or initiate any other traveler service. | This flow would not be required for an initial implementation since the kiosk does not necessarily need to forward traveler requests back to the TOC/ARTIC. More advanced interactive services could be supported by this flow in future implementations. |
| Medford TOC | Automatic Vehicle Location | fleet vehicle command* | Requests provided by dispatchers to vehicle operators to place vehicle activities in locations where they would be most beneficial. | The chain of command as shown in the flow diagram may be simpler than actually occurs. Other agencies involved in traffic management, such as law enforcement, may also be involved in vehicle dispatching and command decisions. |
| Medford TOC and Redding ARTIC | Cable Television Station | traveler information for media | General traveler information regarding incidents, unusual traffic conditions, transit issues, or other advisory information that has been desensitized and provided to the media. | Would also include camera images in this implementation, although this is not specifically identified in the National ITS Architecture description. |
| Medford TOC and Redding ARTIC | CCTV Cameras | sensor and surveillance control | Information used to configure and control sensor and surveillance systems at the roadside. | Included in the initial project architecture. This is the interface between the sign display and the driver’s eyes which is subject to uniformity standards (e.g., MUTCD) but as a human interface, is not the focus of the regional architecture. |
| Medford TOC and Redding ARTIC | Highway Advisory Radio, Variable Message Sign | roadway information system data | Information used to initialize, configure, and control roadside systems that provide driver information (e.g., dynamic message signs, highway advisory radio, beacon systems). This flow can provide message content and delivery attributes, local message store maintenance requests, control mode commands, status queries, and all other commands and associated parameters that support remote management of these systems. | Included in the initial project architecture. The highway advisory radio and variable message sign are additions to an existing system, which will influence standards decisions for this interface. |

* - Customized Architecture Flow

88Western Transportation Institute

Table 5-1: Regional Architecture Flows Related to Early Winner Project (cont.).

| Source | Destination | Flow | Definition | COATS Application |
| --- | --- | --- | --- | --- |
| Medford TOC and Redding ARTIC | Dynamic Warning VMS | roadway information system data | Information used to initialize, configure, and control roadside systems that provide driver information (e.g., dynamic message signs, highway advisory radio, beacon systems). This flow can provide message content and delivery attributes, local message store maintenance requests, control mode commands, status queries, and all other commands and associated parameters that support remote management of these systems. | Preliminary project descriptions indicate the first DWS installations will be autonomous. Two-way communications would allow system operation (and vehicle behavior) to be remotely monitored and collected. |
| Medford TOC and Redding ARTIC | Dynamic Warning VMS | sensor and surveillance control | Information used to configure and control sensor and surveillance systems at the roadside. | Real-time measures of road surface conditions can be measured and factored into the safe speed warnings that are displayed. These sensors will be deployed as part of the project. |
| Medford TOC and Redding ARTIC | Information Kiosk | traveler information | Traveler information comprised of traffic status, advisories, incidents, payment information and many other travel-related data updates and confirmations. | Included in the initial project architecture. Camera images may be provided to the kiosks based on the project description. This information stream could include a variety of visitor and traveler information relevant to I-5 travelers. |
| Medford TOC and Redding ARTIC | RWIS Sensor Stations | sensor and surveillance control | Information used to configure and control sensor and surveillance systems at the roadside. | Included in the initial project architecture. Two-way communications would allow the sensor stations to be remotely controlled. |
| Motorist-Aide Call Boxes | Medford TOC | incident information* | Manual reports from travelers using roadside telephones, describing vehicle breakdowns, incidents, or other factors that may severely disrupt traffic flow. | Call boxes can provide important early notification of situations which may cause severe traffic disruptions. They provide an important supplement for motorists who do not have access to wireless communications. |
| Pavement & Weather Conditions | Automated Anti-Icing, Dynamic Warning VMS, Automated Visibility Warning | weather conditions | Collected weather condition data from sensors. | This data flow differs from that transmitted by RWIS sensors because it would consist only a couple of measurements, specific to the needs of that site. It also would need to be measured continuously, and not in ten-minute polling intervals as is typical of RWIS sensors. |
| RWIS Sensor Stations | Medford TOC and Redding ARTIC | environmental conditions | Current environment conditions (e.g., air temperature, wind speed, surface temperature) as measured by environmental sensors and communicated by supporting field equipment. | Included in the initial project architecture. |

Western Transportation Institute89

* - Customized Architecture Flow

Table 5-1: Regional Architecture Flows Related to Early Winner Project (cont.).

| Source | Destination | Flow | Definition | COATS Application |
| --- | --- | --- | --- | --- |
| Variable Message Sign | Medford TOC and Redding ARTIC | roadway information system status | Current operating status of dynamic message signs, highway advisory radios, beacon systems, or other configurable field equipment that provides dynamic information to the driver. | Included in the initial project architecture. This variable message sign is an addition to an existing system, which will influence standards decisions for this interface. |
| Various devices | Medford TOC and Redding ARTIC | fault reports | Reports from field equipment (sensors, signals, signs, controllers, etc.) which indicate current operational status. | This capability ensures that maintenance staff are made aware of malfunctioning equipment in a timely fashion. |
| Redding ARTIC and Medford TOC | Medford TOC and Redding ARTIC | traffic information coordination | Traffic information exchanged between ARTIC/TOCs. Normally would include incidents, congestion data, traffic data, signal timing plans, and real-time signal control information. | This flow would allow coordination of data that is collected from RWIS sensors and traffic detectors and traveler information that is being displayed on the variable message signs that are installed on both sides of the state boundary at Siskiyou pass. This flow supports coordination of this information between the two state DOT management centers that operate this equipment. In the future, this interface could be expanded to also support control coordination so that either TOC/ARTIC could control the Siskiyou Pass field equipment within some mutually agreed operating conventions. |

* - Customized Architecture Flow

90Western Transportation Institute

Table 5-2: ITS Standards Relevant to Early Winner ProjectTable 5-2: ITS Standards Relevant to Early Winner Project.

| Flow | SDO | Standard Title | Doc ID | Protocol |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | C | D | M |
| environmental conditions | AASHTO | NTCIP - Base Standard: Octet Encoding Rules (OER) | 1102 | C |  |  |
| environmental conditions | AASHTO | NTCIP - Class B Profile | 2001 | C |  |  |
| environmental conditions | AASHTO | NTCIP - Point to Multi-Point Protocol Using RS-232 Subnetwork Profile | 2101 | C |  |  |
| environmental conditions | AASHTO | NTCIP - Internet (TCP/IP and UDP/IP) Transport Profile | 2202 | C |  |  |
| environmental conditions | AASHTO | NTCIP - Application Profile for Simple Transportation Management Framework (STMF) | 2301 | C |  |  |
| environmental conditions | AASHTO | NTCIP - Application Profile for Trivial File Transfer Protocol | 2302 | C |  |  |
| environmental conditions | AASHTO | NTCIP - Application Profile for File Transfer Protocol (FTP) | 2303 | C |  |  |
| environmental conditions | AASHTO | NTCIP - Simple Transportation Management Framework (STMF) | 1101 |  | D | M |
| environmental conditions | AASHTO | NTCIP - Simple Transportation Management Protocol (STMP) | 1103 |  | D | M |
| environmental conditions | AASHTO | NTCIP - Global Object Definitions | 1201 |  | D | M |
| environmental conditions | AASHTO | NTCIP - Object Definitions for Environmental Sensor Stations & Roadside Weather Information System | 1204 |  | D | M |
| environmental conditions | AASHTO | NTCIP - Subnetwork Profile for Point-to-Point Protocol using RS 232 | 2103 |  | D | M |
| environmental conditions | AASHTO | NTCIP - Subnetwork Profile for Ethernet | 2104 |  | D | M |
| environmental conditions | AASHTO | NTCIP - TP-Null Transport Profile | 2201 |  | D | M |
| roadway information system data | AASHTO | NTCIP - Base Standard: Octet Encoding Rules (OER) | 1102 | C |  |  |
| roadway information system data | AASHTO | NTCIP - Class B Profile | 2001 | C |  |  |
| roadway information system data | AASHTO | NTCIP - Point to Multi-Point Protocol Using RS-232 Subnetwork Profile | 2101 | C |  |  |
| roadway information system data | AASHTO | NTCIP - Internet (TCP/IP and UDP/IP) Transport Profile | 2202 | C |  |  |
| roadway information system data | AASHTO | NTCIP - Application Profile for Simple Transportation Management Framework (STMF) | 2301 | C |  |  |
| roadway information system data | AASHTO | NTCIP - Application Profile for Trivial File Transfer Protocol | 2302 | C |  |  |
| roadway information system data | AASHTO | NTCIP - Application Profile for File Transfer Protocol (FTP) | 2303 | C |  |  |

Legend: SDO = Standards Development Organization; Doc ID = Document Identification Number; C = Communications Protocol; D = Data Dictionary Protocol; M = Message Set Protocol.

Table 5-2: ITS Standards Relevant to Early Winner Project (cont.).

| Flow | SDO | Standard Title | Doc ID | Protocol |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | C | D | M |
| roadway information system data | AASHTO | NTCIP - Simple Transportation Management Framework (STMF) | 1101 |  | D | M |
| roadway information system data | AASHTO | NTCIP - Simple Transportation Management Protocol (STMP) | 1103 |  | D | M |
| roadway information system data | AASHTO | NTCIP - Global Object Definitions | 1201 |  | D | M |
| roadway information system data | AASHTO | NTCIP - Object Definitions for Dynamic Message Signs | 1203 |  | D | M |
| roadway information system data | AASHTO | NTCIP - Data Dictionary for Closed Circuit Television (CCTV) | 1205 |  | D | M |
| roadway information system data | AASHTO | NTCIP - Object Definitions for Video Switches | 1208 |  | D | M |
| roadway information system data | AASHTO | NTCIP - Subnetwork Profile for Point-to-Point Protocol using RS 232 | 2103 |  | D | M |
| roadway information system data | AASHTO | NTCIP - Subnetwork Profile for Ethernet | 2104 |  | D | M |
| roadway information system data | AASHTO | NTCIP - TP-Null Transport Profile | 2201 |  | D | M |
| roadway information system status | AASHTO | NTCIP - Base Standard: Octet Encoding Rules (OER) | 1102 | C |  |  |
| roadway information system status | AASHTO | NTCIP – Class B Profile | 2001 | C |  |  |
| roadway information system status | AASHTO | NTCIP – Point to Multi-Point Protocol Using RS-232 Subnetwork Profile | 2101 | C |  |  |
| roadway information system status | AASHTO | NTCIP – Internet (TCP/IP and UDP/IP) Transport Profile | 2202 | C |  |  |
| roadway information system status | AASHTO | NTCIP – Application Profile for Simple Transportation Management Framework (STMF) | 2301 | C |  |  |
| roadway information system status | AASHTO | NTCIP – Application Profile for Trivial File Transfer Protocol | 2302 | C |  |  |
| roadway information system status | AASHTO | NTCIP – Application Profile for File Transfer Protocol (FTP) | 2303 | C |  |  |
| roadway information system status | AASHTO | NTCIP – Simple Transportation Management Framework (STMF) | 1101 |  | D | M |
| roadway information system status | AASHTO | NTCIP – Simple Transportation Management Protocol (STMP) | 1103 |  | D | M |
| roadway information system status | AASHTO | NTCIP – Global Object Definitions | 1201 |  | D | M |
| roadway information system status | AASHTO | NTCIP – Object Definitions for Dynamic Message Signs | 1203 |  | D | M |

Legend: SDO = Standards Development Organization; Doc ID = Document Identification Number; C = Communications Protocol; D = Data Dictionary Protocol; M = Message Set Protocol.

Table 5-2: ITS Standards Relevant to Early Winner Project (cont.).

| Flow | SDO | Standard Title | Doc ID | Protocol |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | C | D | M |
| roadway information system status | AASHTO | NTCIP – Subnetwork Profile for Point-to-Point Protocol using RS 232 | 2103 |  | D | M |
| roadway information system status | AASHTO | NTCIP – Subnetwork Profile for Ethernet | 2104 |  | D | M |
| roadway information system status | AASHTO | NTCIP – TP-Null Transport Profile | 2201 |  | D | M |
| sensor and surveillance control | AASHTO | NTCIP – Base Standard: Octet Encoding Rules (OER) | 1102 | C |  |  |
| sensor and surveillance control | AASHTO | NTCIP – Class B Profile | 2001 | C |  |  |
| sensor and surveillance control | AASHTO | NTCIP – Point to Multi-Point Protocol Using RS-232 Subnetwork Profile | 2101 | C |  |  |
| sensor and surveillance control | AASHTO | NTCIP – Internet (TCP/IP and UDP/IP) Transport Profile | 2202 | C |  |  |
| sensor and surveillance control | AASHTO | NTCIP – Application Profile for Simple Transportation Management Framework (STMF) | 2301 | C |  |  |
| sensor and surveillance control | AASHTO | NTCIP – Application Profile for Trivial File Transfer Protocol | 2302 | C |  |  |
| sensor and surveillance control | AASHTO | NTCIP – Application Profile for File Transfer Protocol (FTP) | 2303 | C |  |  |
| sensor and surveillance control | AASHTO | NTCIP – Simple Transportation Management Framework (STMF) | 1101 |  | D | M |
| sensor and surveillance control | AASHTO | NTCIP – Simple Transportation Management Protocol (STMP) | 1103 |  | D | M |
| sensor and surveillance control | AASHTO | NTCIP – Global Object Definitions | 1201 |  | D | M |
| sensor and surveillance control | AASHTO | NTCIP – Data Collection & Monitoring Devices | 1206 |  | D | M |
| sensor and surveillance control | AASHTO | NTCIP – Transportation System Sensor Objects | 1209 |  | D | M |
| sensor and surveillance control | AASHTO | NTCIP – Subnetwork Profile for Point-to-Point Protocol using RS 232 | 2103 |  | D | M |
| sensor and surveillance control | AASHTO | NTCIP – Subnetwork Profile for Ethernet | 2104 |  | D | M |
| sensor and surveillance control | AASHTO | NTCIP – TP-Null Transport Profile | 2201 |  | D | M |
| traffic images | AASHTO | NTCIP – Base Standard: Octet Encoding Rules (OER) | 1102 | C |  |  |
| traffic images | AASHTO | NTCIP – Internet (TCP/IP and UDP/IP) Transport Profile | 2202 | C |  |  |

Legend: SDO = Standards Development Organization; Doc ID = Document Identification Number; C = Communications Protocol; D = Data Dictionary Protocol; M = Message Set Protocol.

Table 5-2: ITS Standards Relevant to Early Winner Project (cont.).

| Flow | SDO | Standard Title | Doc ID | Protocol |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | C | D | M |
| traffic images | AASHTO | NTCIP – Application Profile for Simple Transportation Management Framework (STMF) | 2301 | C |  |  |
| traffic images | AASHTO | NTCIP – Application Profile for Trivial File Transfer Protocol | 2302 | C |  |  |
| traffic images | AASHTO | NTCIP – Application Profile for File Transfer Protocol (FTP) | 2303 | C |  |  |
| traffic images | AASHTO | NTCIP – Simple Transportation Management Framework (STMF) | 1101 |  | D | M |
| traffic images | AASHTO | NTCIP – Simple Transportation Management Protocol (STMP) | 1103 |  | D | M |
| traffic images | AASHTO | NTCIP – Global Object Definitions | 1201 |  | D | M |
| traffic images | AASHTO | NTCIP – Data Dictionary for Closed Circuit Television (CCTV) | 1205 |  | D | M |
| traffic images | AASHTO | NTCIP – Object Definitions for Video Switches | 1208 |  | D | M |
| traffic images | AASHTO | NTCIP – Subnetwork Profile for Ethernet | 2104 |  | D | M |
| traffic images | AASHTO | NTCIP – TP-Null Transport Profile | 2201 |  | D | M |
| traveler information | SAE | Standard for ATIS Message Sets Delivered Over Bandwidth Restricted Media | J2369 | C | D | M |
| traveler information | SAE | ISP-Vehicle Location Referencing Message Profiles | J1746 |  | D |  |
| traveler information | SAE | Advanced Traveler Information System (ATIS) Data Dictionary | J2353 |  | D |  |
| traveler information | SAE | Advanced Traveler Information System (ATIS) Message Set | J2354 |  |  | M |
| traveler information for media | AASHTO | NTCIP – Base Standard: Octet Encoding Rules (OER) | 1102 | C |  |  |
| traveler information for media | AASHTO | NTCIP – Internet (TCP/IP and UDP/IP) Transport Profile | 2202 | C |  |  |
| traveler information for media | AASHTO | NTCIP – Application Profile for Trivial File Transfer Protocol | 2302 | C |  |  |
| traveler information for media | AASHTO | NTCIP – Application Profile for File Transfer Protocol (FTP) | 2303 | C |  |  |
| traveler information for media | AASHTO | NTCIP – Applications Profile for Data Exchange ASN.1 (DATEX) | 2304 | C |  |  |
| traveler information for media | AASHTO | NTCIP – Applications Profile for Common Object Request Broker Architecture (CORBA) | 2305 | C |  |  |
| traveler information for media | AASHTO | NTCIP – Subnetwork Profile for Ethernet | 2104 |  | D | M |

Legend: SDO = Standards Development Organization; Doc ID = Document Identification Number; C = Communications Protocol; D = Data Dictionary Protocol; M = Message Set Protocol.

Table 5-2: ITS Standards Relevant to Early Winner Project (cont.).

| Flow | SDO | Standard Title | Doc ID | Protocol |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | C | D | M |
| traveler information for media | SAE | Advanced Traveler Information System (ATIS) Data Dictionary | J2353 |  | D |  |
| traveler information for media | SAE | Advanced Traveler Information System (ATIS) Message Set | J2354 |  |  | M |
| traveler request | SAE | Advanced Traveler Information System (ATIS) Data Dictionary | J2353 |  | D |  |
| traveler request | SAE | Advanced Traveler Information System (ATIS) Message Set | J2354 |  |  | M |
| traffic information coordination | AASHTO | NTCIP – Base Standard: Octet Encoding Rules (OER) | 1102 | C |  |  |
| traffic information coordination | AASHTO | NTCIP – Internet (TCP/IP and UDP/IP) Transport Profile | 2202 | C |  |  |
| traffic information coordination | AASHTO | NTCIP – Application Profile for Trivial File Transfer Protocol | 2302 | C |  |  |
| traffic information coordination | AASHTO | NTCIP – Application Profile for File Transfer Protocol (FTP) | 2303 | C |  |  |
| traffic information coordination | AASHTO | NTCIP – Applications Profile for Data Exchange ASN.1 (DATEX) | 2304 | C |  |  |
| traffic information coordination | AASHTO | NTCIP – Applications Profile for Common Object Request Broker Architecture (CORBA) | 2305 | C |  |  |
| traffic information coordination | AASHTO | NTCIP – Subnetwork Profile for Ethernet | 2104 |  | D | M |
| traffic information coordination | ITE | Standard for Functional Level Traffic Management Data Dictionary (TMDD) | TM 1.03 |  | D |  |
| traffic information coordination | ITE | Message Set for External TMC Communication (MS/ETMCC) | TM 2.01 |  |  | M |

Legend: SDO = Standards Development Organization; Doc ID = Document Identification Number; C = Communications Protocol; D = Data Dictionary Protocol; M = Message Set Protocol.

This page is intentionally blank

# Next Steps

This report has provided an overview of the process of developing the COATS Regional Architecture, and has given some general information about the content of its associated Turbo Architecture database. The regional architecture has been defined at a very high level for the COATS region as a whole, and includes three subregional architectures and an early winner project architecture. As this effort satisfies TEA-21 requirements of conformity to the National ITS Architecture, it provides an important input into the development of the COATS ITS Strategic Deployment Plan.

It should be understood, however, that this initial ITS architecture development is only the first step in the sustained planning and integration effort. The first steps in the process involved identification and definition of the regional stakeholders and systems as well as the regional needs and challenges. The architecture development phase has involved the identification and definition of the system interconnects and information flows.

The development of an ITS architecture is, by its very nature, very dynamic, as shown in Figure 6-1. The architecture is a blueprint for project development and, consequently, will need regular review and updating to insure its validity. Therefore, the next step in the process is the validation process. The validation process occurs at three levels.

Figure 6-1: Development and Refinement of a Regional ITS ArchitectureFigure 6-1: Development and Refinement of a Regional ITS Architecture.

- Stakeholder and system level. Each stakeholder and associated organization needs to identify and validate their systems, the interfaces and information flows. This step is best done by the stakeholders themselves, since they are most familiar with their own systems as well as the needs and challenges of their individual organization and related systems.

- Subregional level. The validation of the subregional ITS architecture includes the identification of the subregional systems, interfaces and information flows. It is suggested that workshops be held with subregional stakeholders as part of the validation effort for subregional architectures.

- COATS regional level. It is also recommended that a regional workshop be held for validation of the COATS Regional Architecture.

Each of these validation activities is beyond the scope of the strategic deployment plan of which this ITS architecture is a component; however, they would be quite valuable in making the architecture more useful to stakeholders. To facilitate this validation, the COATS Regional Architecture is maintained as a “living” database, available on the Internet from http://www.coe.montana.edu/wti. The database can be adapted in the future as regional requirements evolve.

The initial ITS architecture for the Early Winner Project on the Siskiyou Pass shows that there is a need to improve communication and information exchange across the Oregon –California State line. For motorists on Interstate 5, the state line is almost invisible, with the exception of the respective states’ welcome signs. The early winner ITS architecture indicates that the ODOT equipment communicates with the Medford TOC and the Caltrans equipment communicates with the Redding ARTIC. However, the Redding and Medford centers do not have any advanced communication systems between each other at this time. The development of a seamless communication and information exchange between the Medford and Redding centers will be necessary in order to develop a truly regional transportation system, which can more adequately address the region’s transportation needs and challenges.

The regional architecture presented in this report may provide significant benefits to stakeholders throughout the region. It may help to leverage resources for ITS projects in the COATS study area. It highlights new opportunities for partnership in order to utilize scarce resources more efficiently, promotes interoperability and integration, and provides guidance for defining new projects. However, just as the ongoing success of the COATS project hinges on the development and pursuit of working relationships with new stakeholders, so the success of this regional architecture depends upon the ongoing input from stakeholders to maximize its potential. Consequently, validation efforts should be undertaken by COATS stakeholders in order to help address the transportation challenges of the COATS region in a truly regional way.

# References

1.	California / Oregon Advanced Transportation Systems Project Regional Stakeholder Partnership Business Plan Working Paper. Western Transportation Institute, Bozeman [MT]: June 1998.

2.	California / Oregon Advanced Transportation Systems (COATS) Stakeholder Outreach Workshop Summary. Western Transportation Institute, Bozeman [MT]: March 1999.

3.	Cuelho, Eli and Amy Gill, COATS Technical Memorandum One: Volume One, Review of Legacy Systems. Western Transportation Institute, Bozeman [MT]: November 1998.

4.	McGowen, Pat and Alyssa Reynolds, COATS Technical Memorandum One: Volume Two, Conditions and Performance. Western Transportation Institute, Bozeman [MT]: November 1998.

5.	Harry, Scott R., Randy W. Carroll and John M. Mounce, Northern California / Southern Oregon Rural Intelligent Transportation Systems (ITS) Areawide Travel and Safety Improvement Project: Traveler Needs Survey (Volume I). Western Transportation Institute, Bozeman [MT]: September 1998.

6.	Harry, Scott R., Randy W. Carroll and John M. Mounce, Northern California / Southern Oregon Rural Intelligent Transportation Systems (ITS) Areawide Travel and Safety Improvement Project: Traveler Needs Survey Significance Testing (Volume II). Western Transportation Institute, Bozeman [MT]: August 1998.

7.	California / Oregon Advanced Transportation Systems: Project Infrastructure. Western Transportation Institute, Bozeman [MT]: March 1999.

8.	California / Oregon Advanced Transportation Systems: Candidate Early-Winner Projects. Western Transportation Institute, Bozeman [MT]: April 1999.

9.	California Department of Transportation Web site, http://svhqsgi4.dot.ca.gov/hq/traffops/saferesr/trafdata/1998all/r005-6i.htm. Accessed June 22, 2000.

10.	“Siskiyou Pass Traveler Safety and Incident Management: Evaluation Plan.” Western Transportation Institute, Bozeman [MT]: January 2000.

This page is intentionally blank

# Appendix A  Communications and Information Survey

See following pages.

california – oregon advanced transportation systems Project

Questionnaire

Name

Title

Agency/Affiliation

Purpose:	To identify your agency’s priority transportation-related information needs that may be improved through transportation system improvements being developed under the COATS program. The survey will help determine how those improvements will be most compatible with the ways your agency receives and transmits information, both currently and in the near future.

Survey Content:

1A: Transportation-related information CURRENTLY RECEIVED by your organization (either from other organizations or from remote technologies).

1B: Transportation-related information that your organization would LIKE TO RECEIVE in the future (either from other organizations or from remote technologies).

2A: Transportation-related information CURRENTLY PROVIDED by your organization.

2B: Transportation-related information that your organization would LIKE TO PROVIDE in the future.

Instructions:

- Review the purpose of the survey.

- Provide organizations name AND/OR technology types in t1he areas provided in the table. If more than three lines are needed for any category, use the space labeled “Other”; specifying the type of information in the blank provided.

- Match the organization OR technology to a communication method using the symbols provided in the legend at the bottom of each page (see example provided on page 2 of the survey).

- To ensure your transportation-related needs are incorporated in the results, send the completed survey to the Western Transportation Institute, MSU-Bozeman, using the contact information provided at the end of the survey by Monday, November 1, 1999.

Section 1A: Information CURRENTLY RECEIVED by Your Agency/Organization

| Information Types | ##### From what ORGANIZATIONS AND/OR TECHNOLOGIES are you CURRENTLY RECEIVING these types of information? | ##### Associated Communication Methods (A-H) ##### See table below |
| --- | --- | --- |
| EXAMPLE | 1. XYZ Organization 2. Remote Weather Station 3. On-site personnel | 1. B, D 2. A 3. C |
| Weather Conditions | 1. 2. 3. | 1. 2. 3. |
| Road Conditions | 1. 2. 3. | 1. 2. 3. |
| Unusual Conditions (e.g., extreme weather, emergency travel only, etc.) | 1. 2. 3. | 1. 2. 3. |
| Crashes and Breakdowns | 1. 2. 3. | 1. 2. 3. |
| Maintenance Needs (e.g., sand, carcass removal, etc.) | 1. 2. 3. | 1. 2. 3. |
| Construction Activities | 1. 2. 3. | 1. 2. 3. |
| Other: (please specify) _________________________ | 1. 2. 3. | 1. 2. 3. |
| Other: (please specify) _________________________ | 1. 2. 3. | 1. 2. 3. |

| Symbol | Method of Communication |
| --- | --- |
| A | Telephone |
| B | Fax |
| C | Field radio |
| D | Email |
| E | Internet |
| F | Standard mail (USPS) |
| G | Television, Radio |
| H | Other (please specify) |

Section 1B: Information Your Agency would LIKE TO RECEIVE

| ##### Information Types | From what organizations AND/OR TECHNOLOGIES would you LIKE TO receive these types of information? | Associated Communication Methods (A-H) See table below |
| --- | --- | --- |
| Weather Conditions | 1. 2. 3. | 1. 2. 3. |
| Road Conditions | 1. 2. 3. | 1. 2. 3. |
| Unusual Conditions (e.g., extreme weather, emergency travel only, etc.) | 1. 2. 3. | 1. 2. 3. |
| Crashes and Breakdowns | 1. 2. 3. | 1. 2. 3. |
| Maintenance Needs (e.g., sand, carcass removal, etc.) | 1. 2. 3. | 1. 2. 3. |
| Construction Activities | 1. 2. 3. | 1. 2. 3. |
| Other: (please specify) _________________________ | 1. 2. 3. | 1. 2. 3. |
| Other: (please specify) _________________________ | 1. 2. 3. | 1. 2. 3. |

| Symbol | Method of Communication |
| --- | --- |
| A | Telephone |
| B | Fax |
| C | Field radio |
| D | Email |
| E | Internet |
| F | Standard mail (USPS) |
| G | Television, Radio |
| H | Other (please specify) |

Section 2A: Information CURRENTLY PROVIDED by Your Agency/Organization

| ##### Information Types | To what organizations do you PROVIDE these types of information? | Associated Communication Methods (A-H) See table below |
| --- | --- | --- |
| Weather Conditions | 1. 2. 3. | 1. 2. 3. |
| Road Conditions | 1. 2. 3. | 1. 2. 3. |
| Unusual Conditions (e.g., extreme weather, emergency travel only, etc.) | 1. 2. 3. | 1. 2. 3. |
| Crashes and Breakdowns | 1. 2. 3. | 1. 2. 3. |
| Maintenance Needs (e.g., sand, carcass removal, etc.) | 1. 2. 3. | 1. 2. 3. |
| Construction Activities | 1. 2. 3. | 1. 2. 3. |
| Other: (please specify) _________________________ | 1. 2. 3. | 1. 2. 3. |
| Other: (please specify) _________________________ | 1. 2. 3. | 1. 2. 3. |

| Symbol | Method of Communication |
| --- | --- |
| A | Telephone |
| B | Fax |
| C | Field radio |
| D | Email |
| E | Internet |
| F | Standard mail (USPS) |
| G | Television, Radio |
| H | Other (please specify) |

Section 2B: Information Your Organization would LIKE TO PROVIDE

| ##### Information Types | To what organizations would you LIKE TO PROVIDE these types of information? | Associated Communication Methods (A-H) See table below |
| --- | --- | --- |
| Weather Conditions | 1. 2. 3. | 1. 2. 3. |
| Road Conditions | 1. 2. 3. | 1. 2. 3. |
| Unusual Conditions (e.g., extreme weather, emergency travel only, etc.) | 1. 2. 3. | 1. 2. 3. |
| Crashes and Breakdowns | 1. 2. 3. | 1. 2. 3. |
| Maintenance Needs (e.g., sand, carcass removal, etc.) | 1. 2. 3. | 1. 2. 3. |
| Construction Activities | 1. 2. 3. | 1. 2. 3. |
| Other: (please specify) _________________________ | 1. 2. 3. | 1. 2. 3. |
| Other: (please specify) _________________________ | 1. 2. 3. | 1. 2. 3. |

| Symbol | Method of Communication |
| --- | --- |
| A | Telephone |
| B | Fax |
| C | Field radio |
| D | Email |
| E | Internet |
| F | Standard mail (USPS) |
| G | Television, Radio |
| H | Other (please specify) |

Other Information

1. What specific pieces of transportation related information would help you do your job more effectively?

1.a. Which agencies could possibly provide this information?

1.b. What plans do you have to receive this information?

1.c. What would be the communication mode?

2. What major projects do you have coming up that could benefit from some amount of collaboration with any of these agencies listed above?

3. Are you satisfied with the timeliness and regularity with which you currently receive information related to:

 unplanned conditions such as severe weather, crashes, breakdowns, closures?	___ Yes	___ No

 planned conditions such as roadway construction, events, closures and detours?	___ Yes	___ No

3.a. If no, what would you like to see changed?

4. Please provide any additional comments:

Please send completed survey no later than Monday, November 1, 1999.

Via fax:	ATTN: Eli Cuelho
	(406) 994-1697

Via mail:	ATTN: Eli Cuelho

Western Transportation Institute

PO Box 173910

Montana State University – Bozeman

Bozeman, MT 59717-3910

##### Thank you for your input!

If you have questions, please contact Eli Cuelho at:

ph: (406) 994-7886 OR elic@coe.montana.edu

This page is intentionally blank

# Appendix B  Architecture Workshop Attendance

March 1, 2000 – Coos Bay, Oregon

Greg Aldrich, Coquille Economic Development Corporation

John Bennett, Region 3, Oregon Department of Transportation

Jim Burrfield, Curry County Transit

Jay Cable, Reedsport Emergency Management System Coordinator

Larry Close, Mill Casino

Steve Doty, Coos Bay Streets Division

Phil George, Oregon State Police

Sue Holmes, Coos County Transit

Chuck Larsen, Oregon Department of Transportation

Leo Lightle, City of Brookings

Dave McDaniel, Coos County Sheriff, Emergency Management

Galen McGill, ITS Manager, Oregon Department of Transportation

Robin Phillips, Transit, Oregon Department of Transportation

Nathaniel Price, Federal Highway Administration

March 22, 2000 – Klamath Falls, Oregon

Cindy Deas, Klamath/Lake/Modoc/ Siskiyou Outdoor Recreation Working Group

Bob Doran, Oregon Department of Transportation

Starla Fenner, Basin Transit Service

John R. Hawkins, Amtrak

Chuck Larsen, ITS Unit, Oregon Department of Transportation

Mark Macfarlane, USDA Forest Service

Galen McGill, ITS Manager, Oregon Department of Transportation

Jim Mediger, Oregon State Police

Ernest Palmer, Basin Transit Service

Stan Strickland, Klamath County Public Works

Bill Thompson, Klamath County Emergency Services

April 10, 2000 – Redding, California

Nancy Braga, Communications Supervisor, CHP Communications Center – Yreka

Charles Byrd, Sheriff, Shasta County Sheriff’s Department

Frank Cechini, Federal Highway Administration

Todd Chadd, Lieutenant, California Highway Patrol – Northern Division

Craig Dorman, Superintendent, Lava Beds National Monument

Jay Gunsauls, Deputy Fire Chief, Redding Fire Department

Scott Holmquist, Field Battalion Chief, California Department of Forestry

Tim Huckabay, Traffic Engineering and Operations, Caltrans District 2

David King, Assistant Chief, California Highway Patrol – Northern Division

Dan Kovacich, Executive Office, Shasta County RTPA

Martin Nicholas, Chief of Police, City of Weed

Craig Sharp, Director, City of Weed Department of Public Works

John Stokes, Transportation Planner, Shasta County RTPA

Steve Thede, Chief of Interpretation, Whiskeytown National Recreation Area

Myrlane Wederbrook, Transportation Engineering Technician, Caltrans District 2

Russ Wenham, Division Chief, Caltrans District 2

Scott White, Associate Planner II, Trinity County RTPA

April 11, 2000 – Eureka, California

Spencer Clifton, Executive Director, Humboldt County Association of Governments

Mia Edington, Caltrans District 1

Janet Jackson, Assistant Roads Director, Hoopa Valley Tribal Roads Department

Michael Lucas, Caltrans District 1

Carol McCall, Public Affairs & Community Relations, Redwood National & State Parks and Smith River National Recreation Area

Randy Mendosa, Lieutenant, Arcata Police Department

Dale Neiman, City Manager, City of Fortuna

Rod Sanderson, Superintendent of Distribution, Pacific Lumber Company

Monty Soulis, Communications Operator II, California Highway Patrol

Martin Van Zandt, Deputy District Director, Maintenance and Operations, Caltrans District 1

Cheryl Willis, Chief, Division Planning, Caltrans District 1

April 13, 2000 – Ukiah, California

Chris Brown, Air Quality Planner, Mendocino County Air Quality Management District

Lisa Davey, Assistant Planner, Mendocino Council of Governments

Phil Dow, Executive Director, Mendocino Council of Governments

Loretta Ellard, Deputy (Planning), Mendocino Council of Governments

Cathy McKeon, Senior Civil Engineer, City of Ukiah

Jim Mulheren, Representative Member, Ukiah Chamber of Commerce

Stanley Townsend, Director, Mendocino County Department of Transportation

Steve Turner, Assistant General Manager, Mendocino Transit Authority

April 24, 2000 – Medford, Oregon

Craig Ackerman, Superintendent, Oregon Caves National Monument

Michael Baker, Oregon Department of Transportation

Mathew Barnes, Rogue Valley Transit District

Ron Butros, Region 3 Traffic Manager, Oregon Department of Transportation

Jared Castle, Public Information Office, Oregon Department of Transportation

Sherrin Coleman, Rogue Valley Transportation District

Sandy Eccker, Jackson County Emergency Management

Robert Fynn, ITS Unit, Oregon Department of Transportation

Joe Hunkins, Southern Oregon Visitors Association

Cindy Johnson, Mobility Works

Dave Marshall, Douglas County Sheriff’s Office

Jim McClellan, Mayor, City of Winston

Galen McGill, ITS Manager, Oregon Department of Transportation

Norm Nelson, Douglas County Sheriff’s Office

Nathaniel Price, Federal Highway Administration

Gordon Renskers, Lieutenant, Oregon State Police

Mike Spaeth, Medford TOC Dispatch, Oregon Department of Transportation

Wayne Stinson, Douglas County Emergency Services

Chandra Swaminathan, Mobility Works

# Appendix C  Inventory of Stakeholders and Subsystems

Table C-1 lists all stakeholdrs and subsystems included in the COATS region. For descriptions on individual

112Western Transportation Institute

Table C-1: Inventory of Stakeholders and SystemsTable C-1: Inventory of Stakeholders and Systems.

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 1-800-427-ROAD | Existing |  |  |  | - Personal Information Access | This is a toll-free number run by Caltrans which provides current road conditions information. This may be superceded by 511 at a later date. |
| 1-800-977-ODOT | Existing |  |  |  | - Personal Information Access | This toll-free number provides current road conditions from all areas within Oregon by touchtone phone (outside of Oregon, this is available at 503-588-2941). The service is provided by ODOT. This may be superceded by 511 at a later date. |
| Airline Schedule Operations | Existing |  |  |  | - Multimodal Transportation Service Provider | Includes Ascot, Horizon, Skywest, United and other airlines offering regular scheduled passenger service. |
| Amtrak Schedule Operations | Existing |  |  |  | - Multimodal Transportation Service Provider | This is the operational entity that provides static and real-time Amtrak schedule information for the COATS architecture. |
| California Highway Information Web Page | Existing |  |  |  | - Information Service Provider - Other ISP - Personal Information Access | This interactive Web-based service (http://www.dot.ca.gov/hq/roadinfo) provides key road construction, detour and closure information based on user-specified route numbers. |
| California Weigh Stations | Existing |  |  |  | - Commercial Vehicle Check | These are locations where commercial vehicles may be weighed and inspected. |
| Caltrans District 1 Public Affairs | Existing |  |  |  | - Media - Public Information Office* | This office is responsible for communicating with local media all news regarding key Caltrans activities in District 1. It may be used to disseminate information about road closures, ongoing construction projects, and other activities that may affect highway travel. |
| Caltrans District 2 Public Affairs | Existing |  |  |  | - Media - Public Information Office* | This office is responsible for communicating with local media all news regarding key Caltrans activities in District 2. It may be used to disseminate information about road closures, ongoing construction projects, and other activities that may affect highway travel. |
| Caltrans Field Equip District 1 (Eureka) | Existing |  |  |  | - Roadway Subsystem | This includes the environmental sensors, variable message signs, HAR, signals, traffic detectors, and any other field equipment and instrumentation operated by Caltrans District 1. |
| Caltrans Field Equip District 2 (Redding) | Existing |  |  |  | - Roadway Subsystem | This includes the environmental sensors, variable message signs, HAR, signals, traffic detectors, and any other field equipment and instrumentation operated by Caltrans District 2. |
| Caltrans HQ Construction Program | Existing |  |  |  | - Construction and Maintenance | This program is responsible for implementing the state’s highway construction program. It will provide information to local districts about future construction projects as they are introduced. |

Western Transportation Institute113

Table C-1: Inventory of Stakeholders and Systems (cont.).

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Caltrans HQ Permits | Existing |  |  |  | - Permitting Office* | The Encroachment Permits department is a part of the office of development services within Caltrans’ Traffic Operations program. As the responsible department for protecting the public’s investment in the State highway system, the permits office reviews all requests from utility companies, developers, volunteers, nonprofit organizations, etc., desiring to conduct various activities within the right of way. Such activities could include for example: construction of highway improvements, driveway installation and maintenance, highway landscaping and graffiti removal, commercial filming, special events such as parades commemorating an event, e.g. Independence Day. |
| Caltrans HQ Traffic | Existing |  |  |  | - Archived Data Management Subsystem - Other Archives | This office is responsible for coordinating the state’s transportation management centers, facilitating data exchange and interoperability. It will also serve as a repository for field data collected at districts across the state. |
| Caltrans Maintenance District 1 (Eureka) | Existing |  |  |  | - Construction and Maintenance - Maintenance Dispatch Office* - Other Maintenance Dispatch Office* | Covers the resource management and dispatch systems operated by Caltrans District 1. |
| Caltrans Maintenance District 1 (Eureka) Vehicles | Existing |  |  |  | - Maintenance Vehicles* - Other Maintenance Vehicles* | These Caltrans vehicles are responsible for maintenance activities, including setting up road closures, assisting in incident removal, and performing winter maintenance. It is assumed that there is seamless communication between the vehicle and its driver, who will actually be directing and performing the maintenance activity. |
| Caltrans Maintenance District 2 (Redding) | Existing |  |  |  | - Construction and Maintenance - Maintenance Dispatch Office* - Other Maintenance Dispatch Office* | Covers the resource management and dispatch systems operated by Caltrans District 2. |
| Caltrans Maintenance District 2 (Redding) Vehicles | Existing |  |  |  | - Maintenance Vehicles* - Other Maintenance Vehicles* | These Caltrans vehicles are responsible for maintenance activities, including setting up road closures, assisting in incident removal, and performing winter maintenance. It is assumed that there is seamless communication between the vehicle and its driver, who will actually be directing and performing the maintenance activity. |
| Caltrans RWIS | Existing |  |  |  | - Weather Service | These are RWIS stations maintained by Caltrans for providing information on atmospheric and pavement temperature conditions at remote sites. Information from these sites is helpful for maintenance officials in making maintenance decisions. |

114Western Transportation Institute

Table C-1: Inventory of Stakeholders and Systems (cont.).

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Caltrans Weather Warning Systems | Existing |  |  |  | - Roadway Subsystem | These are driver advisory systems which are activated based on certain meteorological conditions or thresholds (e.g. high wind, icy bridge, etc.). They process information provided by a localized weather station and provide a warning to motorists when specified conditions arise. |
| Caltrans/CHP Eureka ARTIC | Existing |  |  |  | - Other TM - Traffic Management | This ARTIC (Advanced Rural Technology Integration Center) plays a central role in the COATS architecture for managing and monitoring the transportation system in Del Norte, Humboldt, Mendocino and Lake Counties. It is a Tier 3 Satellite Operation Center connected to the Oakland Regional TMC. |
| Caltrans/CHP Oakland Regional TMC | Existing |  |  |  | - Other TM | This Tier 1 TMC is tied to Tier 3 Satellite Operation Centers in Eureka and San Luis Obispo. |
| Caltrans/CHP Redding ARTIC | Existing |  |  |  | - Other TM - Traffic Management | This ARTIC (Advanced Rural Technology Integration Center) coordinates transportation operations for north central California, serving Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama, and Trinity Counties. It is a Tier 3 Satellite Operation Center which is connected to the Sacramento TMC. |
| Caltrans/CHP Sacramento Regional TMC | Existing |  |  |  | - Other TM | This Tier 1 TMC is tied to Tier 2 (Urban) TMCs in Stockton and Fresno and Tier 3 Satellite Operation Centers in Redding and Kingvale |
| CHIN | Existing |  |  |  | - Information Service Provider - Other ISP | The California Highway Information Network provides daily adverse travel conditions information. Information is made available to telephone and Internet. |
| CHP Eureka and Humboldt Division Dispatch and 911 Centers | Existing |  |  |  | - Emergency Management - Other EM | This includes offices in Crescent City, Humboldt, Garberville, Ukiah and Clearlake. |
| CHP Northern District Dispatch and 911 Centers | Existing |  |  |  | - Emergency Management - Other EM | This placeholder includes the Redding, Susanville and Yreka dispatch centers. These centers are responsible for CHP offices in Alturas, Cottonwood, Mt. Shasta, Quincy, Redding, Red Bluff, Susanville, Trinity River, and Yreka, as well as Dunsmuir Grade Inspection Facility. |
| CHP Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | California Highway Patrol vehicles. |
| COATS Kiosks | Planned |  |  |  | - Remote Traveler Support | This is a planned system which would provide real-time traveler and tourism information to travelers throughout the COATS region. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

Western Transportation Institute115

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Commercial Vehicle (Onboard) Systems | Existing |  |  |  | - Commercial Vehicle Subsystem | This subsystem resides in a commercial vehicle and provides the sensory, processing, storage, and communications functions necessary to support safe and efficient commercial vehicle operations. The Commercial Vehicle Subsystem provides two-way communications between the commercial vehicle drivers, their fleet managers, and roadside officials, and provides HAZMAT response teams with timely and accurate cargo contents information after a vehicle incident. |
| Commercial Vehicle Driver | Existing |  |  |  | - Commercial Vehicle Driver | This terminator represents the human entity that operates vehicles transporting goods including both long haul trucks and local pick up and delivery vans. |
| Commercial Vehicles | Existing |  |  |  | - Commercial Vehicle | The actual commercial vehicle along with the special aspects of large commercial vehicles and vehicles designed to carry cargo that extend beyond the characteristics defined for the Basic Vehicle. This terminator thus represents a special type of Basic Vehicle that is used to transport goods or services which are operated by professional drivers, typically administered as part of a larger fleet, and regulated by a Commercial Vehicle Manager. |
| Coos Bay-Area City Traffic Control Equip | Existing |  |  |  | - Roadway Subsystem | This includes traffic equipment maintained by the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, and Roseburg. |
| Coos Bay-Area City Traffic Control Systems | Existing |  |  |  | - Other TM - Traffic Management | This includes traffic management activities for the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, Brookings, Port Orford, Gold Beach, Bandon, Myrtle Point, Elkton, Sutherland, Myrtle Creek, and Wells Creek. |
| Coos Bay-Area County Maintenance Systems | Existing |  |  |  | - Construction and Maintenance | This includes Coos County, Curry County, Douglas County and Lane County Maintenance Department systems. |
| Coos Bay-Area E-911 Comm Centers | Existing |  |  |  | - Emergency Management - Other EM | This includes the two communications centers each in Coos and Curry Counties. |
| Coos Bay-Area EMS Provider Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes the broad range of private EMS, ambulance and air evacuation providers that serve the Coos Bay area, including Bay Cities Ambulance, Coquille Valley Ambulance, Lower Umpqua EMS, Medic 4 Ambulance, Myrtle Point, Curry County EMS, and Del Norte. |
| Coos Bay-Area EMS Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes the broad range of private EMS, ambulance and air evacuation vehicles that serve the Coos Bay area, including Bay Cities Ambulance, Coquille Valley Ambulance, Lower Umpqua EMS, Medic 4 Ambulance, Myrtle Point, Curry County EMS, and Del Norte. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

116Western Transportation Institute

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Coos Bay-Area Fire and Rescue Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes fire and rescue dispatch centers for Coos Bay, North Bend, Florence, Reedsport, Coquille, Hauser, East Bay, Millington, and Charleston. (The last four cities are volunteer.) |
| Coos Bay-Area Fire and Rescue Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes fire and rescue vehicles serving Coos Bay, North Bend, Florence, Reedsport, Coquille, Hauser, East Bay, Millington, and Charleston. (The last four cities are volunteer.) |
| Coos Bay-Area Paratransit Dispatch | Existing |  |  |  | - Other TRM - Transit Management | Includes Curry-Coos County Transit System, Senior Transportation. |
| Coos Bay-Area Paratransit Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | Includes Curry-Coos County Transit System, Senior Transportation. |
| Coos Bay-Area Planning Agencies | Existing |  |  |  | - Archived Data Management Subsystem - Other Archives | This includes various city and county planning agencies that may have use for archived data, including but not limited to the Umpqua Regional Council of Governments. |
| Coos Bay-Area Police and Sheriff Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes the police departments for the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, Brookings, Port Orford, Gold Beach, Bandon, Myrtle Point, Elkton, Sutherland, Myrtle Creek, and Wells Creek. This also includes county sheriffs from Coos, Curry and Douglas Counties, as well as tribal police. |
| Coos Bay-Area Police and Sheriff Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes law enforcement vehicles for the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, Brookings, Port Orford, Gold Beach, Bandon, Myrtle Point, Elkton, Sutherland, Myrtle Creek, and Wells Creek. This also includes county sheriffs from Coos, Curry and Douglas Counties, as well as tribal police. |
| Coos Bay-Area Radio Stations and Newspapers | Existing |  |  |  | - Media | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the radio and print media intended for human, rather than system, processing and interpretation. |
| Coos Bay-Area Towing/Recovery | Existing |  |  |  | - Other EM | This includes the private tow dispatch systems that clear incidents in the Coos Bay area. This includes numerous individual operators that serve in the region. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

Western Transportation Institute117

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Coos Bay-Area Traveler Information Resources | Existing |  |  |  | - Yellow Pages Service Providers | This includes Parks and Recreation Departments for the cities of Coos Bay, North Bend, Florence, Reedsport, Coquille, and Roseburg; the Bureau of Land Management; U.S. Forest Service; Oregon Parks & Recreation; Coos County Bikeway Committee; Coos County and Curry County Parks; Bandon Dunes Recreational Area; as well as area hotels, restaurants, gas stations, shopping, etc. |
| Coos Bay-Area Visitor Information Services | Existing |  |  |  | - Information Service Provider - Other ISP | This includes City Chambers of Commerce, Central Oregon Coast Association, Southern Oregon Visitors Association, Oregon Tourism Commission, Oregon Lodging Association, and Coos Bay-North Bend Tourism Promotion Bureau. |
| CVO Inspector | Existing |  |  |  | - CVO Inspector | This terminator represents the human entities who perform regulatory inspection of Commercial Vehicles in the field. CVO Inspectors support the roadside inspection, weighing, and checking of credentials either through automated preclearance or manual methods. |
| DEQ Medford Regional Office | Existing |  |  |  | - Emissions Management | DEQ is responsible for protecting and enhancing Oregon’s water and air quality, for cleaning up spills and releases of hazardous materials, and for managing the proper disposal of solid and hazardous wastes. DEQ operates a vehicle inspection program in the Rogue Valley to make sure cars are maintained properly to reduce emissions. |
| Downtown Parking District | Existing |  |  |  | - Parking Management | This district covers the downtown Klamath Falls area. |
| Drivers | Existing |  |  |  | - Driver | This terminator represents the human entity that operates a licensed vehicle on the roadway. Included are operators of private, Transit, Commercial, and Emergency vehicles where the data being sent or received is not particular to the type of vehicle. Thus this external originates driver requests and receives driver information that reflects the interactions which might be useful to all drivers, regardless of vehicle classification. |
| Eureka-Area Air Districts | Existing |  |  |  | - Emissions Management | This includes various districts responsible for air quality management, including the North Coast Unified AQMD (responsible for Del Norte, Humboldt and Trinity Counties), the Lake County AQMD and the Mendocino County AQMD. |
| Eureka-Area City Traffic Control Equip | Existing |  |  |  | - Roadway Subsystem | This includes the field equipment owned and operated by the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Lakeport, Point Arena, Rio Dell, Trinidad. Ukiah and Willits. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

118Western Transportation Institute

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Eureka-Area City Traffic Control Systems | Existing |  |  |  | - Other TM - Traffic Management | This includes the traffic management functions performed by the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Lakeport, Point Arena, Rio Dell, Trinidad. Ukiah and Willits. |
| Eureka-Area County Maintenance Systems | Existing |  |  |  | - Construction and Maintenance | This includes public works or transportation departments for Del Norte, Humboldt, Lake and Mendocino Counties. |
| Eureka-Area E-911 Comm Centers | Existing |  |  |  | - Emergency Management - Other EM | This includes the wireline public safety answering points on the northern California coast, including but not limited to a center in Curry County, Oregon which provides service to northern Del Norte County. |
| Eureka-Area EMS Provider Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes the broad range of private EMS, ambulance and air evacuation providers, including Arcata Mad River Ambulance Inc, City Ambulance of Eureka, City of Lakeport, Coast Guard, Del Norte Air Ambulance, Hoopa Ambulance, Medivac (CalSTAR), Mendocino Coast Hospital Ambulance, North Coast EMS, North Pacific Emergency Services, Redwood Empire Life Support, and Ukiah Ambulance. |
| Eureka-Area EMS Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes the broad range of private EMS, ambulance and air evacuation vehicles, including those operated by Arcata Mad River Ambulance Inc, City Ambulance of Eureka, City of Lakeport, Coast Guard, Del Norte Air Ambulance, Hoopa Ambulance, Medivac (CalSTAR), Mendocino Coast Hospital Ambulance, North Coast EMS, North Pacific Emergency Services, Redwood Empire Life Support, and Ukiah Ambulance. |
| Eureka-Area Fire and Rescue Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes dispatch functions provided for Arcata Fire District, Blue Lake Fire District, Brooktrails Fire Dept , California Department of Forestry, Clearlake Fire District, Coast Guard (Clear Lake), Coast Guard (Fort Bragg), Crescent City Volunteer Fire Department, Eureka Fire District, Ferndale Fire District, Fort Bragg Fire Department, Fortuna Fire District, Garberville Rescue, Lakeport Fire Dept, Redwood Valley Fire District, Samoa Peninsula Fire District, South Trinity Area Rescue, Ukiah Fire Dept, Ukiah Valley Fire District, Upper Lake Fire District, and Willits Fire Department. This also includes California Fish & Game, National Park Service, U.S. Fish & Wildlife and U.S. Forest Service. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

Western Transportation Institute119

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Eureka-Area Fire and Rescue Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes vehicles owned by Arcata Fire District, Blue Lake Fire District, Brooktrails Fire Dept , California Department of Forestry, Clearlake Fire District, Coast Guard (Clear Lake), Coast Guard (Fort Bragg), Crescent City Volunteer Fire Department, Eureka Fire District, Ferndale Fire District, Fort Bragg Fire Department, Fortuna Fire District, Garberville Rescue, Lakeport Fire Dept, Redwood Valley Fire District, Samoa Peninsula Fire District, South Trinity Area Rescue, Ukiah Fire Dept, Ukiah Valley Fire District, Upper Lake Fire District, and Willits Fire Department. This also includes California Fish & Game, National Park Service, U.S. Fish & Wildlife and U.S. Forest Service. |
| Eureka-Area Fixed Route Dispatch | Existing |  |  |  | - Other TRM - Transit Management | This includes dispatch and management operations for the following transit systems: Arcata-Mad River Transit System, which serves Arcata and Eureka; Eureka Transit Service (run by Humboldt Transit Authority), which provides service to Eureka; Mendocino Transit Authority, which connects cities in Mendocino County; Redwood Coast Transit, which serves Crescent City; and Redwood Transit System (Humboldt Transit Authority), which goes throughout Humboldt County. |
| Eureka-Area Fixed Route Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | This includes transit vehicles for the following transit systems: Arcata-Mad River Transit System, which serves Arcata and Eureka; Eureka Transit Service (run by Humboldt Transit Authority), which provides service to Eureka; Mendocino Transit Authority, which connects cities in Mendocino County; Redwood Coast Transit, which serves Crescent City; and Redwood Transit System (Humboldt Transit Authority), which goes throughout Humboldt County. |
| Eureka-Area Municipal Planning Agencies | Existing |  |  |  | - Archived Data Management Subsystem - Other Archives | This includes planning divisions for the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Hoopa, Lakeport, Point Arena, Rio Dell, Trinidad, Ukiah, and Willits. |
| Eureka-Area Paratransit Dispatch | Existing |  |  |  | - Other TRM - Transit Management | This includes various paratransit dispatch operations, including Coastal Cab, Crescent City Paratransit Services, Door to Door Airporter, Fortuna Senior Bus, Lake Transit Authority, Mendocino Limousine, Mendocino Transit Authority, Redwood Dial A Ride, Yellow Cab. This also includes various senior centers (such as in Mendocino County) that each have paratransit services. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

120Western Transportation Institute

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Eureka-Area Paratransit Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | This includes transit vehicles to support paratransit operations, including Coastal Cab, Crescent City Paratransit Services, Door to Door Airporter, Fortuna Senior Bus, Lake Transit Authority, Mendocino Limousine, Mendocino Transit Authority, Redwood Dial A Ride, Yellow Cab. This also includes various senior centers (such as in Mendocino County) that each have paratransit services. |
| Eureka-Area Police and Sheriff Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes law enforcement agencies for the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Lakeport, Point Arena, Rio Dell, Trinidad, Ukiah and Willits, and for the Hoopa Tribe. It also includes county sheriff’s offices for Del Norte, Humboldt, Lake and Mendocino Counties. This also includes various other law enforcement agencies with specialized jurisdictions, such as California Department of Forestry, California Fish & Game, National Park Service, and the U.S. Forest Service. |
| Eureka-Area Police and Sheriff Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes vehicles used for law enforcement by the cities of Arcata, Blue Lake, Clearlake, Crescent City, Eureka, Ferndale, Fort Bragg, Fortuna, Lakeport, Point Arena, Rio Dell, Trinidad, Ukiah and Willits, and for the Hoopa Tribe. It also includes county sheriff’s vehicles for Del Norte, Humboldt, Lake and Mendocino Counties. This also includes vehicles associated with various other law enforcement agencies with specialized jurisdictions, such as California Department of Forestry, California Fish & Game, National Park Service, and the U.S. Forest Service. |
| Eureka-Area Regional Transportation Planning Agencies | Existing |  |  |  | - Archived Data Management Subsystem - Other Archives | This includes organizations responsible for transportation planning along the northern California coast, including the Del Norte Local Transportation Commission (LTC); the Humboldt County Association of Governments; the Lake Council of Governments; and the Mendocino Council of Governments. |
| Eureka-Area Towing/Recovery | Existing |  |  |  | - Other EM | This includes the private tow dispatch systems that clear incidents in the Eureka area. This includes numerous individual operators that serve in the region. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

Western Transportation Institute121

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Eureka-Area Traveler Information Resources | Existing |  |  |  | - Yellow Pages Service Providers | This includes Bureau of Land Management, California Parks & Recreation, Department of Fish and Game, Harbor Recreation and Conservation, National Forest Service, National Park Service, North Coast Redwoods, Redwood National and State Parks, Six Rivers (includes Smith River), various State Parks (including Anderson Marsh, Clear Lake, Jackson, Jughandle, Richardson Grove, Russian Gulch, Standish-Hickey). This also includes City (Clearlake, Ukiah) and County (Humboldt, Lake and Mendocino) Recreation Departments. This also includes area hotels, restaurants, gas stations, shopping, etc. |
| Eureka-Area Traveler Information Systems | Planned |  |  |  | - Information Service Provider - Other ISP | These systems collect traveler information from operational sources (e.g., local traffic and public safety agencies/systems), visitor information systems (e.g. chambers of commerce), and local travel destinations (e.g., Parks). This information is then made available to travelers through both broadcast and interactive digital information services through the Internet and other media. |
| Eureka-Area TV and Radio Stations | Existing |  |  |  | - Media | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the analog TV, radio and print media intended for human, rather than system, processing and interpretation. |
| Eureka-Area Visitor Information Services | Existing |  |  |  | - Information Service Provider - Other ISP | This includes chambers of commerce for Clearlake, Eureka, Hopland, Lakeport, Mendocino Coast/Fort Bragg, Ukiah, and Willits. Eureka Convention and Visitors Bureau, Mendocino County Convention and Visitors Bureau, Northern California Tourism, and Redwood Empire Association. |
| Green Light Admin System | Existing |  |  |  | - Commercial Vehicle Administration - Other CVAS | The Green Light program equips trucks with small pager-like devices called transponders. Transponders allow trucks to identify themselves while they’re weighed at high speed by sensors and scales in the highway. A truck equipped with a Green Light transponder can go past an Oregon weigh station at normal cruising speed, instead of pulling off the highway and lining up to weigh at static scales. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

122Western Transportation Institute

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| HELP | Existing |  |  |  | - Commercial Vehicle Administration - Other CVAS | HELP (Heavy Vehicle Electronic License Plate, Inc.), is a non-profit partnership between motor carriers and government agencies (including Caltrans) whose mission is to develop and deploy advanced technology systems to create a cooperative operating and regulatory environment which improves the efficient and safe movement of commercial vehicles and the performance of highway systems. |
| Highway Travel Conditions Reporting System | Existing |  |  |  | - Information Service Provider - Other ISP | This is an ODOT-run interface in which information about road closures, construction activities and traffic incidents can be entered. The database is accessible in a variety of formats, including a Web-based format (TripCheck) and an 800-number. |
| IFTA Clearinghouse | Existing |  |  |  | - Commercial Vehicle Administration - Other CVAS | This is a national clearinghouse designed to allocate fuel taxes between multiple states for motor carrier activities across jurisdictional lines, in accordance with the International Fuel Tax Agreement. |
| Inter-City Transit | Existing |  |  |  | - Other TRM - Transit Management | This includes dispatch systems for Amtrak ThruWay Bus, Coastline Enterprises, Greyhound, North Coast Railroad and Porter Stage Lines. |
| Inter-City Transit Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | This includes transit vehicles for Amtrak ThruWay Bus, Coastline Enterprises, Greyhound, and Porter Stage Lines. |
| IRP Clearinghouse | Existing |  |  |  | - Commercial Vehicle Administration - Other CVAS | This is a registration reciprocity agreement among jurisdictions in the United States and Canada which provides for payment of license fees on the basis of fleet miles operated in various jurisdictions |
| Klamath Falls-Area City Traffic Control Equip | Existing |  |  |  | - Roadway Subsystem | This includes traffic control equipment owned and operated by the cities of Klamath Falls and Lakeview. |
| Klamath Falls-Area City Traffic Control Systems | Existing |  |  |  | - Other TM - Traffic Management | This includes the traffic control systems operated by the cities of Klamath Falls and Lakeview. |
| Klamath Falls-Area County Maintenance | Existing |  |  |  | - Construction and Maintenance | This includes the maintenance systems (road maintenance, snow removal, etc.) operated in Klamath and Lake Counties. |
| Klamath Falls-Area E-911 Comm Centers | Existing |  |  |  | - Emergency Management - Other EM | This represents the public safety answering point for Klamath County that receives 911 calls. |
| Klamath Falls-Area EMS Provider Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes the broad range of private EMS, ambulance and air evacuation providers that serve the Klamath Falls area, including Klamath County Emergency Services. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

Western Transportation Institute123

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Klamath Falls-Area EMS Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes the broad range of private EMS, ambulance and air evacuation vehicles that serve the Klamath Falls area, including Klamath County Emergency Services. |
| Klamath Falls-Area Fire Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes Klamath County, Lake County and Keno Rural Fire Districts. |
| Klamath Falls-Area Fire Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes Klamath County, Lake County and Keno Rural Fire Districts. |
| Klamath Falls-Area Fixed Route Dispatch | Existing |  |  |  | - Other TRM - Transit Management | This includes all fixed-route transit systems serving the greater Klamath Falls area, including Basin Transit Service Transportation District. |
| Klamath Falls-Area Fixed Route Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | This includes transit vehicles operating on fixed-route transit systems in the greater Klamath Falls area, including Basin Transit Service Transportation District. |
| Klamath Falls-Area Paratransit Dispatch | Existing |  |  |  | - Other TRM - Transit Management | This includes Red Ball Stage Lines Transit Dispatch Operations. |
| Klamath Falls-Area Paratransit Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | This includes Red Ball Stage Lines Transit. |
| Klamath Falls-Area Planning Agencies | Existing |  |  |  | - Archived Data Management Subsystem - Other Archives | This includes city and county planning agencies that may use data archived from ITS. |
| Klamath Falls-Area Police and Sheriff Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes the cities of Klamath Falls and Lakeview. This also includes county sheriffs’ dispatch operations in Klamath County and Lake County. |
| Klamath Falls-Area Police and Sheriff Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes law enforcement vehicles for the cities of Klamath Falls and Lakeview, and for county sheriffs in Klamath and Lake Counties. |
| Klamath Falls-Area Towing/Recovery | Existing |  |  |  | - Other EM | This includes the private tow dispatch systems that clear incidents in the Klamath Falls area. This includes numerous individual operators that serve in the region. |
| Klamath Falls-Area Traveler Information Resources | Existing |  |  |  | - Yellow Pages Service Providers | This includes Collier State Park, Crater Lake National Park, Klamath Falls Parks & Recreation, Kla-Mo-Ya Casino, Lava Beds National Monument, Running Y Ranch, and Winema National Forest. It also includes area hotels, restaurants, gas stations, shopping, etc. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

124Western Transportation Institute

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Klamath Falls-Area TV, Radio Stations and Newspapers | Existing |  |  |  | - Media | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the analog TV, radio and print media intended for human, rather than system, processing and interpretation. |
| Klamath Falls-Area Visitor Information Services | Existing |  |  |  | - Information Service Provider - Other ISP | This includes Klamath County Department of Tourism, Klamath Falls Chamber of Commerce, and Lakeview Chamber of Commerce. |
| Mayday Service Provider | Existing |  |  |  | - Emergency Management - Other EM | Service centers that provide Mayday and other special concierge services for their clients. These centers will typically be outside the COATS region, but will interact with local agencies when their client’s have an emergency in the region. Current examples of this system include the service centers operated by GM (OnStar), ATX Technologies (Ford and others), and AAA. |
| MCMIS | Existing |  |  |  | - Commercial Vehicle Administration - Other CVAS | MCMIS contains information on the safety fitness of commercial motor carriers and hazardous material shippers subject to the Federal Motor Carrier Safety Regulations and the Hazardous Materials Regulations. This information is available to the general public through the MCMIS Data Dissemination Program. |
| Medford-Area City Traffic Control Equip | Existing |  |  |  | - Roadway Subsystem | This includes field equipment for the cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston. |
| Medford-Area City Traffic Control Systems | Existing |  |  |  | - Other TM - Traffic Management | This includes traffic control systems for the cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston that are owned and operated by the respective Public Works Departments for each city. |
| Medford-Area County Maintenance Systems | Existing |  |  |  | - Construction and Maintenance | This includes the highway maintenance systems for Douglas County, Jackson County, and Josephine County. |
| Medford-Area E-911 Comm Centers | Existing |  |  |  | - Emergency Management - Other EM | This includes public safety answering points for the greater Medford area, including Douglas Co. 911 Comm Center, Jackson Co. 911 Comm Center, Josephine Co. 911 Comm Center, Myrtle Creek Comm Center, Medford Comm Center, Ashland Comm Center, and Southern Oregon Regional Communications Center. |
| Medford-Area EMS Provider Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes Douglas County Emergency Services, Jackson County Emergency Management, Josephine County Emergency Services, Mercy Flights and Pacific Flights. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

Western Transportation Institute125

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Medford-Area EMS Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes vehicles operated by Douglas County Emergency Services, Jackson County Emergency Management, Josephine County Emergency Services, Mercy Flights and Pacific Flights. |
| Medford-Area Fire/Rescue Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes the Fire/Rescue Dispatch Operations in the Medford region including county fire districts (Douglas, Jackson, and Josephine County) and municipal fire departments (Cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, Winston, and Wolf Creek). |
| Medford-Area Fire/Rescue Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes the Fire/Rescue vehicles in the Medford region including those owned by county fire districts (Douglas, Jackson, and Josephine County) and municipal fire departments (Cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, Winston, and Wolf Creek). |
| Medford-Area Fixed Route Dispatch | Existing |  |  |  | - Other TRM - Transit Management | This includes all fixed-route transit systems serving the greater Medford area, including Rogue Valley Transportation District and Roseburg Transit. |
| Medford-Area Fixed Route Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | This includes transit vehicles operating on fixed-route transit systems in the greater Medford area, including Rogue Valley Transportation District and Roseburg Transit. |
| Medford-Area Paratransit Dispatch | Existing |  |  |  | - Other TRM - Transit Management | This includes private shuttle services (e.g., Airport Transit) and public services (Southern Oregon Transportation) that provide door-to-door transit services in the Medford area. |
| Medford-Area Paratransit Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | This includes private shuttle services (e.g., Airport Transit) and public services (Southern Oregon Transportation) that provide door-to-door transit services in the Medford area. |
| Medford-Area Planning Agencies | Existing |  |  |  | - Archived Data Management Subsystem - Other Archives | This includes various city and county planning agencies that may have use for archived data, including but not limited to the Rogue Valley Council of Governments. |
| Medford-Area Police and Sheriff Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes the local police department dispatch systems operating in the cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston. This also includes county sheriffs for Jackson and Josephine Counties. |
| Medford-Area Police and Sheriff Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes law enforcement vehicles serving the cities of Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston. This also includes county sheriff vehicles for Jackson and Josephine Counties. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

126Western Transportation Institute

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Medford-Area Towing/Recovery | Existing |  |  |  | - Other EM | This includes the private tow dispatch systems that clear incidents in the Medford area. This includes numerous individual operators that serve in the region. |
| Medford-Area Traveler Information Resources | Existing |  |  |  | - Yellow Pages Service Providers | This includes the State Parks and Recreation Department, parks and recreation departments for Douglas County, Jackson County, and Josephine County, the City Parks and Recreation areas for the cities in the region (Ashland, Central Point, Grants Pass, Medford, Roseburg, and Winston), Oregon Caves National Monument, Shakespeare Festival, Britt Festival, and Seven Feathers Casino. It also includes area hotels, restaurants, gas stations, shopping, etc. |
| Medford-Area TV and Radio Stations | Existing |  |  |  | - Media | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the analog TV, radio and print media intended for human, rather than system, processing and interpretation. |
| Medford-Area Visitor Centers | Existing |  |  |  | - Information Service Provider - Other ISP | This includes visitor centers in Medford and Ashland. |
| Medford-Area Visitor Information Services | Existing |  |  |  | - Information Service Provider - Other ISP | This includes the systems operated by Oregon Tourism Commission, Chambers of Commerce and other organizations that provide tourist information for the local region. |
| Motor Carrier Dispatch and Back Office Systems | Existing |  |  |  | - Fleet and Freight Management | These provide the capability for commercial drivers and dispatchers to receive real-time routing information and access databases containing vehicle and cargo locations as well as carrier, vehicle, cargo, and driver information. |
| ODOT Bend TOC | Existing |  |  |  | - Other TM | This center is at the periphery of the COATS region. It provides traffic control capabilities for the ODOT-operated facilities in eastern Oregon. |
| ODOT Communications Division | Existing |  |  |  | - Media - Public Information Office* | The Communications Division produces and/or oversees ODOT’s internal and external communications, including the agency’s Internet site. It has offices located in each of ODOT’s five regions to provide information to local news media outlets. |
| ODOT Field Equip (Medford TOC) | Existing |  |  |  | - Roadway Subsystem | This includes the environmental sensors, variable message signs, HAR, signals, traffic detectors, and any other field equipment and instrumentation operated by ODOT Region 3. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

Western Transportation Institute127

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| ODOT Maintenance Districts (Medford TOC) | Existing |  |  |  | - Construction and Maintenance - Maintenance Dispatch Office* - Other Maintenance Dispatch Office* | Covers the resource management and dispatch systems operated by ODOT maintenance districts in Coos Bay, Roseburg, Medford and Klamath Falls. |
| ODOT Maintenance Vehicles (Medford TOC) | Existing |  |  |  | - Maintenance Vehicles* - Other Maintenance Vehicles* | These ODOT vehicles are responsible for maintenance activities, including setting up road closures, assisting in incident removal, and performing winter maintenance. It is assumed that there is seamless communication between the vehicle and its driver, who will actually be directing and performing the maintenance activity. |
| ODOT Medford TOC | Existing |  |  |  | - Other TM - Traffic Management | The ODOT Transportation Operations Center in Medford plays a central role in the COATS architecture. It connects to centers in Bend and Salem and Caltrans centers in Eureka and Redding. |
| ODOT RWIS Server | Existing |  |  |  | - Weather Service | This is a central, statewide server which collects data from each of the state’s regional RWIS servers. |
| ODOT Salem TOC | Existing |  |  |  | - Other TM | This center is at the periphery of the COATS region. It provides traffic control capabilities for the ODOT-operated facilities in the Lane, Linn and Marion Counties portion of the COATS region. |
| ODOT Statewide Planning | Existing |  |  |  | - Archived Data Management Subsystem - Other Archives | This includes all divisions within ODOT that will use archived data. |
| ODOT Weather Warning System | Existing |  |  |  | - Roadway Subsystem | These are driver advisory systems which are activated based on certain meteorological conditions or thresholds (e.g. high wind, icy bridge, etc.). They process information provided by a localized weather station and provide a warning to motorists when specified conditions arise. |
| Oregon Weigh Stations | Existing |  |  |  | - Commercial Vehicle Check | These are locations where commercial vehicles may be weighed and inspected. |
| OSP Dispatch | Existing |  |  |  | - Emergency Management - Other EM | The Oregon State Police operates several dispatch/communications centers in southern Oregon, all of which operate on a common computer-aided dispatch system for tracking call and response activities. These are often collocated with ODOT facilities for improved incident management, although ODOT and OSP staff will have differing roles in incident management. |
| OSP Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | Oregon State Police vehicles. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

128Western Transportation Institute

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Other Weather Data | Existing |  |  |  | - Weather Service | This terminator refers to weather information received from the National Weather Service or state agencies which also have weather-related information. This could include fully equipped weather stations, isolated stream gauges, or other similar technologies. |
| Redding-Area Air Districts | Existing |  |  |  | - Emissions Management | This placeholder includes Colusa County APCD, Glenn County APCD, Lassen County APCD, Modoc County APCD, Northern Sierra APCD, Shasta County AQMD, Siskiyou County APCD, and Tehama County APCD |
| Redding-Area City Traffic Control Equip | Existing |  |  |  | - Roadway Subsystem | This includes traffic equipment for the cities of Alturas, Anderson , Corning, Mount Shasta, Orland, Red Bluff, Redding, Shasta Lake, Susanville, Weed, Willows, and Yreka. |
| Redding-Area City Traffic Control Systems | Existing |  |  |  | - Other TM - Traffic Management | This includes traffic management functions for the cities of Alturas, Anderson, Corning, Mount Shasta, Orland, Red Bluff, Redding, Shasta Lake, Susanville, Weed, Willows, and Yreka. |
| Redding-Area County Maintenance Systems | Existing |  |  |  | - Construction and Maintenance | This includes public works departments for the following counties: Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama, and Trinity. |
| Redding-Area E-911 Comm Centers | Existing |  |  |  | - Emergency Management - Other EM | This includes public safety answering points receiving wireline 911 calls in the Redding area. |
| Redding-Area EMS Provider Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes the broad range of private EMS, ambulance, and air evacuation providers that serve the Redding area, including American Med. Response (includes North Valley Ambulance), City of Dorris, Emergency Medical Center, Mount Shasta Ambulance Service, NorCal EMS, Northern Siskiyou Ambulance Service, Shascom, and Trinity County Life Support. |
| Redding-Area EMS Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes the broad range of private EMS, ambulance, and air evacuation vehicles that serve the Redding area, including American Med. Response (includes North Valley Ambulance), City of Dorris, Emergency Medical Center, Mount Shasta Ambulance Service, NorCal EMS, Northern Siskiyou Ambulance Service, Shascom and Trinity County Life Support. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

Western Transportation Institute129

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Redding-Area Fire Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes fire departments for the communities of Alturas, Anderson, Corning, Cottonwood, Happy Valley, Jones Valley, Mountain Gate, Mount Shasta, Orland, Red Bluff, Redding, Shasta County, Shasta, Shasta Lake City, Susanville, Weed, Willows, and Yreka. This also includes other agencies that have fire/rescue jurisdiction in certain areas, including California Department of Forestry; County Health Departments (Hazmat), National Park Service, Office of Emergency Services (Hazmat), and US Forest Service. |
| Redding-Area Fire Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes vehicles for community fire departments in Alturas, Anderson, Corning, Cottonwood, Happy Valley, Jones Valley, Mountain Gate, Mount Shasta, Orland, Red Bluff, Redding, Shasta County, Shasta, Shasta Lake City, Susanville, Weed, Willows, and Yreka. This also includes vehicles from other agencies that have fire/rescue jurisdiction in certain areas, including California Department of Forestry; County Health Departments (Hazmat), National Park Service, Office of Emergency Services (Hazmat), and US Forest Service. |
| Redding-Area Fixed Route Dispatch | Existing |  |  |  | - Other TRM - Transit Management | This includes the following transit systems: Lassen Rural Bus, serving Lassen County; Modoc Sage Stage; Redding Area Bus Authority (RABA) serving Redding and the surrounding communities; Siskiyou County Transit (the Stage) serving the Yreka area; Tehama Area Rural Express (TRAX) serving the Red Bluff area; and Trinity County Transit. |
| Redding-Area Fixed Route Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | This includes vehicles operating on the following transit systems: Lassen Rural Bus, serving Lassen County; Modoc Sage Stage; Redding Area Bus Authority (RABA) serving Redding and the surrounding communities; Siskiyou County Transit (the Stage) serving the Yreka area; Tehama Area Rural Express (TRAX) serving the Red Bluff area; and Trinity County Transit. |
| Redding-Area Municipal Planning Agencies | Existing |  |  |  | - Archived Data Management Subsystem - Other Archives | These data collection and analysis system(s) provide planning data for cities and towns in Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama and Trinity Counties. |
| Redding-Area Paratransit Dispatch | Existing |  |  |  | - Other TRM - Transit Management | This includes California Association for Coordinated Transit, DART, Lassen Rural Bus, Modoc Senior Citizens/Meals on Wheels |
| Redding-Area Paratransit Vehicles | Existing |  |  |  | - Transit Vehicle Subsystem | This includes California Association for Coordinated Transit, DART, Lassen Rural Bus, Modoc Senior Citizens/Meals on Wheels |

Table C-1: Inventory of Stakeholders and Systems (cont.).

130Western Transportation Institute

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Redding-Area Police and Sheriff Dispatch | Existing |  |  |  | - Emergency Management - Other EM | This includes the cities of Alturas, Anderson, Corning, Lake Shastina, Mount Shasta, Orland, Red Bluff, Redding, Susanville, Weed, Willows, and Yreka. This also includes county sheriffs for Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama, and Trinity Counties. |
| Redding-Area Police and Sheriff Vehicles | Existing |  |  |  | - Emergency Vehicle Subsystem | This includes the vehicles for the cities of Alturas, Anderson, Corning, Lake Shastina, Mount Shasta, Orland, Red Bluff, Redding, Susanville, Weed, Willows, and Yreka; and for the county sheriffs for Colusa, Glenn, Lassen, Modoc, Plumas, Shasta, Siskiyou, Tehama, and Trinity Counties. |
| Redding-Area Regional Transportation Planning Agencies | Existing |  |  |  | - Archived Data Management Subsystem - Other Archives | This includes county agencies responsible for transportation planning in the Redding vicinity. This includes the Lassen County Transportation Commission (TC), Modoc County Local Transportation Commission (LTC), Plumas County TC, Shasta County RTPA (also a MPO), Siskiyou County TC, Tehama County TC, and Trinity County TC. |
| Redding-Area Towing/Recovery | Existing |  |  |  | - Other EM | This includes the private tow dispatch systems that clear incidents in the Redding area. This includes numerous individual operators that serve in the region. |
| Redding-Area Traveler Information Resources | Existing |  |  |  | - Yellow Pages Service Providers | This includes Ashland (OR) Shakespeare Festival, BLM-Alturas Resource Area, Castle Crags State Park, Klamath National Forest, Lassen National Forest, Lassen Volcanic National Park, Lavabeds National Monument, Modoc National Forest, Mt. Shasta Ranger Station, Mt. Shasta Ski Park, Redding Recreation Department, Shasta-Trinity National Forest, Turtle Bay Park / Museum, U.S. Forest Service, Whiskeytown National Recreational Area. It also includes area hotels, restaurants, gas stations, shopping, etc. |
| Redding-Area Traveler Information Systems | Planned |  |  |  | - Information Service Provider - Other ISP | These systems collect traveler information from operational sources (e.g., local traffic and public safety agencies/systems), visitor information systems (e.g. chambers of commerce), and local travel destinations (e.g., Parks). This information is then made available to travelers through both broadcast and interactive digital information services through the Internet and other media. |
| Redding-Area TV and Radio Stations | Existing |  |  |  | - Media | Local media that includes traveler information with entertainment, general news and other topical information. In contrast to other traveler information inventory items, this entry represents the analog TV, radio and print media intended for human, rather than system, processing and interpretation. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

Western Transportation Institute131

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Redding-Area Visitor Information Services | Existing |  |  |  | - Information Service Provider - Other ISP | This includes chambers of commerce in Alturas, Dunsmuir, Redding, Trinity County, Weaverville and Yreka. It also includes Northern California Tourism , Redding Convention and Visitors Bureau, Shasta Cascade Wonderland Association, and Siskiyou County Tourism. |
| SAFER | Existing |  |  |  | - Commercial Vehicle Administration - Other CVAS | SAFER provides carrier, vehicle, and driver safety and credential information to fixed and mobile roadside inspection stations. This information will allow the roadside inspector to select vehicles and/or drivers for inspection based on the number of prior carrier inspections, as well as carrier, vehicle, and driver safety and credential historical information. |
| SOVA Kiosks | Existing |  |  |  | - Remote Traveler Support | These kiosks will be upgraded to be Web-based with potential access to TripCheck and other traveler information services in the Southern Oregon region. |
| Transit Users | Existing |  |  |  | - Transit User | This terminator represents the human entities using Public Transit vehicles. They may be in the act of embarking or debarking the vehicles and are thus sensed for the purpose of determining passenger loading and fares, or on the vehicles and able to request and receive information. |
| Traveler Information Devices | Planned |  |  |  | - Personal Information Access | These are personal devices which may be used to access real-time transportation information. |
| Travelers | Existing |  |  |  | - Traveler | This terminator represents any individual (human) who uses transportation services. At the time that data is passed to or from the terminator the individual is neither a driver, pedestrian, or transit user. This means that the data provided is that for pre-trip planning or multi-modal personal guidance and includes their requests for assistance in an emergency. Subsequent to receipt of pre-trip information, a Traveler may become a vehicle driver, passenger, transit user, or pedestrian. |
| TripCheck | Existing |  |  |  | - Information Service Provider - Other ISP - Personal Information Access | TripCheck is the latest version of ODOT’s travel information Internet Web site. Its goal is to provide the best travel information possible in a way that is easy to access. Tripcheck uses maps extensively, providing users with a convenient interface and a fast way to navigate the site. It includes roadway incident maps, camera images, RWIS information, and other features. |
| Vehicle Information Systems | Existing |  |  |  | - Other Vehicle - Vehicle | These are devices installed in passenger vehicles that support access to digital traveler information. A wide range of evolving technologies are available. |

Table C-1: Inventory of Stakeholders and Systems (cont.).

132Western Transportation Institute

| Element Name | Status | South. Oregon | NCent. Calif. | N Cal. Coast | National Architecture Mapping | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle Mayday Systems | Existing |  |  |  | - Other Vehicle - Vehicle | These are devices installed in passenger vehicles that provide Mayday services to the vehicle occupants. In many cases, these systems are integrated with Vehicle Information Systems (see related Inventory entry). |
| Vehicles | Existing |  |  |  | - Basic Vehicle | This terminator represents the basic vehicle platform that interfaces with and hosts ITS electronics. The Basic Vehicle terminator provides an interface to drive train, driver convenience and entertainment systems, and other non-ITS electronics on-board the vehicle. This interface allows general vehicle systems (e.g., the stereo speaker system) to be shared by ITS and non-ITS systems. It also allows monitoring and control of the vehicle platform for advanced vehicle control system applications. |
| Virtual Transit Mall | Planned |  |  |  | - Information Service Provider - Other ISP | This is a Web-based service that will coordinate Oregon’s 200-plus transit providers in order to offer: greater access to transit information, statewide trip planning, improved connections between providers, and enhanced transit planning data. |

* – Extension to National Architecture

# Appendix D  Architecture Flow Diagrams

The following is a list of all stakeholders and subsystems included in the COATS Regional Architecture, along with the page numbers on which architecture flow context diagrams are located. Page numbers listed in boldface refer to a diagram for which the indicated entity is the center. Turbo Architecture did not display architecture flows to and from certain terminators, although these are still included in the architecture. These are indicated by alphabetic codes.

Entity Name	Pages

1-800-427-ROAD	151, 177, 321

1-800-977-ODOT	152, 232, 321

Airline Schedule Operations	153, 228, 314, 328

Amtrak Schedule Operations	154, 228, 314, 328

California Highway Information Web Page	155, 156, 157, 159, 161, 164, 177, 184, 229, 315, 321

California Weigh Stations	not included in diagrams

Caltrans District 1 Public Affairs	155, 156, 175, 229,

Caltrans District 2 Public Affairs	155, 157, 176, 315

Caltrans Field Equip District 1 (Eureka)	155, 159, 166, 175, 180, 183, 207, 208, 217, 219, 226, 327, A

Caltrans Field Equip District 2 (Redding)	155, 161, 170, 176, 182, 183, 207, 295, 303, 305, 312, 327, B

Caltrans HQ Construction Program	162, 166, 170,

Caltrans HQ Permits	163, 166, 170, 175, 176,

Caltrans HQ Traffic	155, 164, 166, 170, 175, 176,

Caltrans Maintenance District 1 (Eureka)	159, 162, 163, 164, 166, 167, 170, 175, 177, 220, 282

Caltrans Maintenance District 1 (Eureka) Vehicles	166, 167, 171, 183, 217, 219, 226, 283, 292

Caltrans Maintenance District 2 (Redding)	161, 162, 163, 164, 166, 170, 171, 176, 177, 282, 306

Caltrans Maintenance District 2 (Redding) Vehicles	167, 170, 171, 183, 283, 292, 303, 305, 312

Caltrans RWIS	173, 174, 175, 176, 180, 182, 211, 284, 298

Caltrans Weather Warning Systems	173, 174, 207,

Caltrans/CHP Eureka ARTIC	
	156, 159, 163, 164, 166, 173, 175, 176, 177, 180, 211, 213, 214, 217, 219, 220, 221, 226, 284, 294, A, D

Caltrans/CHP Oakland Regional TMC	159, 175, 177, A, B

Caltrans/CHP Redding ARTIC	
	157, 161, 163, 164, 170, 173, 175, 176, 177, 182, 284, 294, 298, 299, 301, 303, 305, 306, 308, 312, B, I

Caltrans/CHP Sacramento Regional TMC	161, 176,  177, A, B

CHIN	151, 155, 166, 170, 175, 176, 177, 180, 182, A, B

CHP Eureka and Humboldt Division Dispatch and 911 Centers	
	159, 173, 175, 177, 180, 182, 183, 214, 217, 219, 226, 229, 284, 290, 294, 320, D

CHP Northern District Dispatch and 911 Centers	
	161, 173, 176, 177, 180, 182, 183, 290, 294, 301, 303, 305, 312, 315, 320, I

CHP Vehicles	159, 161, 167, 
171, 180, 182, 183, 192, 194, 200, 209, 217, 219, 226, 241, 243, 251, 264, 266, 274, 283, 292, 297, 303, 305, 312

COATS Kiosks	155, 184, 204, 230, 254, 278, 316, 319, 321, 323, 328

Commercial Vehicle (Onboard) Systems	not included in diagrams

Commercial Vehicle Driver	not included in diagrams

Commercial Vehicles	not included in diagrams

Coos Bay-Area City Traffic Control Equip	186, 188, 192, 194, 200, 207, 292, 327

Coos Bay-Area City Traffic Control Systems	186, 188¸189, 190, 192, 194, 196, 198, 200, 202, 284, 294, C

Coos Bay-Area County Maintenance Systems	188, 189, 196, 198, 284

Coos Bay-Area E-911 Comm Centers	188, 190, 192, 194, 198, 200, 255, 284, 290, 325

Coos Bay-Area EMS Provider Dispatch	188, 190, 192¸194, 196, 198, 200, 202, 284, 290

Coos Bay-Area EMS Vehicles	183, 186, 192, 194, 200, 217, 219, 226, 264, 266, 274, 280, 283, 292

Coos Bay-Area Fire and Rescue Dispatch	188, 190, 192, 194, 196, 198, 200, 202, 284, 290, C

Coos Bay-Area Fire and Rescue Vehicles	183, 186, 192, 194, 200, 217, 219, 226, 264, 266, 274, 280, 283, 292

Coos Bay-Area Paratransit Dispatch	188, 189, 192, 194, 196, 198, 200, 284, 290, 328

Coos Bay-Area Paratransit Vehicles	196, 319

Coos Bay-Area Planning Agencies	188, 189, 190, 192, 194, 196, 198, 200, 287

Coos Bay-Area Police and Sheriff Dispatch	188, 190, 192, 194, 196, 198, 200, 202, 284, 290, C

Coos Bay-Area Police and Sheriff Vehicles	183, 186, 192, 194, 200, 217, 219, 226, 264, 266, 274, 280, 283, 292

Coos Bay-Area Radio Stations and Newspapers	188, 192, 194, 200, 202¸ 279, 284, 290, 323

Coos Bay-Area Towing/Recovery	188, 194, 200, 284, 290, C

Coos Bay-Area Traveler Information Resources	203, 204,

Coos Bay-Area Visitor Information Services	184, 203, 204, 317,

CVO Inspector	not included in diagrams

DEQ Medford Regional Office	205, 257, 272, 280, 284

Downtown Parking District	206, 236, 249

Drivers	159, 161, 174, 186, 207, 209, 235, 257, 280, 289, 297, 324

Eureka-Area Air Districts	159, 208¸209, 223,

Eureka-Area City Traffic Control Equip	183, 207, 208, 209, 211, 217, 219, 226, 327

Eureka-Area City Traffic Control Systems
	173, 175, 209, 211, 213, 214, 217, 219, 220, 221, 223, 226, 228, 229, 294, D

Eureka-Area County Maintenance Systems	175, 211, 213, 220, 221, 223,

Eureka-Area E-911 Comm Centers	175, 180, 211, 214, 217, 219, 223, 226, 255, 325

Eureka-Area EMS Provider Dispatch	175, 180, 211, 214, 217, 219, 220, 221, 223, 226, 229,

Eureka-Area EMS Vehicles	159, 167, 183, 192, 194, 200, 209, 217, 219, 226, 264, 266, 274, 292, 303, 305, 312

Eureka-Area Fire and Rescue Dispatch	175, 180, 211, 214, 217, 219, 220, 221, 223, 226, 229, D

Eureka-Area Fire and Rescue Vehicles
	159, 167, 183, 192, 194, 200, 209, 217, 219¸226, 264, 266, 274, 292, 303, 305, 312

Eureka-Area Fixed Route Dispatch	166, 175, 211, 213, 217, 219, 220, 221, 223, 226, 228,

Eureka-Area Fixed Route Vehicles	220¸319

Eureka-Area Municipal Planning Agencies	211, 214, 217, 219, 220, 221, 223, 226,

Eureka-Area Paratransit Dispatch	175, 211, 213, 217, 219, 220, 221, 223, 226,

Eureka-Area Paratransit Vehicles	221, 319

Eureka-Area Police and Sheriff Dispatch	175, 180, 211, 214, 217, 219, 220, 221, 223, 226, 229, D

Eureka-Area Police and Sheriff Vehicles
	159, 167, 183, 192, 194, 200, 209, 217, 219, 226, 264, 266, 274, 292, 303, 305, 312

Eureka-Area Regional Transportation Planning Agencies	208, 213, 220, 221, 223

Eureka-Area Towing/Recovery	175, 180, 211, 219, 226, D

Eureka-Area Traveler Information Resources	227, 230

Eureka-Area Traveler Information Systems	153, 154, 211, 220, 228, 233, 314,

Eureka-Area TV and Radio Stations	155, 156, 180, 211, 217, 219, 226, 229

Eureka-Area Visitor Information Services	184, 227, 230

Green Light Admin System	not included in diagrams

HELP	not included in diagrams

Highway Travel Conditions Reporting System	152, 232, 282, 284, 286, 287, 290, 294, 323, G, H

IFTA Clearinghouse	not included in diagrams

Inter-City Transit	228, 233, 314, 328

Inter-City Transit Vehicles	233¸319

IRP Clearinghouse	not included in diagrams

Klamath Falls-Area City Traffic Control Equip	207, 235, 236, 241, 243, 251, 292, 327

Klamath Falls-Area City Traffic Control Systems
	206, 235, 236, 237, 238, 241, 243, 245, 247, 249, 251, 253, 284, 294, E, G

Klamath Falls-Area County Maintenance	236, 237, 245, 247, 249, 284

Klamath Falls-Area E-911 Comm Centers	236, 238, 241, 243, 249, 251, 255, 284, 290, 325, G

Klamath Falls-Area EMS Provider Dispatch	236, 238, 241, 243, 245, 247, 249, 251, 253, 284, 290, G

Klamath Falls-Area EMS Vehicles	183, 235, 241, 243, 251, 264, 266, 274, 280, 283, 292, 303, 305, 312

Klamath Falls-Area Fire Dispatch	236, 238, 241, 243, 245, 247, 249, 251, 253, 284, 290

Klamath Falls-Area Fire Vehicles	183, 235, 241, 243, 251, 264, 266, 274, 280, 283, 292, 303, 305, 312, E

Klamath Falls-Area Fixed Route Dispatch	236, 237, 241, 243, 245, 247, 249, 251, 282, 284, 328

Klamath Falls-Area Fixed Route Vehicles	245, 319

Klamath Falls-Area Paratransit Dispatch	236, 237, 241, 243, 245, 247, 249, 251, 284, 290, 328

Klamath Falls-Area Paratransit Vehicles	247, 319

Klamath Falls-Area Planning Agencies	206, 236, 237, 238, 241, 243, 245, 247, 249, 251, 287

Klamath Falls-Area Police and Sheriff Dispatch	236, 238, 241, 243, 245, 247, 249, 251, 253, 284, 290, E

Klamath Falls-Area Police and Sheriff Vehicles	183, 235, 241, 243, 251¸264, 266, 274, 280, 283, 292, 303, 305, 312

Klamath Falls-Area Towing/Recovery	236, 243, 251, 284, 290, E

Klamath Falls-Area Traveler Information Resources	252, 254

Klamath Falls-Area TV, Radio Stations and Newspapers	236, 241, 243, 251, 253, 279, 284, 290, 323

Klamath Falls-Area Visitor Information Services	184, 252, 254, 317

Mayday Service Provider	190, 214, 238, 255, 262, 301, 325

MCMIS	not included in diagrams

Medford-Area City Traffic Control Equip	205, 207, 257, 259, 264, 266, 274, 292, 327

Medford-Area City Traffic Control Systems	257, 259, 260, 262, 264, 266, 268, 270, 272, 274, 276, 284, 294

Medford-Area County Maintenance Systems	259, 260, 268, 270, 272, 284

Medford-Area E-911 Comm Centers	255, 259, 262, 264, 266, 272, 274, 284, 290, 325

Medford-Area EMS Provider Dispatch	259, 262, 264, 266, 268, 270, 272, 274, 276, 284, 290

Medford-Area EMS Vehicles
	183, 192, 194, 200, 217, 219, 226, 241, 243, 251, 257, 264, 266, 274, 280, 283, 292, 303, 305, 312

Medford-Area Fire/Rescue Dispatch	259, 262, 264, 266, 268, 270, 272, 274, 276, 284, 290, F

Medford-Area Fire/Rescue Vehicles
	183, 192, 194, 200, 217, 219, 226, 241, 243, 251, 257, 264, 266, 274, 280, 283, 292, 303, 305, 312

Medford-Area Fixed Route Dispatch	259, 260, 264, 266, 268, 270, 272, 274, 282, 284, 328

Medford-Area Fixed Route Vehicles	268, 319

Medford-Area Paratransit Dispatch	259, 260, 264, 266, 268, 270, 272, 274, 284, 290, 328

Medford-Area Paratransit Vehicles	270, 319

Medford-Area Planning Agencies	205, 259, 260, 262, 264, 266, 268, 270, 272, 274, 287

Medford-Area Police and Sheriff Dispatch	259, 262, 264, 266, 268, 270, 272, 274, 276, 284, 290, F, G

Medford-Area Police and Sheriff Vehicles
	183, 192, 194, 200, 217, 219, 226, 241, 243, 251, 257, 264, 266, 274, 280, 283, 292, 303, 305, 312

Medford-Area Towing/Recovery	266, 274, 284, 290, F, G

Medford-Area Traveler Information Resources	275, 277, 278

Medford-Area TV and Radio Stations	259, 264, 266, 274, 276, 279, 284, 290, 323

Medford-Area Visitor Centers	275, 277, 278,

Medford-Area Visitor Information Services	184, 275, 277, 278, 317

Motor Carrier Dispatch and Back Office Systems	not included in diagrams

ODOT Bend TOC	232, 236, 238, 241, 274, 284, 287, 290, E, F, G, H

ODOT Communications Division	202, 253, 276, 279, 284, 323

ODOT Field Equip (Medford TOC)
	192, 194, 200, 205, 207, 241, 243, 251, 264, 266, 274, 280, 282, 284, 287, 290, 292, 323, 327

ODOT Maintenance Districts (Medford TOC)	166, 170, 232, 245, 268, 280, 282¸283, 284, 287, 323

ODOT Maintenance Vehicles (Medford TOC)	
	167, 171, 183, 192, 194, 200, 241, 243, 251, 264, 266, 274, 282, 283, 292

ODOT Medford TOC	
	173, 175, 176, 180, 188, 189, 190, 192, 194, 196, 200, 202, 205, 232, 236, 237, 238, 241, 243, 245, 
247, 251, 253, 259, 260, 262, 264, 266, 268, 270, 274, 276, 279, 280, 282, 284, 287, 290, 294, 323, C, E, F, G, H

ODOT RWIS Server	232, 286, 287, 289

ODOT Salem TOC	232, 284, 287, 290, G, H

ODOT Statewide Planning	198, 232, 249, 272, 280, 282, 284, 286, 287, G, H

ODOT Weather Warning System	207, 286, 289

Oregon Weigh Stations	not included in diagrams

OSP Dispatch	180, 182, 190, 192, 194, 196, 
200, 202, 232, 238, 241, 243, 247, 251, 253, 262, 264, 266, 270, 274, 276, 280, 284, 290, 292, 294, C, E, F, G, H

OSP Vehicles	167, 171, 
183, 186, 192, 194, 200, 217, 219, 226, 235, 241, 243, 251, 257, 264, 266, 274, 280, 283, 290, 292, 303, 305, 312

Other Weather Data	175, 176, 180, 182, 188, 211, 232, 236, 259, 284, 290, 294, 298

Redding-Area Air Districts	161, 295, 297, 310

Redding-Area City Traffic Control Equip	183, 207, 295, 297, 298, 303, 305, 312, 327

Redding-Area City Traffic Control Systems	
	173, 176, 294, 297, 298, 299, 301, 303, 305, 306, 308, 310, 312, 314, 315,

Redding-Area County Maintenance Systems	176, 298, 299, 306, 308, 310

Redding-Area E-911 Comm Centers	176, 182, 255, 298, 301, 303, 305, 310, 312, 325

Redding-Area EMS Provider Dispatch	176, 182, 298, 301, 303, 305, 306, 308, 310, 312, 315,

Redding-Area EMS Vehicles	161, 171, 183, 217, 219, 226, 241, 243, 251, 264, 266, 274, 292, 297, 303, 305, 312

Redding-Area Fire Dispatch	176, 182, 298, 301, 303, 305, 306, 308, 310, 312, 315, I

Redding-Area Fire Vehicles	161, 171, 183, 217, 219, 226, 241, 243, 251, 264, 266, 274, 292, 297, 303, 305, 312

Redding-Area Fixed Route Dispatch	170, 176, 298, 299, 303, 305, 306, 308, 310, 312, 314,

Redding-Area Fixed Route Vehicles	306, 319

Redding-Area Municipal Planning Agencies	298, 301, 303, 305, 306, 308, 310, 312

Redding-Area Paratransit Dispatch	176, 298, 299, 303, 305, 306, 308, 310, 312

Redding-Area Paratransit Vehicles	308, 319

Redding-Area Police and Sheriff Dispatch	176, 182, 298, 301, 303, 305, 306, 308, 310, 312, 315, I

Redding-Area Police and Sheriff Vehicles	
	161, 171, 183, 217, 219, 226, 241, 243, 251, 264, 266, 274, 292, 297, 303, 305, 312

Redding-Area Regional Transportation Planning Agencies	295, 298, 299, 306, 308, 310

Redding-Area Towing/Recovery	176, 182, 305, 312, I

Redding-Area Traveler Information Resources	313, 316

Redding-Area Traveler Information Systems	153, 154, 228, 233, 298, 306, 314

Redding-Area TV and Radio Stations	155, 157, 182, 298, 303, 305, 312, 315

Redding-Area Visitor Information Services	184, 313, 316

SAFER	not included in diagrams

SOVA Kiosks	204, 254, 278, 317, 319, 321, 323, 328

Transit Users	184, 196, 220, 221, 233, 245, 247, 268, 270, 306, 308, 317, 319

Traveler Information Devices	180, 182, 320, 321, 323

Travelers	151, 152, 155, 184, 317, 320, 321, 323

TripCheck	184, 202, 232, 253, 276, 279, 280, 282, 284, 317, 320, 321, 323, 324, 328

Vehicle Information Systems	207, 323, 324

Vehicle Mayday Systems	190, 214, 238, 255, 262, 301, 325, 327

Vehicles	159, 161, 186, 209, 235, 257, 280, 297, 325, 327

Virtual Transit Mall	153, 154, 184, 196, 233, 245, 247, 268, 270, 317, 323, 328

Figure D-1: Regional Diagram for 1-800-427-ROADFigure D-1: Regional Diagram for 1-800-427-ROAD.

Western Transportation Institute139

Figure D-2: Regional Diagram for 1-800-977-ODOTFigure D-2: Regional Diagram for 1-800-977-ODOT.

140Western Transportation Institute

Figure D-3: Regional Diagram for Airline Schedule OperationsFigure D-3: Regional Diagram for Airline Schedule Operations.

Western Transportation Institute141

Figure D-4: Regional Diagram for Amtrak Schedule OperationsFigure D-4: Regional Diagram for Amtrak Schedule Operations.

142Western Transportation Institute

Figure D-5: Regional Diagram for California Highway Information Web PageFigure D-5: Regional Diagram for California Highway Information Web Page.

Western Transportation Institute143

Figure D-6: Regional Diagram for Caltrans District 1 Public AffairsFigure D-6: Regional Diagram for Caltrans District 1 Public Affairs.

144Western Transportation Institute

Figure D-7: Regional Diagram for Caltrans District 2 Public AffairsFigure D-7: Regional Diagram for Caltrans District 2 Public Affairs.

Western Transportation Institute145

Figure D-8: Regional Diagram for Caltrans Field Equip District 1 (Eureka)Figure D-8: Regional Diagram for Caltrans Field Equip District 1 (Eureka).

146Western Transportation Institute

Figure D-9: Regional Diagram for Caltrans Field Equip District 2 (Redding)Figure D-9: Regional Diagram for Caltrans Field Equip District 2 (Redding).

Western Transportation Institute147

Figure D-10: Regional Diagram for Caltrans HQ Construction ProgramFigure D-10: Regional Diagram for Caltrans HQ Construction Program.

148Western Transportation Institute

Western Transportation Institute149

Figure D-11: Regional Diagram for Caltrans HQ PermitsFigure D-11: Regional Diagram for Caltrans HQ Permits.

Figure D-12: Regional Diagram for Caltrans HQ TrafficFigure D-12: Regional Diagram for Caltrans HQ Traffic.

150Western Transportation Institute

Figure D-13: Regional Diagram for Caltrans Maintenance District 1 (Eureka)Figure D-13: Regional Diagram for Caltrans Maintenance District 1 (Eureka).

Western Transportation Institute151

Figure D-14: Regional Diagram for Caltrans Maintenance District 1 (Eureka) VehiclesFigure D-14: Regional Diagram for Caltrans Maintenance District 1 (Eureka) Vehicles.

152Western Transportation Institute

Figure D-15: Regional Diagram for Caltrans Maintenance District 2 (Redding)Figure D-15: Regional Diagram for Caltrans Maintenance District 2 (Redding).

Western Transportation Institute153

Figure D-16: Regional Diagram for Caltrans Maintenance District 2 (Redding) VehiclesFigure D-16: Regional Diagram for Caltrans Maintenance District 2 (Redding) Vehicles.

154Western Transportation Institute

Western Transportation Institute155

Figure D-17: Regional Diagram for Caltrans RWISFigure D-17: Regional Diagram for Caltrans RWIS.

156Western Transportation Institute

Figure D-18: Regional Diagram for Caltrans Weather Warning SystemsFigure D-18: Regional Diagram for Caltrans Weather Warning Systems.

Figure D-19: Regional Diagram for Caltrans/CHP Eureka Figure D-19: Regional Diagram for Caltrans/CHP Eureka ARTIC.

Western Transportation Institute157

158Western Transportation Institute

Figure D-20: Regional Diagram for Caltrans/CHP Redding Figure D-20: Regional Diagram for Caltrans/CHP Redding ARTIC.

Figure D-21: Regional Diagram for CHINFigure D-21: Regional Diagram for CHIN.

Western Transportation Institute159

Figure D-22: Regional Diagram for CHP Eureka and Humboldt Division Dispatch and 911 CentersFigure D-22: Regional Diagram for CHP Eureka and Humboldt Division Dispatch and 911 Centers.

160Western Transportation Institute

Western Transportation Institute161

Figure D-23: Regional Diagram for CHP Northern District Dispatch and 911 CentersFigure D-23: Regional Diagram for CHP Northern District Dispatch and 911 Centers.

162Western Transportation Institute

Figure D-24: Regional Diagram for CHP VehiclesFigure D-24: Regional Diagram for CHP Vehicles.

Figure D-25: Regional Diagram for COATS KiosksFigure D-25: Regional Diagram for COATS Kiosks.

Western Transportation Institute163

Figure D-26: Regional Diagram for Coos Bay-Area City Traffic Control EquipFigure D-26: Regional Diagram for Coos Bay-Area City Traffic Control Equip.

164Western Transportation Institute

Figure D-27: Regional Diagram for Coos Bay-Area City Traffic Control SystemsFigure D-27: Regional Diagram for Coos Bay-Area City Traffic Control Systems.

Western Transportation Institute165

Figure D-28: Regional Diagram for Coos Bay-Area County Maintenance SystemsFigure D-28: Regional Diagram for Coos Bay-Area County Maintenance Systems.

166Western Transportation Institute

Figure D-29: Regional Diagram for Coos Bay-Area E-911 Comm CentersFigure D-29: Regional Diagram for Coos Bay-Area E-911 Comm Centers.

Western Transportation Institute167

Figure D-30: Regional Diagram for Coos Bay-Area EMS Dispatch and VehiclesFigure D-30: Regional Diagram for Coos Bay-Area EMS Dispatch and Vehicles.

168Western Transportation Institute

Figure D-31: Regional Diagram for Coos Bay-Area Fire and Rescue Dispatch and VehiclesFigure D-31: Regional Diagram for Coos Bay-Area Fire and Rescue Dispatch and Vehicles.

Western Transportation Institute169

Figure D-32: Regional Diagram for Coos Bay-Area Paratransit Dispatch and VehiclesFigure D-32: Regional Diagram for Coos Bay-Area Paratransit Dispatch and Vehicles.

170Western Transportation Institute

Figure D-33: Regional Diagram for Coos Bay-Area Planning AgenciesFigure D-33: Regional Diagram for Coos Bay-Area Planning Agencies.

Western Transportation Institute171

Figure D-34: Regional Diagram for Coos Bay-Area Police and Sheriff Dispatch and VehiclesFigure D-34: Regional Diagram for Coos Bay-Area Police and Sheriff Dispatch and Vehicles.

172Western Transportation Institute

Figure D-35: Regional Diagram for Coos Bay-Area Radio Stations and NewspapersFigure D-35: Regional Diagram for Coos Bay-Area Radio Stations and Newspapers.

Western Transportation Institute173

Figure D-36: Regional Diagram for Coos Bay-Area Traveler Information ResourcesFigure D-36: Regional Diagram for Coos Bay-Area Traveler Information Resources.

174Western Transportation Institute

Figure D-37: Regional Diagram for Coos Bay-Area Visitor Information ServicesFigure D-37: Regional Diagram for Coos Bay-Area Visitor Information Services.

Western Transportation Institute175

Figure D-38: Regional Diagram for DEQ Medford Regional OfficeFigure D-38: Regional Diagram for DEQ Medford Regional Office.

176Western Transportation Institute

Figure D-39: Regional Diagram for Downtown Parking DistrictFigure D-39: Regional Diagram for Downtown Parking District.

Western Transportation Institute177

Figure D-40: Regional Diagram for DriversFigure D-40: Regional Diagram for Drivers.

178Western Transportation Institute

Figure D-41: Regional Diagram for Eureka-Area Air DistrictsFigure D-41: Regional Diagram for Eureka-Area Air Districts.

Western Transportation Institute179

Figure D-42: Regional Diagram for Eureka-Area City Traffic Control EquipFigure D-42: Regional Diagram for Eureka-Area City Traffic Control Equip.

180Western Transportation Institute

Figure D-43: Regional Diagram for Eureka-Area City Traffic Control SystemsFigure D-43: Regional Diagram for Eureka-Area City Traffic Control Systems.

Western Transportation Institute181

Figure D-44: Regional Diagram for Eureka-Area County Maintenance SystemsFigure D-44: Regional Diagram for Eureka-Area County Maintenance Systems.

182Western Transportation Institute

Figure D-45: Regional Diagram for Eureka-Area E-911 Comm CentersFigure D-45: Regional Diagram for Eureka-Area E-911 Comm Centers.

Western Transportation Institute183

Figure D-46: Regional Diagram for Eureka-Area EMS Dispatch and VehiclesFigure D-46: Regional Diagram for Eureka-Area EMS Dispatch and Vehicles.

184Western Transportation Institute

Figure D-47: Regional Diagram for Eureka-Area Fire and Rescue Dispatch and VehiclesFigure D-47: Regional Diagram for Eureka-Area Fire and Rescue Dispatch and Vehicles.

Western Transportation Institute185

Figure D-48: Regional Diagram for Eureka-Area Fixed Route Dispatch and VehiclesFigure D-48: Regional Diagram for Eureka-Area Fixed Route Dispatch and Vehicles.

186Western Transportation Institute

Figure D-49: Regional Diagram for Eureka-Area Paratransit Dispatch and VehiclesFigure D-49: Regional Diagram for Eureka-Area Paratransit Dispatch and Vehicles.

Western Transportation Institute187

Figure D-50: Regional Diagram for Eureka-Area Planning AgenciesFigure D-50: Regional Diagram for Eureka-Area Planning Agencies.

188Western Transportation Institute

Figure D-51: Regional Diagram for Eureka-Area Police and Sheriff Dispatch and VehiclesFigure D-51: Regional Diagram for Eureka-Area Police and Sheriff Dispatch and Vehicles.

Western Transportation Institute189

Figure D-52: Regional Diagram for Eureka-Area Traveler Information ResourcesFigure D-52: Regional Diagram for Eureka-Area Traveler Information Resources.

190Western Transportation Institute

Figure D-53: Regional Diagram for Eureka-Area Traveler Information SystemsFigure D-53: Regional Diagram for Eureka-Area Traveler Information Systems.

Western Transportation Institute191

Figure D-54: Regional Diagram for Eureka-Area TV and Radio StationsFigure D-54: Regional Diagram for Eureka-Area TV and Radio Stations.

192Western Transportation Institute

Figure D-55: Regional Diagram for Eureka-Area Visitor Information ServicesFigure D-55: Regional Diagram for Eureka-Area Visitor Information Services.

Western Transportation Institute193

Figure D-56: Regional Diagram for Highway Travel Conditions Reporting SystemFigure D-56: Regional Diagram for Highway Travel Conditions Reporting System.

194Western Transportation Institute

Figure D-57: Regional Diagram for Inter-City Transit Dispatch and VehiclesFigure D-57: Regional Diagram for Inter-City Transit Dispatch and Vehicles.

Western Transportation Institute195

Figure D-58: Regional Diagram for Klamath Falls-Area City Traffic Control EquipFigure D-58: Regional Diagram for Klamath Falls-Area City Traffic Control Equip.

196Western Transportation Institute

Figure D-59: Regional Diagram for Klamath Falls-Area City Traffic Control SystemsFigure D-59: Regional Diagram for Klamath Falls-Area City Traffic Control Systems.

Western Transportation Institute197

Figure D-60: Regional Diagram for Klamath Falls-Area County Maintenance SystemsFigure D-60: Regional Diagram for Klamath Falls-Area County Maintenance Systems.

198Western Transportation Institute

Figure D-61: Regional Diagram for Klamath Falls-Area E-911 Comm CenterFigure D-61: Regional Diagram for Klamath Falls-Area E-911 Comm Center.

Western Transportation Institute199

Figure D-62: Regional Diagram for Klamath Falls-Area EMS Dispatch and VehiclesFigure D-62: Regional Diagram for Klamath Falls-Area EMS Dispatch and Vehicles.

200Western Transportation Institute

Figure D-63: Regional Diagram for Klamath Falls-Area Fire and Rescue Dispatch and VehiclesFigure D-63: Regional Diagram for Klamath Falls-Area Fire and Rescue Dispatch and Vehicles.

Western Transportation Institute201

Figure D-64: Regional Diagram for Klamath Falls-Area Fixed Route Dispatch and VehiclesFigure D-64: Regional Diagram for Klamath Falls-Area Fixed Route Dispatch and Vehicles.

202Western Transportation Institute

Figure D-65: Regional Diagram for Klamath Falls-Area Paratransit Dispatch and VehiclesFigure D-65: Regional Diagram for Klamath Falls-Area Paratransit Dispatch and Vehicles.

Western Transportation Institute203

Figure D-66: Regional Diagram for Klamath Falls-Area Planning AgenciesFigure D-66: Regional Diagram for Klamath Falls-Area Planning Agencies.

204Western Transportation Institute

Figure D-67: Regional Diagram for Klamath Falls-Area Police and Sheriff Dispatch and VehiclesFigure D-67: Regional Diagram for Klamath Falls-Area Police and Sheriff Dispatch and Vehicles.

Western Transportation Institute205

Figure D-68: Regional Diagram for Klamath Falls-Area Traveler Information ResourcesFigure D-68: Regional Diagram for Klamath Falls-Area Traveler Information Resources.

206Western Transportation Institute

Figure D-69: Regional Diagram for Klamath Falls-Area TV, Radio Stations and NewspapersFigure D-69: Regional Diagram for Klamath Falls-Area TV, Radio Stations and Newspapers.

Western Transportation Institute207

Figure D-70: Regional Diagram for Klamath Falls-Area Visitor Information ServicesFigure D-70: Regional Diagram for Klamath Falls-Area Visitor Information Services.

208Western Transportation Institute

Western Transportation Institute209

Figure D-71: Regional Diagram for Mayday Service ProviderFigure D-71: Regional Diagram for Mayday Service Provider.

210Western Transportation Institute

Figure D-72: Regional Diagram for Medford-Area City Traffic Control EquipFigure D-72: Regional Diagram for Medford-Area City Traffic Control Equip.

Western Transportation Institute211

Figure D-73: Regional Diagram for Medford-Area City Traffic Control SystemsFigure D-73: Regional Diagram for Medford-Area City Traffic Control Systems.

212Western Transportation Institute

Figure D-74: Regional Diagram for Medford-Area County Maintenance SystemsFigure D-74: Regional Diagram for Medford-Area County Maintenance Systems.

Western Transportation Institute213

Figure D-75: Regional Diagram for Medford-Area E-911 Comm CentersFigure D-75: Regional Diagram for Medford-Area E-911 Comm Centers.

214Western Transportation Institute

Figure D-76: Regional Diagram for Medford-Area EMS Dispatch and VehiclesFigure D-76: Regional Diagram for Medford-Area EMS Dispatch and Vehicles.

Western Transportation Institute215

Figure D-77: Regional Diagram for Medford-Area Fire and Rescue Dispatch and VehiclesFigure D-77: Regional Diagram for Medford-Area Fire and Rescue Dispatch and Vehicles.

216Western Transportation Institute

Figure D-78: Regional Diagram for Medford-Area Fixed Route Dispatch and VehiclesFigure D-78: Regional Diagram for Medford-Area Fixed Route Dispatch and Vehicles.

Western Transportation Institute217

Figure D-79: Regional Diagram for Medford-Area Paratransit Dispatch and VehiclesFigure D-79: Regional Diagram for Medford-Area Paratransit Dispatch and Vehicles.

218Western Transportation Institute

Figure D-80: Regional Diagram for Medford-Area Planning AgenciesFigure D-80: Regional Diagram for Medford-Area Planning Agencies.

Western Transportation Institute219

Figure D-81: Regional Diagram for Medford-Area Police and Sheriff Dispatch and VehiclesFigure D-81: Regional Diagram for Medford-Area Police and Sheriff Dispatch and Vehicles.

220Western Transportation Institute

Figure D-82: Regional Diagram for Medford-Area Traveler Information ResourcesFigure D-82: Regional Diagram for Medford-Area Traveler Information Resources.

Western Transportation Institute221

Figure D-83: Regional Diagram for Medford-Area TV and Radio StationsFigure D-83: Regional Diagram for Medford-Area TV and Radio Stations.

222Western Transportation Institute

Figure D-84: Regional Diagram for Medford-Area Visitor CentersFigure D-84: Regional Diagram for Medford-Area Visitor Centers.

Western Transportation Institute223

Figure D-85: Regional Diagram for Medford-Area Visitor Information ServicesFigure D-85: Regional Diagram for Medford-Area Visitor Information Services.

224Western Transportation Institute

Figure D-86: Regional Diagram for ODOT Communications DivisionFigure D-86: Regional Diagram for ODOT Communications Division.

Western Transportation Institute225

Figure D-87: Regional Diagram for ODOT Field Equip (Medford TOC)Figure D-87: Regional Diagram for ODOT Field Equip (Medford TOC).

226Western Transportation Institute

Figure D-88: Regional Diagram for ODOT Maintenance Districts (Medford TOC)Figure D-88: Regional Diagram for ODOT Maintenance Districts (Medford TOC).

Western Transportation Institute227

Figure D-89: Regional Diagram for ODOT Maintenance Vehicles (Medford TOC)Figure D-89: Regional Diagram for ODOT Maintenance Vehicles (Medford TOC).

Figure D-90: Regional Diagram for ODOT Medford TOCFigure D-90: Regional Diagram for ODOT Medford TOC.

228Western Transportation Institute

Figure D-91: Regional Diagram for ODOT RWIS ServerFigure D-91: Regional Diagram for ODOT RWIS Server.

Western Transportation Institute229

Figure D-92: Regional Diagram for ODOT Statewide PlanningFigure D-92: Regional Diagram for ODOT Statewide Planning.

230Western Transportation Institute

Western Transportation Institute231

Figure D-93: Regional Diagram for ODOT Weather Warning SystemFigure D-93: Regional Diagram for ODOT Weather Warning System.

Figure D-94: Regional Diagram for OSP DispatchFigure D-94: Regional Diagram for OSP Dispatch.

232Western Transportation Institute

Figure D-95: Regional Diagram for OSP VehiclesFigure D-95: Regional Diagram for OSP Vehicles.

Western Transportation Institute233

Figure D-96: Regional Diagram for Other Weather DataFigure D-96: Regional Diagram for Other Weather Data.

234Western Transportation Institute

Figure D-97: Regional Diagram for Redding-Area Air DistrictsFigure D-97: Regional Diagram for Redding-Area Air Districts.

Western Transportation Institute235

Figure D-98: Regional Diagram for Redding-Area City Traffic Control EquipFigure D-98: Regional Diagram for Redding-Area City Traffic Control Equip.

236Western Transportation Institute

Figure D-99: Regional Diagram for Redding-Area City Traffic Control SystemsFigure D-99: Regional Diagram for Redding-Area City Traffic Control Systems.

Western Transportation Institute237

Figure D-100: Regional Diagram for Redding-Area County Maintenance SystemsFigure D-100: Regional Diagram for Redding-Area County Maintenance Systems.

238Western Transportation Institute

Figure D-101: Regional Diagram for Redding-Area E-911 Comm CentersFigure D-101: Regional Diagram for Redding-Area E-911 Comm Centers.

Western Transportation Institute239

Figure D-102: Regional Diagram for Redding-Area EMS Dispatch and VehiclesFigure D-102: Regional Diagram for Redding-Area EMS Dispatch and Vehicles.

240Western Transportation Institute

Figure D-103: Regional Diagram for Redding-Area Fire Dispatch and VehiclesFigure D-103: Regional Diagram for Redding-Area Fire Dispatch and Vehicles.

Western Transportation Institute241

Figure D-104: Regional Diagram for Redding-Area Fixed Route Dispatch and VehiclesFigure D-104: Regional Diagram for Redding-Area Fixed Route Dispatch and Vehicles.

242Western Transportation Institute

Figure D-105: Regional Diagram for Redding-Area Paratransit Dispatch and VehiclesFigure D-105: Regional Diagram for Redding-Area Paratransit Dispatch and Vehicles.

Western Transportation Institute243

Figure D-106: Regional Diagram for Redding-Area Planning AgenciesFigure D-106: Regional Diagram for Redding-Area Planning Agencies.

244Western Transportation Institute

Figure D-107: Regional Diagram for Redding-Area Police and Sheriff Dispatch and VehiclesFigure D-107: Regional Diagram for Redding-Area Police and Sheriff Dispatch and Vehicles.

Western Transportation Institute245

Figure D-108: Regional Diagram for Redding-Area Traveler Information ResourcesFigure D-108: Regional Diagram for Redding-Area Traveler Information Resources.

246Western Transportation Institute

Figure D-109: Regional Diagram for Redding-Area Traveler Information SystemsFigure D-109: Regional Diagram for Redding-Area Traveler Information Systems.

Western Transportation Institute247

Figure D-110: Regional Diagram for Redding-Area TV and Radio StationsFigure D-110: Regional Diagram for Redding-Area TV and Radio Stations.

248Western Transportation Institute

Figure D-111: Regional Diagram for Redding-Area Visitor Information ServicesFigure D-111: Regional Diagram for Redding-Area Visitor Information Services.

Western Transportation Institute249

Figure D-112: Regional Diagram for SOVA KiosksFigure D-112: Regional Diagram for SOVA Kiosks.

250Western Transportation Institute

Figure D-113: Regional Diagram for Transit UsersFigure D-113: Regional Diagram for Transit Users.

Western Transportation Institute251

Figure D-114: Regional Diagram for Traveler Information DevicesFigure D-114: Regional Diagram for Traveler Information Devices.

252Western Transportation Institute

Figure D-115: Regional Diagram for TravelersFigure D-115: Regional Diagram for Travelers.

Western Transportation Institute253

254Western Transportation Institute

Figure D-116: Regional Diagram for TripCheckFigure D-116: Regional Diagram for TripCheck.

Figure D-117: Regional Diagram for Vehicle Information SystemsFigure D-117: Regional Diagram for Vehicle Information Systems.

Western Transportation Institute255

Figure D-118: Regional Diagram for Vehicle Mayday SystemsFigure D-118: Regional Diagram for Vehicle Mayday Systems.

256Western Transportation Institute

Figure D-119: Regional Diagram for VehiclesFigure D-119: Regional Diagram for Vehicles.

Western Transportation Institute257

258Western Transportation Institute

Figure D-120: Regional Diagram for Virtual Transit MallFigure D-120: Regional Diagram for Virtual Transit Mall.

There is a placeholder for the CVO-related architecture in the COATS architecture. See Section 4.4 for more detail.

The project team did not look at the needs of travelers who use other modes, such as rail or inter-city bus. Trying to reach travelers of other modes was beyond the resources and scope of the study.